Day in the Life of the Hudson River 10/10/13 Data

(Salt Front RM 69.3)

RIVER MILE -10

South Beach Wolf Park, Staten Island

N 4044.375’  W-740.1820’

Steve Stanne, HREP, NYSDEC

Mary Lee, St. Clare’s School, 70 Seventh graders and 15 adults

Location: South Beach of Wolf Park, Staten Island

Area: Beachfront – see student sketches (attached)

Surrounding Land Use: 90% beach,10% Boardwalk/Park

Sampling Site: Beach area with bulkhead, concrete broken in area

River Bottom: Sandy

Plants in area: No plants

| ITEM | Time |  | Readings |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |
| Air Temperature | 10:00 AM 10:52 AM |  | 18.3C 15C | 65F 59.3F |  |  |
| Wind Speed | Beaufort #4 |  | 3.6 mph | East |  |  |
| Cloud Cover | 12:15 PM 12:46 PM 1:03 PM 1:33 PM |  | 100% 100% 100% 100% |  |  |  |
| Weather today | Cold & damp, overcast |  | Became colder as day progressed | Water surface |  | Very choppy – high waves |
| Weather recently | Windy, cool, some rain |  |  |  |  |  |
| Water Temperature | 10:03 AM 10:08 AM 10:13 AM | 20C 19C 18C |  | 68F 64F 60F | Weather grew colder during day |  |
| Turbidity Turbidity tube | 10:45 AM | 20.0 cm 27.1 22.5 |  | 23 cm avg |  |  |
| Chemical | Time | ppm |  | Temp | % Sat |  |
| DO Estuary wide monitoring Kit – | 10:10 AM 10:19 AM 10:36 AM 10:58 AM 11:13 AM 11:30 AM | 8 ppm 7 7 8 10 8 |  | 15 C 15C 15C 16C 17C 17C | 80% 70% 70% 80% 100% 90% |  |
| pH Litmus Paper | 10:00 AM 10:25 AM 10:45 AM 11:03 AM 11:19 AM 11:35 AM | 6 6 6.5 5.5 6.5 6 |  | 6 average |  |  |
| Salinity - hydrometer | 10:07 AM 10:27 AM 10:46 AM 11:05 AM 11:21 AM 11:40 AM | 32 ppt 30 ppt 28 ppt 27.75 ppt 29 ppt 28 ppt |  | 29 ppt | Specific gravity 1.028 |  |
| Fish Catch – seine 50ft. net – 20-30 ft. pull | Time | Species |  | Number |  |  |
|  | 10:00-11:30 AM | Hermit Crab Mole Crab Mussels Spider Crab Comb Jellies |  | 1 16 15 3 8 | No fish but macro-invertebrates also netted 1 blue crab claw |  |
| Tides (from pier to water) | Time | Distance to water |  | Rate of change | Flood or Ebb |  |
|  | 10:02 AM 10:15 AM 10:30 AM 10:45 AM 11:00 AM 11:15 AM *stick float away | 50 cm 60 cm 70 cm 80 cm 90 cm 90cm |  | Start point 6.6cm/min 6.6cm/min 6.6cm/min 6.6cm/min 0 cm/min | Baseline Flooding Flooding Flooding Flooding Unchanged |  |
| Currents - Time | Cm/30sec | Cm/sec |  | Knots | N/S Ebb/Flood |  |
| 10:05 AM | 258 | 8.6 |  | 0.2 | N - F |  |
| 10:24 AM | 540 | 18 |  | 0.4 | N- F |  |
| 10:46 AM | 600 | 20 |  | 0.4 | N-F |  |
| 11:03 AM | 0 | 0 |  | 0 | 0 |  |
| 11;16 AM | 62 | 2.1 |  | 0.04 | N-F |  |
| 11:30 AM | 0 | 0 |  | 0 | 0 |  |
| Core Sample | Length | Abundant |  | Common |  |  |
|  |  | Sandy |  | Gravel pebbles |  |  |
| Shipping – Time | Type | Name |  | Color | Direction/ light or loaded |  |
| 10:02 AM | Cargo | Zim |  | R/W/B | N loaded |  |
| 10:24 AM | Cruise | Seastruck |  | B/W | S loaded |  |
| 10:30 AM | Cargo |  |  |  | S |  |
| 10:33 AM | Speedboat |  |  |  | S |  |
| 10:38 AM | Cruiseship | AIDA |  |  | N loaded |  |
| 10:38 AM | Cargo |  |  | R/B | N loaded |  |
| 11:20 AM | Tanker |  |  | R | N loaded |  |
| 11:32 AM | Sailboat |  |  | W | W |  |
| 11:34 AM | Cargo | Harbor Fashion |  | R/B | N loaded |  |