Snapshot Day 10/12/05 Data

(Salt Front RM 53.2)

RIVER MILE 85

Norrie Point Environmental Center, Staatsburgh, NY, Dutchess County

Location: Sampling done on the southern inlet of Norrie Point.  East side of Hudson far from the main channel in a protected cove

Area: Substantial paved areas nearby, large environmental center building adjacent to the water.

Surrounding Land Use: forested 60%; Environmental Education Center (EEC) and concrete patio 40%

Sampling Site: By seining area: large parking lot adjacent to the shallows, fairly protected bay.  By water chemistry area: samples collected off concrete patio adjacent to dock and EEC; Wind and weather taken from EEC

Shoreline: covered with vegetation, banks altered for EEC building, riprap, collected wood and debris in shallow cove

Plants in area:  15% of water in sampling site is covered with plans.  10% of area adjacent to the water is covered with plants

| Tidal Shallows Plants | Water chestnut 20% |  |  | water celery 20% |  | Duckweed 60% |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tidal Marshes |  | Jewelweed 5% | Narrow-leaved cattail 40% |  | Purple loosestrife 40% |  | Yellow Flag 15% |

Water depth:  3-6 feet deep where chemistry water collected

River Bottom – gravel and rock bottom where chemistry done.  Rocky and partly muddy where seining/sediment/plankton done.

| ITEM | Time |  |  |  | Reading 1 |  |  | Reading 2 |  |  | Comments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Air Temperature | 9:37 AM |  |  |  | 9C 47 F |  |  | 10:00 |  |  | 6C 43 F |  |  |
|  | 10:45 |  |  |  | 7C 44 F |  |  | 11:59 |  |  | 5C 41 F |  |  |
|  | 12:42 |  |  |  | 7C 45 F |  |  | Averages |  |  | 6.8C 44.2 F |  |  |
| Wind Speed | 2 beaufort |  |  |  | 6-11 kph |  |  | Water choppy |  |  |  |  |  |
| Cloud Cover | overcast |  |  |  |  |  |  |  |  |  |  |  |  |
| Weather today | Rain. 9:37 floodwatch; 10:45 heavy rain; 11:59 pouring |  |  |  |  |  |  |  |  |  |  |  |  |
| Weather recently | 10 inches of ran in last 3 days |  |  |  |  |  |  |  |  |  |  |  |  |
| Water Temperature | 9:50 AM |  |  |  | 61.7 F 16.5C |  |  | 10:13 |  |  | 62.6 F 17C |  |  |
|  | 11:00 |  |  |  | 62.6 F 17C |  |  | 12:25 |  |  | 61.7 F 16.5C |  |  |
|  | 1:30 |  |  |  | 61.7 F 16.5C |  |  |  |  |  |  |  |  |
| Turbidity | 10:15 |  |  |  | 35.5 JTU |  |  | 11:10 |  |  | 35.5 JTU |  |  |
| Sight tube | 12:17 |  |  |  | 25 JTU |  |  | 1:07 |  |  | 29 JTU |  |  |
| Chlorophyll | 10:30 |  |  |  | 0.3 |  |  |  |  |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DO (drop count) | 9:50 |  |  |  | 8.0 mg/L |  |  | 16C |  |  | 82.5% |  |  |
|  | 10:20 |  |  |  | 9.0 mg/L |  |  | 17C |  |  | 95% |  |  |
|  | 11:00 |  |  |  | 8.5 mg/L |  |  | 17C |  |  | 90% |  |  |
|  | 12:25 |  |  |  | 8.5 mg/L |  |  | 17C |  |  | 90% |  |  |
|  | 1:10 |  |  |  | 8.5 mg/L |  |  | 16C |  |  | 87.5% |  |  |
| pH | 9:39-12:45 |  |  | 7.5 |  |  |  |  |  |  | Ind. Sol |  |  |
| Phosphate |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Nitrate |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Alkalinity |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Salinity | 9:40 AM |  |  | 35 ppm |  |  |  | 40 ppm |  |  | Avg – 37.5 ppm |  |  |
| - | 10:10 |  |  | 35 ppm |  |  |  | 40 ppm |  |  | Avg – 37.5 ppm |  |  |
|  | 10:45 |  |  | 35 ppm |  |  |  | 45 ppm |  |  | Avg – 40 ppm |  |  |
|  | 12:10 |  |  | 35 ppm |  |  |  | 45 ppm |  |  | Avg – 40 ppm |  |  |
|  | 12:50 |  |  | 40 ppm |  |  |  | 35 ppm |  |  | Avg – 37.5 ppm |  |  |
| Fish seined | Group1 |  | Group 2 |  |  | Group 3 |  | Group 4 |  | Group 5 |  |  | TOTAL |
| Alewife |  |  |  |  |  |  |  | 11 |  |  |  |  | 11 |
| Blueback Herring | 1 |  |  |  |  | 14 |  | 36 |  | 213 |  |  | 264 |
| Bluegill |  |  | 1 |  |  | 1 |  | 3 |  |  |  |  | 5 |
| Golden Shiner | 1 |  |  |  |  | 4 |  | 3 |  | 3 |  |  | 11 |
| Pumpkinseed | 2 |  | 2 |  |  | 1 |  | 2 |  |  |  |  | 7 |
| Smallmouth Bass |  |  |  |  |  | 1 |  |  |  |  |  |  | 1 |
| Spottail Shiner | 1 |  | 10 |  |  | 11 |  | 10 |  |  |  |  | 32 |
| Striped Bass |  |  |  |  |  | 1 |  |  |  |  |  |  | 1 |
| White Perch |  |  | 1 |  |  |  |  |  |  |  |  |  | 1 |
| Yellow Perch |  |  |  |  |  | 1 |  |  |  |  |  |  | 1 |
| Killifish | 1 |  |  |  |  |  |  |  |  | 1 |  |  | 2 |
| TOTALS | 6 |  | 14 |  |  | 34 |  | 65 |  | 217 |  |  | 336 |
| 30 ft net/40 ft pull | 365.76 m |  |  | 6 total |  |  |  | 0.016 |  |  | Seine #1 |  |  |
| 30 ft net/50 ft pull | 457.20 |  |  | 14 total |  |  |  | 0.031 |  |  | Seine #2 |  |  |
| 30 ft net/45 ft pull | 411.48 |  |  | 34 total |  |  |  | 0.083 |  |  | Seine #3 |  |  |
| 30 ft net/40 ft pull | 365.76 |  |  | 30 total |  |  |  | 0.082 |  |  | Seine #4 |  |  |
| 30 ft net/40 ft pull | 365.76 |  |  | 35 total |  |  |  | 0.096 |  |  | Seine #5* |  |  |
| 30 ft net/50 ft pull | 457.20 |  |  | 217 total |  |  |  | 0.475 |  |  | Seine #6 |  |  |
| * hole in net on this seine – maybe lost fish |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tides |  |  |  | Inch |  |  | cm |  |  |  |  |  |  |
|  | 9:37 AM |  |  | 48 |  |  | 121.92 | Start |  |  |  |  |  |
|  | 9:58 AM |  |  | 47 |  |  | 119.38 | Falling |  |  |  |  |  |
|  | 10:00 |  |  | 47 |  |  | 119.38 | Falling |  |  |  |  |  |
|  | 10:30 |  |  | 43.5 |  |  | 110.49 |  |  |  |  |  |  |
|  | 10:48 |  |  | 44 |  |  | 111.76 |  |  |  |  |  |  |
|  | 11:10 |  |  | 42 |  |  | 106.68 |  |  |  |  |  |  |
|  | 11:58 |  |  | 35 |  |  | 88.90 |  |  |  |  |  |  |
|  | 12:29 PM |  |  | 32 |  |  | 81.28 |  |  |  |  |  |  |
|  | 12:42 PM |  |  | 29.5 |  |  | 74.93 |  |  |  |  |  |  |
|  | 1:10 PM |  |  | 26 |  |  | 66.04 |  |  |  |  |  |  |
| Currents | South flowing |  |  | Ebbing |  |  |  | Entire sampling period |  |  |  |  |  |
| * Site is far from main channel – unable to calculate current due to site restrictions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Traffic |  | Time |  |  | Direction |  |  |  |  |  |  |  |  |
| Sailboat under power |  | 10:25 |  |  | South |  |  |  |  |  |  |  |  |
| Freighter (Intermarine) |  | 11:00 |  |  | North |  |  |  |  |  |  |  |  |
| Tanker |  | 12:09 |  |  | North |  |  |  |  |  |  |  |  |
| Pleasure Boat |  | 12:19 |  |  | South |  |  |  |  |  |  |  |  |
| Tugboat |  | 12:21 |  |  | South |  |  |  |  |  |  |  |  |
| Commercial Tour Boat |  | 12:52 |  |  | South |  |  |  |  |  |  |  |  |