Snapshot Day 10/12/06 Data

(Salt Front RM 52.5)

RIVER MILE 115

Cohotate Preserve, Athens, Greene Co.

Liz LoGuidice, Cornell Cooperative Extension of Greene County

Tony Loughran, Coxsackie Athens High School Env. Sci. (12th grade)

Location: Cohotate preserve – forested area with education center.  Large waterfront vegetated.  North section bulkheaded, south section is beach

Area: education center, has a building

Surrounding Land Use: Forested 90%, Beach 5%; College field station 5%

Sampling Site: beach, altered banks, bulkhead, covered with vegetation, collected wood/debris in area,

Plants in area: 85% of water has plants

| Water plants | Water celery 99% | Water chestnut1% |  |
| --- | --- | --- | --- |
| Tidal -Land plants | Arrow Arum 20% | Three Square 25% | Dotted Smartweed 1% |
|  | Jewelweed 3% | Pickerelweed 5% | Purple Loosestrife 1% |
|  | Spatterdock 40% | Spotted Joe Pye Weed 2% | Wildrice 1% |

Water depth:

River Bottom : muddy/rocky bottom

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 9:45 AM |  | 16C 60.8F |  |  |
| Wind Speed | 2 knts SE |  |  | Water calm |  |
| Cloud Cover | overcast |  |  |  |  |
| Weather today | misty and foggy, 11:35 AM sun came out |  |  |  |  |
| Weather recently | rain last night, but seasonable temperatures & partly sunny lately |  |  |  |  |
| Water Temperature | 11:00 AM | 16C 60.8 F |  |  |  |
| Turbidity | 11:15 AM | 20 JTU |  |  | Site tube |
| Chlorophyll | 10:00 AM | 0.7 |  |  |  |
| Chemical |  |  |  |  |  |
| DO (drop count kit) | 11:20 AM | 9.0 mg/L |  | 16C | 90% |
| pH – ind. solution | 11:15 AM | 7 |  |  |  |
| Phosphate |  |  |  |  |  |
| Nitrate |  |  |  |  |  |
| Alkalinity |  |  |  |  |  |
| Salinity |  |  |  |  |  |
| Fish Catch 60 ft. seine X 80 ft. pull | Number Caught | Species |  | CPUE | Seine 5 seines |
|  | 1 | Banded Killifish |  |  |  |
|  | 1 | herring |  |  |  |
|  | 2 | pumpkinseed |  |  |  |
|  | 11 | Spottail Shiners |  |  |  |
|  | 10 | Tesselated Darter |  |  |  |
|  | 8 | White Perch |  |  |  |
|  | 1 | Striped Bass |  |  |  |
|  | 24 | Juvenile Blue Crab |  |  |  |
| Tides | 9:47 AM | Set stick |  |  |  |
|  | 10:15 AM |  |  | Falling |  |
|  | 10:45 AM |  |  | Falling |  |
|  | 11:13 AM |  |  | Falling |  |
|  | 11:30 AM |  |  | Falling |  |
|  | noon |  |  | Falling |  |
| Currents | south |  |  | ebb |  |
| Traffic | 9:45 Am | Cement Barge Maria T. |  | South bound | Loaded cement |
|  | 10:25 AM | Barge B. No 220 Bouchard |  | Southbound | Loaded |
|  | 10:45 AM | Ship – Cos Bonny |  | Southbound | Light |
| Other Items | 2 eagles flying South | Many Cormorants |  | 1 Migrating Monarch | Caught lots of small blue claw crabs! |
| Core Description Cohotate | Yes | Rare |  | Common | Abundant |
| 10:30 AM |  | Gravel |  | Clay |  |
| Water Depth ~ 3ft |  | Leaves |  | Mud |  |
| 14 cm length no layers Blackish-grey sediment |  | Wood |  | Sand |  |
| Core Description Dutchman’s Landing | Yes | Rare |  | Common | Abundant |
| 12:30 |  |  |  | Clay | Mud |
| Water Depth ~ 3ft |  |  |  |  |  |