Snapshot Day 10/12/06 Data

(Salt Front RM 52.5)

RIVER MILE 28

Nyack Memorial Park

Tom Perry Nyack High School APES Class

Latitude 4005.243’N, Longitude 7354.988 ‘W

Location: Nyack Park where Nyack Brook feeds in

Area: Open and grassy, has a parking lot, used for picnics, fishing

Surrounding Land Use:  100% park

Sampling Site: beach, banks altered, riprap on shoreline, bulkhead

Plants in area: no water plants in area

Water depth: varied

River Bottom – sandy/muddy bottom , water calm

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 9:30 AM |  | 18C F |  |  |
| Wind Speed | 1 knot |  | South | Light breeze |  |
| Cloud Cover | overcast |  |  |  |  |
| Weather today | drizzle |  |  |  |  |
| Weather recently | Heavy rain last night |  |  |  |  |
| Water Temperature | 9:30 AM | 18.2C ramp |  | 16.4 C top | 17.9 C bottom |
| Turbidity | 9:30 AM | 30.25 cm |  | Old pier | secchi |
|  |  | 50.0 cm |  | Boat launching ramp |  |
| Chlorophyll | 9:30 AM | 0.5 |  |  |  |
| Chemical |  |  |  |  |  |
| DO | 9:30 AM | 6.8 mg/L |  | 18.2C | 70% |
| pH | 9:30 AM | 7.0 |  | pH paper |  |
| Phosphate | 9:30 Am | <1.0 mg/L ramp |  | <1.0 mg/L top | <1.0 mg/L bottom |
| Nitrate | 9:30 AM | 0.15 mg/L (ramp) |  | 0.04 mg/L (top) | 0.02 mg/L (bottom) |
| Alkalinity |  |  |  |  |  |
| Salinity (meter) | 9:30 AM | 3.6 ppt ramp |  | 0.4 ppt top | 1.6 ppt bottom |
| Fish Catch @ mouth of Nyack Brook & Hudson River | Number Caught | Species |  | CPUE | Seine |
|  | 21 | Atlantic Silversides |  | Menidia menidia |  |
|  | 1 | Banded Killifish |  | Fundulus diaphanus |  |
| Other Catch |  | Glass Shrimp |  |  |  |
| Tides | 9:30 AM | 20 cm depth |  |  |  |
|  | 10:00 AM | 29 cm |  | rising |  |
|  | 10:30 AM | 50 cm |  | rising |  |
|  | 11:00 Am | 65 cn |  | rising |  |
| Currents |  |  |  |  |  |
| Sediment Cores Pier description | 14 cm core | 7 cm Top layer Light & liquidy |  | 1 cm Second layer dark brown and sandy | 5.5 cm 3rd layer most carbonated, black |
| Mud description | Very carbonated, dark, decomposing, liquidy. Came out easily |  |  |  |  |
| Sand Description | Very compact, rocky, big chunks of gravel, difficult to get out, many air bubbles |  |  |  |  |
| Other Items STRATIFICATION STUDY | Did a transect study from Nyack Brook out to the Hudson River. Found a stratification in the “Zone of Mixing” – an estuary within an estuary. Found colder fresher water moving toward the river on the surface (See results above), with warmer saltier water pushing in along the bottom. HEADING EAST FROM THE BROOK OVER TOP 16.6 C and 16.8 C temp. 0.5 ppt and 0.5 ppt salinity HEADING WEST FROM THE HUDSON BELOW 17.9 C and 17.2 C temp. 2.1 ppt and 2.2 ppt salinity |  |  |  |  |
| TRANSECT STUDY NYACK BROOK OUT TO HUDSON RIVER measured every 10 meters | Distance | Salinity |  | Temp C | DO mg/L |
|  | 0 | 0.4 |  | 16.4 | 8.25 |
|  | 10 | 0.4 |  | 16.4 | 7.9 |
|  | 20 | 0.5 |  | 16.4 | 7.1 |
|  | 30 | 0.5 |  | 16.4 | 6.8 |
|  | 40 | 0.5 |  | 16.4 | 6.8 |
|  | 50 | 0.5 |  | 16.4 | 7.15 |
|  | 60 | 0.5 |  | 16.5 | 7.7 |
|  | 70 | 0.5 |  | 16.6 | 7.3 |
|  | 80 | 0.6 |  | 17.1 | 7.5 |
|  | 130 | 7.6 |  | 17.9 | 8.2 |
|  | RAMP | 3.6 |  | 18.2 | 6.8 |