Snapshot Day 10/12/05 Data

(Salt Front RM 53.5)

HARLEM RIVER MILE 13

Swindler’s Cove Park on the Harlem River

GPS Latitude 40 51’ 25.05” N Longitude 73 55’ 18.84” W

Jane Jackson, New York Restoration Project

Anne Marie Nowak & Virginia Young, Booker T. Washington MS, 6-8th grade

Location: Beach at Swindler Cove Park, next to the north end of Harlem River Drive, Manhattan

Area:

Surrounding Land Use: 50% urban/residential, 20% forested, 5% beach, 15% Industrial/commercial, 10% other

Sampling Site: Beach area, covered with vegetation Rip Rap used on beach, shoreline is sandy and rocky, pipes seen entering water (nearby CSO)

Plants in area: none in water

Water depth: 1 foot deep

River Bottom: sandy & rocky bottom

| ITEM | Time |  | Reading 1 |  | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |
| Air Temperature | 12:05 PM |  | 69F 20.57C |  |  |  |
| Wind Speed | 4-6 |  | Lightly breezy |  | North wind |  |
| Cloud Cover | overcast |  |  |  |  |  |
| Weather today | Cool and overcast |  |  |  |  |  |
| Weather recently | 1.5 inches of rain fell in the last 24 hrs. Over the last 3 days we have had sun and higher than normal temperatures for this time of year. |  |  |  |  |  |
| Water Temperature Shady- 1ft. water | 11:10 AM | 18.30 C 64.90 F |  | 18.5 C 65.30 F |  | 18.5 C 65.3  F |
| Turbidity | 11:00 | 0 JTU X 3 readings |  | With sight tube |  | 3 ft. with secchi |
| Chlorophyll | 11:14 AM | 0.3 |  | 11:26 AM |  | 0.4 |
| Chemical |  |  |  |  |  |  |
| DO Calm water in shade- (LaMotte modified winkler) | 11:15 | 6.4 mg/L* |  | Temp. 18.4 C |  | 70% saturated |
| pH – test strips | 11:40 | 7.8 |  | 7.6 |  | 7.8 |
| Phosphate |  |  |  |  |  |  |
| Nitrate |  |  |  |  |  |  |
| Alkalinity |  |  |  |  |  |  |
| Salinity | Unable to read |  |  |  |  |  |
| Fish Catch | Number Caught | Species |  | CPUE |  | Size |
| 1:45 PM | 1 | Blue Crab |  |  |  | 1.25 carpace |
|  | 24 | Atlantic Silversides |  | .01 |  | 7.5 inches |
|  | 1 | White perch |  |  |  | 5 inches |
| Tides | 10:46 AM | 2 cm |  |  |  |  |
|  | 11:16 AM | 14 cm |  | Rising |  |  |
| Currents | 11:42 AM | Flood |  |  |  |  |
| Core Sample Sediments | Whole core | Top layer |  | Bottom layer |  | description |
| 11:10 AM | 11 cm | 6 cm gritty feel |  | 5 cm dense feel & smooth |  | Green & black, bottom sample darker |
|  | Tom layer had bits of vegetation decomposing, soft gritty top, fine bottom layer. Clay (common) Mud (Abundant) Sand, Gravel, Pebbles & wood (rare) Possibly some oil |  |  |  |  |  |
| Traffic |  |  |  |  |  |  |
|  | 11:56 AM | Circle Line/red & white |  | Northbound |  | Passengers |
|  | 12:00 noon | Fishing boat |  | Southbound |  |  |
| Other Items | Ducks, jelly fish, small fish, oysters & crabs |  |  |  |  |  |