Snapshot Day 10/2/07 Data

(Salt Front RM 78.2)

RIVER MILE 18

Vicki Garufi, Tyra Barksdale-  Beczak Environmental Education Center

Harry Hall - Saunders Trade & Technical High School

(30 students) 10th -12th grade

Location: Seining beach, Beczak Center, Yonkers in Westchester County

Area: Used for FISHING, picnics, boats, seining and education programs

Surrounding Land Use: Urban/residential 50%, Forested 1%, Beach 20%, industrial/commercial 25%; other  9%

Sampling Site: beach,  marsh area covered with vegetation, sandy area, banks altered, large riprap on shoreline, collected wood/debris in area –

Plants in area: no plants in the water

Water depth: 4-6 feet

River Bottom – muddy (sticky), rocky

Plants – algae covered rocks in area

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 9:30 AM |  | 20C 68F |  |  |
|  | 11:00 AM |  | 23C 73.4F |  |  |
| Wind Speed | 2 Beaufort |  | 4-6 knots | Light breeze |  |
| Cloud Cover | Little cloudy |  |  |  |  |
| Weather today | Toward 12:30 PM turned a little overcast |  |  |  |  |
| Weather recently | No rain - dry |  |  |  |  |
| Water calm | yes |  |  |  |  |
| Water Temperature | 9:25 AM | 20 C 22 C 21 C |  | 68F 72 F 70F | 21C 70F |
|  | 10:05 AM | 22 C 23 C 22.5 C |  | 71.6F 73.4 F 72.5F | 22.5C 72.5F |
|  | 10:58 AM | 22C 23 C 22.5 C |  | 71.6F 73.4F 72.5F | 22.5C 72.5F |
| Turbidity – sight tube | 10:00 AM | 25.7 cm 23.5 cm 23.2 cm |  |  | 36.2 cm |
|  | 11:04 | 32.3 cm |  |  |  |
| Chlorophyll | 9:34 AM 10:12 AM 12:56 PM | 2.0 5.0 2.0 |  |  |  |
| Chemical |  |  |  |  |  |
| DO (drop count kit) | 10:23 AM | 22.5C |  | 9 mg/L | 100% saturated |
| pH - meter | 10:10 AM | 6.025 |  |  |  |
|  | 10:55 AM | 6.10 |  |  |  |
|  | 11:15 AM | 8.4 |  |  |  |
| Phosphate | 12:30 PM | 5.9 mg/L |  |  |  |
| Nitrate | 12:30 PM | 1.7 mg/L |  |  |  |
| Alkalinity | 10:58 | 0 |  | 0 | 0 |
| Salinity - hydrometer | 9:54 AM | 16 ppt |  |  | 16 ppt |
|  | 10:01 AM | 16 ppt |  |  |  |
|  | 10:58 AM | 16 ppt |  |  |  |
| Fish Catch | Number Caught | Species |  | CPUE | Seine |
| Seine net 30 feet X 4 ft., 1.4 inch mesh 34 ft. pull | 4 | Bay Anchovy |  |  |  |
| 12 seines | 43 | Atlantic Silverside |  |  |  |
|  | 2 | White perch |  |  |  |
|  | 6 | Blue Crabs |  |  |  |
|  | 1 | Grass shrimp |  |  |  |
|  | 1 | Hog choker |  |  |  |
|  | 5 | Striped Bass |  |  |  |
|  | 1 | Northern Pipefish |  |  |  |
|  | 1 | Winter flounder |  |  |  |
| Tides | 9:56 |  |  |  |  |
|  | 10:16 | 21 cm |  | rising | *Differences in measures duing rising tide can be due to wave action |
|  | 10:21 | 30 cm |  | rising |  |
|  | 10:51 | 20.5 cm |  | rising |  |
|  | 11:09 | 32 cm |  | rising |  |
|  | 11:28 | 32 cm |  | rising |  |
| Currents |  |  |  |  |  |
|  | 10m/120 secs. |  |  | 4-6 knots | North/Ebb |
|  | 10m/180 secs |  |  |  | North/Ebb |
| Traffic | 9:37 AM | tugboat |  | southbound |  |
|  | 9:45 AM | Tugboat |  | northboat |  |
|  | 10:50 | barge |  | southbound |  |
|  | 11:01 | Small barge |  | northbound |  |
| Other Items | Yacht | Police boat |  |  |  |

| SEINE# | TIME | CATCH | Pull length | CPUE |
| --- | --- | --- | --- | --- |
| 1 | 9:20 AM | 17 | 34 ft. pull |  |
| 2 | 9:25 AM | 7 |  |  |
| 3 | 9:38 AM | 23 |  |  |
| 4 | 10:00 AM | 2 |  |  |
| 5 | 10:05 | 5 |  |  |
| 6 | 10:09 | 0 |  |  |
| 7 | 10:14 | 1 |  |  |
| 8 | 10:21 | 3 |  |  |
| 9 | 10:27 | 6 |  |  |
| 10 | 10:51 | 11 |  |  |
| 11 | 11:00 | 7 |  |  |
| 12 | 11:03 | 0 |  |  |