Snapshot Day 10/2/07 Data

(Salt Front RM 78.2)

RIVER MILE 41

Steamboat Dock Park, Verplanck, Westchester

Latitude N 4114’.958”    Longitude W 7357’.876"

Beth Rhines, Teatown Lake Reservation

Bob Connick & Mahopac High School 9th - 12th graders – 40 students

Location: Steamboat Dock Park beach, City of Verplanck in Westchester County

Area: located in a small cove between a point and a bulkhead – sampled approximately 30 ft. from south end of dock/parking lot

Surrounding Land Use:  Urban/residential 10% , Forested 10%, beach 10%, Industrial commercial 70%.  Some buildings with large parking lots in front to the north.  Beach, a street with houses with some trees to the south

Sampling Site: beach, riprap, banks altered

Plants in area: mostly washed up on shore

Water Chestnut (Trapa natons) seedpods on shoreline

Vallisinaria – water celery

Land Plants include:  Black locust

Virginia Creeper

Poison Ivy

Riverbank – wild grape

Some type of burdock–

Silverrod

Indian Hemp

Hedgebind weed

Water depth:

River Bottom – sandy bottom, muddy

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 11:55 AM |  | 28.4C |  |  |
| Wind Speed | 1-3 Beaufort |  | W/SW |  |  |
| Cloud Cover | Partly cloudy |  |  |  |  |
| Weather today | Clear & warm |  |  |  |  |
| Weather recently | Cloudy and warm |  |  |  |  |
| Water Temperature | 11:39 AM | 24.7C 24.3C 24.6C |  |  | average 24.5C |
|  | 12:04 PM | 24.7C 24.7C 24.8C |  |  | Average 24.7C |
| Turbidity | 12:30 | 6.4 NTU |  |  | meter |
| Water too shallow off dock to use secchi – could see bottom |  |  |  |  |  |
| Chlorophyll |  |  |  |  |  |
| Core | 13.5 cm Total | 7.5 cm top layer |  | 6 cm bottom layer |  |
|  | Clay rare | Mud rare |  | Sand common | Brick abundant |
| Chemical |  |  |  |  |  |
| DO (drop kits) | 11:25 AM -12:18 PM | 6.67 mg/L |  | 24.5C | 77 % |
| pH Indicator sol | 11:25 AM | 7.7 8.0 8.0 |  |  | Average 7.9 |
| Phosphate |  |  |  |  |  |
| Nitrate |  |  |  |  |  |
| Alkalinity |  |  |  |  |  |
| Salinity Drop test | 11:39 AM – 12:04 PM | 7.2 ppt 7.2 ppt 7.4 ppt |  |  | 7.3 ppt average |
| Fish Catch | Number Caught | Species |  | CPUE | Seine |
| 20 ft net – 3 seines | 12 | Striped bass |  |  |  |
|  | 1 | Northern pipefish |  |  |  |
|  | 180 | Atlantic silversides |  |  |  |
|  | 4 | Blue crab |  |  |  |
|  | 25 | Comb Jelly |  |  |  |
| Tides |  |  |  |  |  |
| 5.5 cm rise in 15 mins. - .37cm/min | 11:50 AM | 10 cm |  |  |  |
|  | 12:05 PM | 15.5 cm |  | rising |  |
| Currents |  |  |  |  |  |
| Core – Some brick throughout | 19.5 cm | 5.5 cm oxidized top – top sandy - lighter |  | 14.0 cm second layer – fine particles (clay?) | Common: Clay Sand brick |
| Traffic |  |  |  |  |  |
| Other Items |  |  |  |  |  |