Snapshot Day 10/2/07 Data

(Salt Front RM 78.2)

RIVER MILE 76

Waryas Park, Poughkeepsie, Dutchess County

Lisa DiMarzo, Mid-Hudson Children’s Museum

Jon Sokol, Sheafe Road Elementary School 4th grade  - 95 students

Location: Near the pavilion structure just south of Fallkill Creek mouth at Waryas Park.

Area: public park

Surrounding Land Use: 80 % urban/residential; 20% Industrial/commercial;

Sampling Site: shoreline is riprap, collected wood and debris; mouth of the Fallkill enters river here.

Plants in area: ~ 20% of area has plants in the water.

Tidal Shallows: Water celery

Water depth:  not noted

River Bottom –muddy past rip rap, sandy, debris at creek mouth

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 10:07 AM |  | 17.5C 59.1 F | 12:25 PM | 22.5C 72.5 F |
| Wind Speed | 1-3 knots |  | Light air | Beaufort force #1 |  |
| Cloud Cover | Partly cloudy |  |  |  |  |
| Weather today | sunnier later in day |  |  |  |  |
| Weather recently | No rain – sunny, seasonable days |  |  |  |  |
| Water Temperature | 1:00 PM | 71 F 21.6C |  |  |  |
| Turbidity | 10:47 AM | 40 JTU |  | 40 JTU | 40 JTU |
| Chlorophyll | 10:30 AM | 0.3 |  |  |  |
| Chemical |  |  |  |  |  |
| DO (Test tabs) | 1:15 PM | 7 mg/L |  | 21.6C | 80% |
| * testabs not very accurate |  |  |  |  |  |
| pH – test strips | 11:30 | 7 |  | 7 | 7 |
| Phosphate |  |  |  |  |  |
| Nitrate |  |  |  |  |  |
| Alkalinity |  |  |  |  |  |
| Salinity - Quantab testtabs- low chloride | 11:30 AM | 1.6 1.6 1.6 |  | 34 ppm | 45 ppm Cl-/.007 %NaCl |
| Fish Catch 30 ft X 20 ft pull | Number Caught | Species |  | CPUE | Seine 4 pulls |
|  | 3 | American Eel |  |  |  |
| 11:15 AM 11:30 AM 11:45 AM 12:00 noon | 55 | Spottail shiners |  |  |  |
|  | 12 | Striped bass |  |  |  |
|  | 18 | Sunfish |  |  |  |
|  | 6 | Banded Killifish |  |  |  |
|  | 16 | Herring |  |  |  |
|  | 6 | Hogchockers |  |  |  |
|  | 2 | Smallmouth Bass |  |  |  |
|  | 8 | Tesselated Darters |  |  |  |
|  | 4 | White Perch |  |  |  |
|  | 2 | Largemouth Bass |  |  |  |
|  | 2 | Bluecrab |  |  |  |
|  | 4 | Isopods |  |  |  |
|  | 1 | Crayfish |  |  |  |
|  | 4 | Isopods |  |  |  |
|  | 30 | Scuds |  |  |  |
| Tides | 10:07 AM |  |  | Falling |  |
|  | 10:22 AM |  |  | Falling |  |
|  | 10:44 AM |  |  | Falling |  |
|  | 11:04 AM |  |  | Falling |  |
|  | 11:44 AM |  |  | Falling |  |
|  | 12:05 PM |  |  | Slack |  |
| Currents |  |  |  |  |  |
| Mouth of the Fallkill makes it difficult to do current reading |  |  |  |  |  |
| Traffic |  |  |  |  |  |
| Other Items | On the third seine found a bright orange construction cone! Also pulled piling wood pieces from net. |  |  |  |  |