Snapshot Day 10/2/07 Data

(Salt Front RM 78.2)

RIVER MILE 300

Henderson Lake/Hudson River

Sarah Fink, Minerva Central School District

10-12th graders, 10 students

4405.467’ N, 7403.353’W

Location: Outflow of Henderson Lake and Calamity Brook.

Area:

Surrounding Land Use: Forested 100%; Interesting to note that ½ mile downstream is the mining (iron) site at Tahawus, where a dam used to support a small lake, and ½ mile upstream is a dam at Henderson Lake

Sampling Site: Covered with vegetation – brush and trees, some driftwood on shoreline, rocks

Plants in area: The shoreline is planted with trees, shrubs and flowers.

Water depth: 5cm in shallows to 150 cm in deep pools

River Bottom –rocky

Plants in water – none – only aquatic vegetation is a small amount of algae on the exposed rocks near shore

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 9:45 AM |  | 60 F 15.5 C |  |  |
| Wind Speed | 9:45AM |  | 4-6 kts | Variable direction |  |
| Cloud Cover | overcast |  |  |  |  |
| Weather yesterday | Fairly cloudy to clear last 3 days, all temps in high 60’s |  |  |  |  |
| Water Temperature | 9:45 AM | 15.2 C 59.4  F |  |  | All 3 trials the same |
|  | 10:05 AM | 15.4 C 59.7  F |  |  | All 3 trails the same |
| Water surface | Water choppy due to gradient not wind distrubance |  |  |  |  |
| Turbidity Large sight tube | Found unlimited transparency to the water, even with a large sight tube. The water had very little sediment |  |  |  |  |
| Chlorophyll | Time | Chla/ug/L |  | Pheo ug/L | Location |
| Sample run by Darrin Freshwater Institute | 9:45 AM | 1.03 ug/L 2.14 ug/L |  | 0.90 ug/L 1.21 ug/L | Upper Hudson River |
| Chemical |  |  |  |  |  |
| DO (ampules) | 10:05 AM | 7 mg/L shallow pool |  | Temp 15 C | 70% saturated |
|  | 10:05 AM | 10 mg/L deep pool |  | Temp 15 C | 95% saturated |
| pH | Time | Acid Rain Paper |  | Universal Ind. Soluiton | pH test strips |
|  | 10:15 AM | 5.5 |  | 6.0 | 6.2 |
| Phosphate | 10:30 AM | No color change in test |  | 0.0 mg/L |  |
| Nitrate | 10:30 AM | No color change in test |  | 0.0 mg/L |  |
| Alkalinity | n/a |  |  |  |  |
| Salinity | Time | Method |  | Reading |  |
| Fish Catch | Number Caught | Species |  |  | Minnow traps |
| Kick nets & overturning rocks | 2 | Dragonfly nymphs |  | Spent 30 minutes looking but it was a relatively, deep, fast-moving area |  |
| Tides |  |  |  |  |  |
| Currents | 5700cm/60 secs | 95cms/sec. |  | South/ebb | 114 kts. In steep areas |
|  | 660cm/60 secs. | 11cm/sec |  | South/ebb | 13.2 kts in shallower areas |
| Traffic |  |  |  |  |  |