Snapshot Day 10/2/07 Data

(Salt Front RM 78.2)

RIVER MILE 7

79th Street Boat Basin

Norman & Joyce Baron, The School in the Forest @ Black Rock Forest

Judy Gutierrez & Diana Santos,  PS/IS 311 4th grade – 44 students

Location: 79th street boat basin.

Area: On docks

Surrounding Land Use: Urban/residential 85%; Commercial (restaurant) 5% ;

Sampling Site: There are rock and timber bulkheads forming the shoreline.  The Marina is made of some stationary and some floating docks.

Plants in area:

The shoreline is planted with trees, shrubs and flowers.

Water depth: 5-8 ft. at sampling site – deeper at end of pier

River Bottom –unable to determine

Plants in water - none

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 10:00 AM |  | 70 F | 1:00 PM | 74 F |
| Wind Speed | 10:00 AM |  | 1-3 mph | 1:00 PM | 4=6 mph |
| Cloud Cover | Partly cloudy |  | Warmer & a bit breezier during the day |  |  |
| Weather yesterday | Clear & dry |  |  |  |  |
| Water Temperature | 11:12 Am | 18.3 C 65  F |  |  |  |
|  | 12:12 PM | 18.3 C 65  F |  |  |  |
|  | 12:40 PM | 18.3 C 65  F |  |  |  |
|  | 1:01 PM | 18.3 C 65  F |  |  |  |
|  | 1:20 PM | 18.3 C 65  F |  |  |  |
| Turbidity | 10:30 AM-1:00 PM | 2.5 ft. |  | X 4 tests | secchi |
| Chlorophyll | 10:45 Am – 1:00 PM | 3 tests |  | .3 and .5 |  |
| Chemical |  |  |  |  |  |
| DO (drop count kit) | 10:45 AM | 5 mg/L |  | Temp 18.3 C | 60% saturated |
|  | 11:12 AM | 6 mg/L |  | Temp 18.3 C | 65% saturated |
|  | 12:12 PM | 6 mg/L |  | Temp 18.3 C | 65% saturated |
|  | 12:40 PM | 6 mg/L |  | Temp 18.3 C | 65% saturated |
|  | 1:01 PM | 6 mg/L |  | Temp 18.3 C | 65% saturated |
|  | 1:20 PM | 6 mg/L |  | Temp 18.3 C | 65% saturated |
| pH | Time | Digital Probe |  | Colorimeter |  |
|  | 10:32 AM | 7.7 |  | 7.3 |  |
|  | 11:09 AM | 7.4 |  | 7.7 |  |
|  | 12:05 PM | 7.5 |  | 7.5 |  |
|  | 12:30 PM | 7.4 |  | 7.5 |  |
|  | 1:10 PM | 7.4 |  | 7.5 |  |
|  | 1:23 PM | 7.3 |  | 7.3 |  |
| Phosphate | n/a |  |  |  |  |
| Nitrate | n/a |  |  |  |  |
| Alkalinity | n/a |  |  |  |  |
| Salinity | Time | Method |  | Reading |  |
|  | 10:45 | High range test strip |  | 9.0 | This test seems to not be working in this high a level of salinity – use data from hydrometer instead |
|  | 11:11 | High range test strip |  | 8.8 |  |
|  | 12:05 | High range test strip |  | 9.3 |  |
|  | 12:35 | High range test strip |  | 8.8 |  |
|  | 1:05 | High range test strip |  | 9.1 |  |
|  | 1:27 | High range test strip |  | 9.1 |  |
|  | 10:45 | Hydrometer |  | 1.15 spec. gravity | 20.1 ppt |
|  | 12:45 | Hydrometer |  | 1.15 spec. gravity | 20.1 ppt |
|  | 1:00 | Hydrometer |  | 1.14 spec. gravity | 18.8 ppt |
| Fish Catch | Number Caught | Species |  |  | Minnow traps |
| Crab pot | 1 | Small clear jelly fish |  |  |  |
| Tides | 8:00 | 0 |  |  |  |
|  | 10:30 AM | 30 inches (76.2 cm) |  |  | rising |
|  | 11:00 AM | 36 inches (91.44 cm) |  |  | rising |
|  | 11:50 AM | 42 inches (106.68 cm) |  |  | rising |
|  | 12:50 PM | 46 inches (116.84 cm) |  |  | rising |
|  | 1:20 PM | 47 inches (119.38 cm) |  |  | rising |
|  | A yardstick was nailed to a piling at 8:00 AM with the bottom of the stick at water surface level. Students observed how the water moved up the yardstick until the top of the yardstick was 11 inches underwater at 1:20 PM. |  |  |  |  |
| Currents | 11:15 Am | 254 cms/minute |  | Northward | Current was measured by dropping popsicle sticks into the River at the end of the pier. |
|  | 1:20 PM | 763 cm/minute |  | Wind had picked up and was from the south – may have effected rate of movement north |  |
| Traffic |  |  |  |  |  |