Snapshot Day 10/7/08 Data

(Salt Front RM 63.4)

RIVER MILE 14

Inwood Hill Park Nature Center, NYC

Sgt. Taylor, New York City Park Rangers

Matthew

Susan Vincent - Young Women’s Leadership School – 12th grade – 24 students

Latitude 4052.509’N/ Longitude 7355.28’W

Location: In the cove of Inwood Hill Park

Area: Forested

Surrounding Land Use: Forested 85%; urban residential 5%; Industrial/commercial 2%; 8% estuary: pipes that exit sewage, drainage, main pipes.  Forested and used by people for picnics, jogging, sitting on park benches.

Sampling Site: large riprap on shoreline, drainage pipes in area, vegetation

Plants in area: Phragmites australis 10%	, Spartina Alterniflora 10%, most of plants unidentified

Water depth: 15-100 cm where seining

River Bottom – mostly mud with large rocks, some vegetation

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 9:30 AM |  | 57 F | 1:00 PM | 67 F |
| Wind Speed | 2.5 Knots |  | NW |  |  |
| Cloud Cover | clear |  |  |  |  |
| Weather today | Temperature warmed during day, wind calmed |  |  |  |  |
| Weather recently | Sunday light rain, cloudy; Monday sunny but cool |  |  |  |  |
| Water Temperature | 11:16 AM | 17.8 C 18.2 C 17.6 C |  |  | 17.9C |
| Water surface: | calm |  |  |  |  |
| Turbidity (long Site tube)_ | 11:30AM | 57.2 cm 75.0 cm 75.6 cm 84.0 cm 86.0 cm 97.0 cm |  |  | Average 79.1 cm |
| Chlorophyll | 12:30 PM | 2.0 water’s edge |  | 0.7 10 m from edge |  |
| Chemical |  |  |  |  |  |
| DO (drop count kit) | 11:16 AM | 7.42 mg/L |  | Temp.17.8 C | 75% |
| pH | 12:38 AM | 7.4 7.6 7.8 |  | 7.6 avg. | Indicator sol. |
| Phosphate |  |  |  |  |  |
| Nitrate | 12:56 PM | 5.0 0 0 |  |  | 1.67 mg/L |
| Alkalinity |  |  |  |  |  |
| Salinity | 11:25 AM | 9.6 ppt 6.5 ppt 7.1 ppt |  | 7.7 ppt average | YSI meter |
| Conductivity | 11:16 AM | 14.42 mS 7.46 mS 9.98 mS |  | 10.62 mS | meter |
| Fish Catch | Number Caught | Species |  | CPUE | Seine |
| Used seine 12 ft. Pull 18 ft. - | 2 | Grass Shrimp |  |  | Seine net |
| Seined – 3 | 2 | Atlantic Silverside |  |  |  |
|  | 2 | Blue crab |  |  |  |
|  | 17 | mummichogs |  |  |  |
| 12:30 PM | 7 catch | 15 ft. pull |  | 1.3 CPUE | Seine 1 |
| 12:45 PM | 4 catch` | 15 ft. pull |  | 0.8 CPUE | Seine 2 |
| 1:00 PM | 19 catch | 15 ft. pull |  | 3.7 CPUE | Seine 3 |
| Tides | 11:25 AM | 15cm |  | Rising |  |
|  | 11:40 AM | 24cm |  | Rising |  |
|  | 11:55 AM | 28cm |  | Rising |  |
|  | 12:10 PM | 33 cm |  | Rising |  |
|  | 12:25 PM | 38 cm |  | Rising |  |
| Currents | 11:25 AM | 480 cm/60 secs |  | 8cm/sec | 0.16 knots |
|  | 12:35 PM | 490 cm/60 secs |  | 8.2 cm/sec | 0.163 knots |
| Traffic | 12:03 PM | Circleline |  | Northbound | Loaded - people |
| Sediment Core description | 19 cm core | No layering |  | Very fine silt | Collected 10 m from shore |
| Other observations |  |  |  |  |  |