Snapshot Day 10/7/08 Data

(Salt Front RM 63.4)

RIVER MILE 144

Rensselaer Boat Launch, Forbes Rd., Rensselaer, NY

Doug Reed, Hudson Basin River Watch

Leah Penniman, Tech Valley High School - 9th grade – 38 students

Location: Rensselaer Boat Launch

Area: Boat launch – beach area

Surrounding Land Use:

Sampling Site:

Plants in area:

Water depth: unknown

River Bottom :

Times: All sampling 9:30-10:30 AM

| ITEM | Readings |  | Readings | Average Reading | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 12C 12C 11C 13.4C |  | 12C 15C 13.4C 13C | 12.725 C |  |
| Wind Speed |  |  |  |  |  |
| Cloud Cover |  |  |  |  |  |
| Weather today |  |  |  |  |  |
| Weather recently |  |  |  |  |  |
| Water Temperature | 15C 14C 16C 15C 14C | 15C 10C 14.6C 13C 14C |  | 14.06 C |  |
| Turbidity (LaMotte) | 10 JTU 10 15 20 25 |  |  | 16 JTU |  |
| Turbidity Turbidity tube | 120 cm 41 120 248 90 | 127 cm 117 54 64.6 |  | 109.1 cm |  |
| Chlorophyll |  |  |  |  |  |
| Chemical |  |  |  |  |  |
| DO (chemet color comparator) | 9 mg/L 9 6 6 9 | 9 mg/L 5 6 10 8 |  | 7.7 mg/L | 62% |
| pH – pH meter | 7.01 7.89 7 8 6.65 | 5 5 7 7 |  | 6.7 |  |
|  |  | 6.9 |  |  | strips |
| Phosphate - Chemet Color comparator | 1 mg/L 0.1 1 0.1 0.1 | 1 mg/L 0 0 0.15 |  | 0.38 mg/L |  |
| Nitrate – LaMotte color comparator | 1 ppm 4.4 4.4 1 4.4 | 1 ppm 1 8.8 8.8 8 |  | *5.18 ppm | * Test was not working correctly - discount results |
| Alkalinity (LaMotte) | 70 ppm *<1 ppm 40 80 80 | 100 ppm 140 |  | 85 ppm | *Did not include <1 ppm result in the average calculation |
| Salinity* (reported as ppt – too high a reading for this area) | 0.2 0.1 0.3 0.375 0.1 | 0.5 0.375 0.3 0 0.1 |  | *0.235 ppt | * This salinity result is a bit high for this area of the river. Plan to change test. |
| Conductivity | 248 128 268 118 | 128 246 250 |  | *173.25 uS |  |
| TDS | 128 124 128 235 | 258 125 125 |  | 160.43 ppm |  |
| Coliform Petrifilm plate 1 ml water • stated range is based on a 100 ml sample | 11 13 14 13 18 | 15 20 14 17 14 |  | *14.9 Colonies per ml water | * Acceptable range more than 20% of the samples NOT in excess of 2,400-5,000 |