Snapshot Day 10/8/09Data

(Salt Front RM 60.3)

RIVER MILE 14

Inwood Hill Park Nature Center, NYC

New York City Park Rangers

Susan Vincent - Young Women’s Leadership School – 12th grade – 24 students

Latitude 4050’N/ Longitude 7355 W

Location: In the cove of Inwood Hill Park

Area: Forested

Surrounding Land Use: Forested 60%; urban residential 10%; Industrial/commercial 10%; 20% paved. Pier, bulk-headed, rip rap shoreline.  Forested and used by people for picnics, jogging, sitting on park benches.

Sampling Site: large riprap on shoreline, drainage pipes in area, vegetation

Plants in area: Yes less than half area has plants.  10% of water area has plants.

Water depth: 55 cm @ 10:30 AM; 105 cm @ 12:30 PM

River Bottom – mostly mud with large rocks, some vegetation

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 10:58 AM |  | 68 F 20 C | 11:30 AM | 68 F 20 C |
| Wind Speed | 2 Beaufort |  | 6-11 kts | 4-6 mph | North |
| Cloud Cover | Partly cloudy |  |  |  |  |
| Weather today | Temperature cooled during day as wind picked up |  |  |  |  |
| Weather recently | Windy and some rain |  |  |  |  |
| Water Temperature | 11:10 AM | 17.9 C 17.6 C 18.3 C |  | 64.22F 63.68 64.94 | 17.93C 64.28 F |
|  | 11:40 AM | 18.0 C 18.4 C 18.5 C |  | 64.40F 65.12 65.13 | 18.3C 64.94 F |
|  | 12:10 AM | 18.7 C 18.8 C 18.6C |  | 65.99F 65.84 65.98 | 18.7C 65.66F |
|  | 12:23 PM | 18.5 C 18.4 C 18.5C |  | 65.3F 65.3 65.12 | 18.48C 65.24 F |
| Water surface: | choppy |  |  |  |  |
| Turbidity (long Site tube)_ | 10:50 AM | 41.5 cm 39.75 cm 33.5 cm |  |  | Average 38.25 cm |
| Chlorophyll | 11:30 PM | 0.1 |  |  |  |
| Chemical |  |  |  |  |  |
| DO meter | 11:28 AM | 8.46 mg/L |  | Temp.18.4 C | 84.7% |
| pH | 11:30 AM | 7.7 7.6 7.7 |  | 7.7 avg. | meter. |
| Salinity | 11:28 AM | 9.2 ppt 8.8 ppt 10.1 ppt |  | 7.7 ppt average | YSI meter |
| Conductivity | 11:28 AM | 13.18 mS 13.59 mS 14.92 mS |  | 13.9 mS | meter |
| Fish Catch | Number Caught | Species |  | CPUE | Seine |
| Used seine 20 ft. Pull 30 ft. - | 1 | White perch |  | 12 inches | + minnow traps (nothing caught in traps) |
| Seined – 2 times | 2 | Blue Crab |  |  |  |
| Tides | 11:15 AM | 90 |  | Rising |  |
|  | 11:45 AM | 100 |  | Rising |  |
|  | 12:15 PM | 105 |  | Rising |  |
| Currents | 11:15 AM | 923 cm/60 secs |  | 815.38cm/sec | 0.30 kts. S/Flood |
|  | 12:00 PM | 923 cm/60 secs |  | 815.38cm/sec | 0.30 kts. S/Flood |
| Traffic | 11:20 AM | Circleline |  | Northbound | Loaded - people |
| Sediment Core description | 7 inch core | 0.9 top |  | 5.7 inch bottom |  |
| Other observations |  |  |  |  |  |