Snapshot Day 10/8/09 Data

(Salt Front RM 60.3)

RIVER MILE 55

Little Stony Point

Pete Salmansohn & Lisa Mechaley of the Hudson Highlands Land Trust

Mark Patinella, Highland High School

17 students - 12th grade

Location: Little Stony Point beach, Cold Spring, Putnam County

Area: On the beach at Little Stony Point, facing Beacon across from Storm King

Surrounding Land Use: 50% forested, 50% beach

Sampling Site: beach area

Plants in area: no plants in water

Water depth: 45 cm

River Bottom –

| ITEM | Time |  | Reading 1 |  | Reading 2 |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |
| Air Temperature | 10:37 |  | 15C 59F |  | 12:33PM |  | 16C 61F |
| Wind Speed | 8 Beaufort |  | 7.5 kts |  |  |  | North |
| Cloud Cover | Partly cloudy |  |  |  |  |  |  |
| Weather today | Overcast, no rain, windy |  |  |  |  |  |  |
| Weather recently | Rain and significant wind over last 3 days |  |  |  |  |  |  |
| Water Temperature | 12:50 PM | 17.5C 63F |  |  |  |  |  |
| Water | Choppy |  |  |  |  |  |  |
| Turbidity – drop kit | 11:32 AM | 72 JTU 50 JTU 65 JTU |  |  | 62.3 average JTU |  |  |
| Chlorophyll |  |  |  |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |
| DO (ampules) | 12:10 PM | 17C |  |  | 8 mg/L |  | 70% saturated |
| pH | 10:48 AM | 7.3 |  |  |  |  |  |
|  | 11:23 AM | 7.5 |  |  |  |  |  |
|  | 12:10 PM | 7.5 |  |  |  |  |  |
| Phosphate |  |  |  |  |  |  |  |
| Nitrate | 10:37 AM | 0.5 ppm 1.0 ppm |  |  |  |  | 0.75 ppm Average |
| Alkalinity |  |  |  |  |  |  |  |
| Salinity Test strips | 10:56 AM | 1.8 quantab unit 1.8 1.8 |  |  | 53 ppm chloride |  |  |
|  | (per Tom Lake) | <1.0 ppt |  |  |  |  |  |
| Fish Catch | Number Caught | Species |  |  | CPUE |  | Seine |
| Seine net 20 feet , X 4 ft. X 50 yrd pull |  |  |  |  |  |  |  |
| Also dip net | 2 | Pumpkinseed |  |  |  |  |  |
|  | 1 | Spottail shiner |  |  | 13.5 cm |  |  |
|  | 1 | White Perch |  |  | 6 inches |  |  |
|  | 2 | Bullhead |  |  |  |  |  |
|  | 1 | Herring |  |  |  |  |  |
|  | 1 | Striped Bass |  |  |  |  |  |
| Tides | 10:20 AM | rising |  |  | 5.1 cm |  |  |
|  | 10:40 AM | Rising |  |  | 12.7 cm |  | .38 cm/min |
|  | 11:15 AM | Rising |  |  | 22.9 cm |  | .29 cm/min |
|  | 11:40 AM | Rising |  |  | 35.6 cm |  | .5 cm/min |
|  | 12:11PM | Rising |  |  | 45.7 cm |  | .34 cm/min |
|  | 12:28 PM | Rising |  |  | 48.3 cm |  | .15 cm/min |
|  | 12:47 PM | Rising |  |  | 53.3 cm |  | .25 cm/min |
| Currents (no time listed) | Time | Cm/60 sec |  | Cm/sec | knots | N/S | Ebb/Flood/Still |
|  | 10:37AM | 720 |  | 12 | .24 | N | Flood |
|  | 11:00AM | 20 |  | .33 | .0066 | S/SW | Flood |
|  | 11:10AM | 144 |  | 2.4 | .046 | SW | Flood |
|  | 12:05PM | 136 |  | 21.7 | 0.434 | S | Flood |
|  | 12:33PM |  |  | 16.3 | .326 | S | Flood |
| Core Description | Rare | Common |  |  | Abundant |  |  |
|  | Mud | Gravel, pebbles |  |  | Sand |  |  |
|  | 7 inches total length | Nothing living in it |  |  | Not much oxidation occurred |  |  |