Snapshot Day 10/8/09 Data

(Salt Front RM 60.3)

RIVER MILE 78

Quiet Cove, Poughkeepsie

Skip Hoover, Cornell Cooperative Extension, Duchess County

Robert Adams & Scott Jameson, DEC Fisheries Dept.

G. W. Krieger Elementary School 4th & 5th students &

Tracy Zeoli, Gayhead School – 76 students total

9-11th grade helpers (8)

41.44°N and 73.56°W

Location: Quiet Cove, Poughkeepsie, Dutchess County

Area: Grassy area with parking lot, beach like waterfront

Surrounding Land Use: Picnics, boating – 60% is beach with rip rap banks, 40% previous industrial site

Sampling Site: An old industrial area that has been turned into a park. Used by people for picnics, fishing.  Shoreline rocky, banks altered with rip rap and wooden bulkhead, boat launch for kayaks and canoes

Plants in area: yes – 5% or area is water celery

Debris in area: remnants of industrial site and some papers in the water

Water depth: 160 cm in the area with concrete embankments

River Bottom – muddy in beach area

| ITEM | Time |  | Reading 1 |  | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |
| Air Temperature | 10:06 AM |  | 15C |  |  |  |
|  | 12:40 PM |  | 20C |  |  |  |
| Wind Speed | 10:39 AM |  | Beaufort #17-20 |  |  | North |
| Cloud Cover | Partly cloudy |  |  |  |  |  |
| Weather today | No rain |  |  |  |  |  |
| Weather recently | Monday sunny, Tuesday rainy, Wednesday very windy but sunny |  |  |  |  |  |
| Water Description | VERY choppy |  |  |  |  |  |
| Water Temperature | 9:45 AM | 15.5C 60F |  |  |  |  |
|  | 10:27 AM | 12.77C 55F |  |  |  |  |
|  | 11:10 AM | 15C 59F |  |  |  |  |
|  | 12:50 PM | 16.1C 61F |  |  |  |  |
| Turbidity | 11:00 AM | 37 cm 37 cm 27 cm 25 c |  | 31.5 cm average |  | Long site tube |
| Chlorophyll | 11:30 AM | 0.4 |  |  |  |  |
| Chemical |  |  |  |  |  |  |
| DO (LaMotte elementary kit tablets) | Very poor readings with kit | <3.0 ppm |  | 15C |  |  |
| pH (LaMotte color match kit) | 11:11 AM | 8 7.5 8 |  |  |  | 7.8 average |
| Nitrates Using LaMotte Elementary test kits | 11:11 Am | <5 ppm |  |  |  |  |
| Phosphates | 11:11 AM | <1 ppm |  |  |  |  |
| Salinity | Not tested |  |  |  |  |  |
| Fish Catch- 25 ft. | Number Caught | Species |  | Macroinvert |  | Seine |
| 3 seines total | 10 | Spottail Shiners |  | 1 Crayfish |  | 9:45 AM |
|  | 6 | Banded killifish |  |  |  |  |
|  | 1 | Pumpkinseed |  |  |  |  |
|  | 1 | Male Bluecrab |  |  |  | TOTAL 18 fish 1 macro |
|  | 2 | Spottail Shiners |  |  |  | 12:30 PM |
|  | 2 | Banded killifish |  |  |  |  |
|  | 6 | Pumpkinseed |  |  |  |  |
|  | 2 | Male Bluecrab |  |  |  |  |
|  | 2 | Bay Anchovies |  |  |  | TOTAL 14 |
| Tides | 9:47 AM | 120 cm |  |  |  |  |
|  | 10:14 AM | 122 cm |  | Falling |  | *measuring down to water level |
|  | 10:27 AM | 1100 cm |  | Rising |  |  |
|  | 10:55 AM | 98 cm |  | Rising |  |  |
|  | 11:11 AM | 93 cm |  | Rising |  |  |
|  | 11:34 AM | 75 cm |  | Rising |  |  |
|  | 12:29 PM | 50 cm |  | Rising |  |  |
|  | 12:55 PM | 35 cm |  | Rising |  |  |
| Currents | 9:47 AM | South |  | Ebb |  |  |
|  | 10:27 AM |  |  | Still |  |  |
|  | 11:11 AM | North |  | Flood |  |  |
|  | 12:29 PM | North |  | Flood |  | 15 ft/22 secs = .68 ft/sec. |
|  |  | NOTE |  | 30 ft/32 secs |  | < 1 ft. per sec. |
| Traffic |  |  |  |  |  |  |
| Sediment Core | 11 inches length | Gray silky smooth much |  | Not much distinction in layers |  | Some roots present |
| Other Items |  |  |  |  |  |  |