Snapshot Day 10/8/09 Data

(Salt Front RM 60.3)

RIVER MILE 96.5

Ulster Landing State Park, Town of Ulster, Ulster Co.

Latitude 42N, Longitude 7356’W

Lia Harris, Cary Institute of Ecosystem Studies

Dixon Onderdonk, Kingston High School 11/12th graders – 12 students

Location: Swimming beach, Ulster Landing Park, Town of Ulster, Ulster County

Area: fishing, swimming beach, has a building with restrooms

Surrounding Land Use: Beach 40%; Forested area 45%, Industrial/Commercial 15%

Sampling Site: beach, altered bank with concrete bulkhead, wood debris driftwood, drainage from bulkhead north of site immediately

Plants in area: yes

Water depth: 3 ft.

River Bottom : sandy, muddy bottom

Plants: Tidal flats – 49% water chestnut; 49% water celery; 2% water milfoil

| ITEM | Time |  |  |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |
| Air Temperature | 9:45 AM |  |  |  | 10.56C 51 F |  |  |
| Wind Speed | 2 Beaufort |  |  |  | 4 kts. | East |  |
| Cloud Cover | partly cloudy |  |  |  | No rain |  |  |
| Weather today |  |  |  |  |  |  |  |
| Weather recently | Few showers and wind |  |  |  |  |  |  |
| Water Temperature | 9:05AM |  |  | 15C 15 15 |  | 59F 59 61 | 15C 59F |
|  | 10:10 AM |  |  | 16C 17 16 |  | 63F 61 63 | 17C 69F |
|  | 11:40 AM |  |  | 19C 19 19 |  | 66F 66 66 | 15C 66F |
| Water Surface | calm |  |  |  |  |  |  |
| Turbidity turbidimeter | 10:35 AM |  |  | 61 NTU 60.2 60.5 |  |  | 60.6 NTU |
| Chlorophyll | 10:17 AM |  |  | 1.0 |  |  |  |
| Chemical |  |  |  |  |  |  |  |
| DO - meter | 10:09AM |  |  | 8.6 mg/L |  | 16C | 85% saturated |
| (Hach DR-890) | 10:09 AM |  |  | 9 mg/L |  | 16C |  |
| pH | 10:20 AM |  |  | 7 8 6.5 |  | 7.17 | Indicator sol. |
|  | 10:00 AM |  |  | 8.01 7.94 8.0 |  | 8.0 | meter |
| Phosphate | 10:30AM |  |  | 2.75 ppm 2.75 ppm 2.75 ppm |  |  | 2.75 ppm |
| Nitrate - | 10:18 AM |  |  | 0.5 ppm |  |  | 0.17 ppm |
| Alkalinity | 10:18 AM |  |  | 164 mg/L 116mg/L 88 mg/L |  |  | 122.7 mg/L |
| Salinity – drop count test kit | 12:00 PM |  |  | 40 ppm 48 ppm 41 ppm |  |  | 43 ppm |
|  | 12:00 PM |  |  | 39 ppm 28 ppm 32 ppm |  | 33 ppm Cl | 41.25 ppm salinity |
| Fish Catch 27.5’seine X 1/18 inch mesh | Number Caught per seine |  |  | Species |  | Length of largest | Seine |
| 7 seines | 3 | 2 |  | Pumpkinseed |  | 11 cm |  |
| Pull 50 ft. | 30 | 19 | 20 | Spottail Shiner |  | 11.5 cm |  |
|  | 15 | 11 | 14 | Bluegill |  |  |  |
|  | 13 | 8 | 4 | Banded Killifish |  | 10 cm |  |
|  | 2 | 3 | 25 | White Perch |  | 14 cm |  |
|  | 1 |  |  | Herring |  |  |  |
|  | 8 | 5 | 10 | Yellow Perch |  |  |  |
|  | 3 | 3 | 3 | Striped Bass |  |  |  |
|  |  | 2 |  | Four spine stickleback |  |  |  |
|  | 8 | 2 |  | Blue Crab |  |  |  |
|  | 3 | 1 | 7 | Crayfish |  |  |  |
| Pull | Time |  |  | Catch |  | CPUE | PULL |
| Seine #1 | 12:00 PM |  |  | 87 fish |  | 0.21 | 50 ft. pull |
| Seine #2 | 12:30 PM |  |  | 56 fish |  | 0.13 | 50 ft. pull |
| Seine #3 | 1:00 PM |  |  | 83 fish |  | 0.19 | 50 ft. pull |
| Tides | 9:00 AM |  |  | Falling |  | 9 cm |  |
|  | 9:15 AM |  |  | Falling |  | 7 cm |  |
|  | 9:30 Am |  |  | Falling |  | 6.5 cm |  |
|  | 9:45 Am |  |  | Falling |  | 2 cm |  |
|  | 10:00 AM |  |  | Falling |  | -5 (away from tide stick |  |
|  | 10:15 AM |  |  | Falling |  | -15 |  |
|  | 10:30 Am |  |  | Falling |  | -40 |  |
|  | 10:45 AM |  |  | Falling |  | -50 |  |
| Currents 8:45 AM | 125 cm/60 secs |  |  | 2:08 cm/sec. |  | 0.04 knots | South Ebb |
| 9:15 Am | 130 cm/60 secs |  |  | 2:16 cm/sec |  | 0.042 knots | South Ebb |
| 9:45 Am | 140 cm/60 secs |  |  | 2:30 cm/sec |  | 0.044 knots | South Ebb |
| 10:15 AM | 133 cm/60 secs |  |  | 2:22 cm/sec |  | 0.043 knots | South Ebb |
| 10:45 AM | 128 cm/60 secs |  |  | 2:13 cm/sec |  | 0.041 knots | South Ebb |
| Traffic | 8:35 AM |  |  | Barge/ |  | Eastern Dawn | South |
|  | 10:17 Am |  |  | US Coast guard |  |  | North |
|  | 11:25 AM |  |  | Barge |  |  | South |
| Other Items | Lots of wrack |  |  |  |  |  |  |
| Core | Length |  |  | Top |  | Bottom |  |
|  | 11 inches |  |  | 4 inch top |  | 7 inch bottom |  |