Snapshot Day 10/14/10

(Salt Front RM 36)

RIVER MILE 4.1

Shay Saleem – Intrepid Sea, Air and Space Museum

Karine Kelley, PS 230/5-219 – district 15

5th grade, 25 students

Latitude N 4045.417 Longitude W07400.11

Location: The Intrepid next to Pier 84

Area: large ship at pier surrounding water access

Surrounding Land Use: 40% urban/residential, 10% forested, 40% industrial commercial

Sampling Site: Pier, Bulkhead (wood timbers), used for boating, banks altered, covered in vegetation

Plants in area: None

Water depth: 15 ft.

River Bottom –muddy, sandy

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 12:20 PM |  | 61F |  |  |
| Wind Speed | Beaufort 1-2 |  | 2-3 mph | From the East to the West |  |
| Cloud Cover | Mostly cloudy |  |  |  |  |
| Weather today | Dry |  |  |  |  |
| Weather recently | Monday thunderstorm, Tuesday drizzling, Wednesday sunny |  |  |  |  |
| Water surface | calm |  |  |  |  |
| Water Temperature | Noon | 24C |  |  |  |
| Turbidity |  |  |  |  |  |
| Chlorophyll |  |  |  |  |  |
| Chemical |  |  |  |  |  |
| DO Color reaction | 12:15 PM | 18C |  | 4 mg/L |  |
|  | 12:23 PM | 18 C |  | 4 mg/L |  |
| pH – Lamotte colormetric | 12:15 PM | 8.0 7.0 8.0 |  |  | Average 7.66 |
|  | 12:30 PM | 8.0 8.0 9.0 |  |  | Average 8.33 |
| Nitrate (color reaction) | 12:24 PM 12:31 12:35 | 0 ppm 0 ppm 0 ppm |  |  | Waiting only one minute on the last test as ran out of time |
| Phosphate (color reaction) | 12:18 PM 12:26 PM 12:34 PM | 1 ppm 0 ppm 0 ppm |  |  |  |
| Salinity (Hydrometer) | 12:17 PM | 10.5 ppt 10 ppt 10 ppt |  |  |  |
|  | 12:31 PM | 10.5 ppt 10.5 ppt 10 ppt |  |  |  |
| Tides |  |  |  |  |  |
| Currents | 12:15 PM | Flowing North |  | Flood |  |
| Boats | 10:46 AM | Circle Line white/green/red |  | Southbound | Empty |
|  | 10:47 AM | Waterways Blue, white |  | Northbound | Full |
| Other Observations | Helicopters | Airplanes |  | kite |  |