Snapshot Day 10/14/10

(Salt Front RM 36.0)

RIVER MILE 13

Bloomers Beach @ Engelwood Boat Basin

Kirk Weber – Meadowlands Environmental Center

Judi Bettencourt – West New York Middle School – 7th & 8th graders

N40.881297	 -W73.943948

|  | Location: Bloomers Beach – north end Area: sandy beach, banks altered, pier with rip rap sections, wood bulkhead, collected wood/debris in area, piping approx. 30” located at sampling site, 2 stone breakwaters as well as on shoreline, + cement storm wall Surrounding Land Use: 20% urban/residential; 40% forested; 20% beach; 20% Industrial/commercial Bottom: sandy & muddy Plants in area: none in the water Water Depth:170 cm at high tide, or 5 ft. 7 inches |
| --- | --- |

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 10:15 AM |  | 18.8C |  |  |
|  | 11:30 AM |  | 18.3C |  |  |
|  | 12:30 PM |  | 16.6C |  |  |
| Wind Speed | Beaufort #2 |  | 5.82 kts 7.21 kts | 6.7 mph | NE |
| Cloud Cover | Sky became increasingly cloudy throughout the observation period |  |  |  |  |
| Weather today | No precipitation |  |  |  |  |
| Weather recently | Nothing unusual to note |  |  |  |  |
| Water | Choppy |  |  |  |  |
| Water Temperature | 9:50 AM | 16.9C 17.7C 18.3C |  | 62.4 F 63.9F 64.8F | 17.6C 63.7F |
|  | 11:15 AM | 18.3C 18C 18C |  | 64.9F 64.3F 64.3F | 18.1C 64.5F |
|  | 12:05 AM | 18.3C 18.4C 18C |  | 64.9F 65.1F 64.4F | 18.2C 64.8F |
| Turbidity – long site tube | 11 AM | 41cm 37 cm 44 cm |  |  | 40.6 cm |
| Chemical |  |  |  |  |  |
| Chlorophyll | 8;40 AM | 0.5 |  |  |  |
| DO – ampules | 11:45AM | 8 mg/L |  | 18.1 C | 82.5% saturated |
| pH – indicator sol. | 12:25 PM | 8.0 7.5 8.0 |  |  | 7.8 avg. |
| Salinity (hydrometer) | 12:40 PM | 6 ppt 6 ppt 6 ppt |  |  | 6ppt average |
| Fish Catch | Count | Fish |  | Size of largest | Macros |
| 14 ft. X 3 ft seine – pulled 20 ft. | 50 plus | Atlantic Silverside |  | 9 cm | Blue Crab |
| 3/16 inch mesh | 1 | Striped Bass |  | 6.5 cm | Moon Jellies |
|  | 1 | White Perch |  | 9 cm | Shore shrimp |
| Seine #1 | 11:00 AM | 15 fish |  | 20 ft pull | .18 |
| Seine #2 | 11:45 AM | 11 fish |  | 20 ft pull | .13 |
| Seine #3 | 12:15 PM | 12 fish |  | 20 ft pull | .14 |
| Tides | 8:50 | falling |  | 47.5 cm |  |
|  | 10:15 | rising |  | 65 cm | .21 cm/min |
|  | 11:20 | rising |  | 75 cm | .15 cm/min |
|  | 12:15 | rising |  | 97.5 cm | .41 cm/min |
| Currents | 11:30 | 13.6 cm/sec |  | North | Flood |
|  | 12:25 | 30 cm/sec |  | North | Flood |
| Core – | 20 cm total – overall black in color | Top layer 6 cm – dark blackish, pudding like |  | Second layer 14 cm –clayey |  |
| Boats |  |  |  |  |  |