Snapshot Day 10/14/10 Data

(Salt Front RM 36)

RIVER MILE 31 – Hook Mountain State Park

Laurie Seeman & Joanne Dickey– Strawtown Art & Garden Studio

Lorraine Marden, Jennifer Armoceda and Natalie Harris -Blue Rock School –

4th, - 8th  – 25 students

40°7.18N Latitude - 73°54.76 W Longitude

Location: Nyack Beach near Hook Mountain Sandy Beach right side of riverfront area

Area: Park/Boat launch, sandy beach area on the point with riprap

Surrounding Land Use: 100% beach

Sampling Site: Sandy beach – with a lot of trees, picnic tables, sandy and rocky at edges, covered with vegetation beyond the beach, stone wall, riprap shore, collected wood debris –not many houses.

Piping coming into river south of sampling site

Plants in water: 1% water celery

Plants in area: Japanese Knotweed 50%; Sumac

Water depth: 36 inches

River Bottom –rocky

Student site description: I see lots of trees. I see some evergreens and willows.  Its pretty rocky and sandy on the beach, and the water is pretty rough.  The water is murky and you can’t see through it.  I think this place is used for fun!  Wavy water.

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 10:22 AM |  | 12.8°C 55.2°F |  |  |
|  | 1:15 PM |  | 14.46°C 58°F |  |  |
| Wind Speed | Beaufort 2-5 |  |  | N |  |
| Cloud Cover | Mostly cloudy |  |  |  |  |
| Weather today | It started cloudy and clouds increased during the day |  |  |  |  |
| Weather last 3 days | Rain on Monday (1.2 inches); Tuesday 64° and sunny; Wednesday 59° and Sunny. |  |  |  |  |
| Water | Choppy - wavy |  |  |  |  |
| Water Temperature | 10:20 AM | 63°F 17.23°C |  | 64.4°F 18°C | 63.7°F 17.62°C |
|  | 11:13 AM | 68°F 20°C |  | 64.4°F 18°C | 66.2°F 19°C |
|  | 12:00 PM | 65°F 18.3°C |  | 68°F 20°C | 66.5°F 19.15°C |
|  | 1:10 PM | 64°F 18°C |  | 64°F 18°C | 64°F 18°C |
| Turbidity | 10:25 AM | 0 |  |  |  |
|  | 11:20 AM | 40 JTU |  |  |  |
|  | 12:00 PM | 40 JTU |  |  |  |
| Chlorophyll | 11:20 AM | 1.0 |  | 12:20 PM | 0.3 |
| Chemical |  |  |  |  |  |
| DO Drop count | 11:20 AM | 4 mg/L |  | 18  C | 50% |
| pH Indicator solution | 10:20 AM | 8.0 8.0 8.0 |  |  | Average 8.0 |
| Phosphate |  |  |  |  |  |
| Nitrates |  |  |  |  |  |
| Salinity Test strip | 10:45 AM | 302 ppm chloride |  | 377.5 ppm | 0.3775 ppt |
| Fish Catch | Seine # | Number Caught |  | Species | Length of largest |
| Seine 12 ft. X 6ft. Mesh 0.25 inch | #1 | 3 |  | * Rainbow Smelt | 2.5 inch |
|  |  | 2 |  | Crab |  |
|  | #2 | 5 |  | Silverside | 2 inch |
|  |  | 4 |  | Crab |  |
|  | #3 | 1 |  | Shiner | 2.5 inch |
| *while not impossible this would be an unusual catch – perhaps Silverside |  | 3 |  | * Rainbow Smelt | 5.5 inch |
|  | #4 | 4 |  | Silverside | 3 inch |
|  | #5 | 5 |  | Silverside | 3 inch |
| Tides | 10:18 AM | 27 cm |  |  |  |
| Low tide here was ~ 10:30 AM | 11:05 AM | 35 cm |  | rising |  |
|  | 11:45 AM | 47 cm |  | rising |  |
|  | 12:30PM | 60 cm |  | rising |  |
| Currents* | CM/60 secs | Cm/Sec |  | North/South Ebb/Flood/Still | Knots |
| 10:20 AM | 0 cm | 0 |  | South (Still) | 0 |
| 11:15 AM | 282 cm | 4.7 |  | North (Flood) | 0.08 kts |
| 11:55 AM | 38 | 0.6 |  | North (Flood) | 0.01 |
| 1:10 PM | 450 | 7.5 |  | North (Flood) | 0.145 |
| Turbidity Short site tube | 10:45 Am | 40 JTU |  | 40 JTU | Average 40 JTU |
| Core Description |  |  |  |  |  |
| Shipping Traffic | 12:30 PM | Dredge |  | Blue | Southbound |
|  | 12:40 PM | RIverkeeper |  |  | Northbound |
|  | 1:30 PM | Cruise |  | White | Southbound |
| Other Observations | There are not a lot of people. We did find some blue jay feathers on the shore. |  |  |  |  |
| *Currents note – group noted that closer to shore current went north, farther out it went south! (looking at the sample time the tide was just starting to turn which could account for this difference) |  |  |  |  |  |