Snapshot Day 10/14/10 Data

(Salt Front RM 36)

RIVER MILE 35 – Bowline Park, Haverstraw Waterfront

Sarah Sterner, DEC

Jeanne Maguire, North Rockland High School – 12th grade – 14 students

41.203N  73.958W

Naked Goby Caught at Bowline Park

Location: Bowline Park, Haverstraw waterfront, Rockland County NY

Area: Parking lot, recreation, fishing

Surrounding Land Use: 10% beach, 50% urban/residential, 40% other

Sampling Site: shoreline is sandy, muddy, rocky, small beach, bank altered, large rip rap, bulkhead,

Piping coming into river

Plants in area: milkweed 2%, water chestnut seed pods in area 1% (traveled on waterway)

Water depth: 3 ft.

River Bottom –muddy

| ITEM | Time | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- |
| Physical |  |  |  |  |
| Air Temperature | 8:30 am | 41°F 5°C | 9:25 am | 51°F 10.5°C |
| Wind Speed | Beaufort #2 | 4-7 km/hr | 4-6 knots | N |
| Cloud Cover | Partly cloudy |  |  |  |
| Weather today | No rain |  |  |  |
| Weather last 3 days | Nightfall rains on Tuesday | Temp. seasonal Mild wind |  |  |
| Water Temperature | 8:30am | 16.66 C 62 F |  |  |
| Direct sun, barely plants, fairly still water, a lot of bricks and rocks | 10:27 am | 16.66 C 62 F |  |  |
|  | 11:47 AM | 22 C 70 F | 20 C 68 F |  |
| Turbidity |  |  |  | Water was cloudy |
| 9:02 am | 27 cm | 30 cm | 72 cm | Avg 43 cm |
| Chlorophyll | 10:28 am | 0.7 |  |  |
| Chemical |  |  |  |  |
| DO Drop count | 8:30 am | 16.67C | 6 PPM | 60 % sat |
|  | 10:30 am | 16.67C | 6 PPM | 60 % sat |
|  | 11:30 am | 20C | 8 PPM | 90 % sat |
| pH Colormetric kit | 8:35 AM 9:20 AM | 7.5 7.5 |  |  |
|  | 10:10 AM | 8.0 7.5 7.5 |  | Average 7.66 |
|  | 11:33 AM | 7.2 |  |  |
| Salinity Titrator strips – high range |  | Quantab strip .75 | .04 mg/L chloride | *striped might have been too high to read this correctly |
| Fish Catch | Number Caught | Species | Length of largest | Catch per Seine - CPUE |
| Seine 12 ft. X 6ft. Mesh 0.25 inch | 3 | Killifish | 7.5 cm | 8:45 AM - 3 |
|  | 3 | Striped Bass | 8 cm | 9:30 AM - 5 |
|  | 1 | Naked goby | 4 cm | 10:45 AM - 0 |
| Macroinvert. | 1 | Blue crab | 3.4 inch | 11:45 AM - 0 |
| Tides | 8:15 am | 30 cm |  |  |
|  | 8:20 am | 29 cm | falling |  |
|  | 9:17 am | 20 cm | falling |  |
|  | 9:45 am | 16 cm | falling |  |
|  | 11:00 am | 21cm | rising |  |
| NOTE: Low Tide 10:38 AM |  |  |  |  |
| Currents | CM/60 secs | Cm/Sec | North/South Ebb/Flood/Still | Knots |
| 8:47 am |  |  | Ebb | 0.04Knots |
| 9:23 am |  |  | Ebb | 0.12Knots |
| 11:20 am |  |  | Flood | 0.168 knots |
| Core – 4 cores | Length | Rare | Common | Abundant |
| Recorded here is one given to Sarah | 16 cm | Mud Leaves Wood Oyster shell Macro Coal Living veg. | Sand slag | gravel pebbles brick |
| Shipping | 10:15 AM | Large red barge with white tug | Southbound | Full of construction material |
| Observations | Garbage was along the shores and many bricks were along the beach showing how there were old brick factories in the area. Also spotted a falcon in a tree. |  |  |  |