Snapshot Day 10/14/10 Data

(Salt Front RM 36)

RIVER MILE 39.5

George’s Island, Montrose, Westchester County

Susan Grasso, Our Montessori School, Yorktown – 1st-6th grade – 38 students

41.16N  74.03W

Location: Waterfront park in Westchester County Park

Area: Mainly used for boating, fishing and picnics.  It had 2 boat launch ramps and was very rocky.  Lots of oyster shells & vegetation

Surrounding Land Use: 70% urban/residential, 15% forested, 5% Beach, 10% other

Sampling Site: shoreline is rocky, beach covered in vegetation, banks altered,

Plants in area: 1% poison ivy, 1% poison oak, 40% moss, 1% willow trees, 40% weeds, other trees 17%

Water depth:

River Bottom –muddy, rocky

| ITEM | Time | Reading 1 | Reading 2 | Comments |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |
| Air Temperature | 11:00 am | 18°C | 11:20 AM | 20 °C |  |  |  |
|  | 11:40 AM | 24 °C | 12:50 PM | 26°C |  |  |  |
|  | 1:50 PM | 20 °C |  |  |  |  |  |
| Wind Speed | Beaufort fluctuated between #1 and # 3 | 1-5 km/hr & 13-19 km/hr |  | SW until afternoon then South |  |  |  |
| Cloud Cover | Partly cloudy |  |  |  |  |  |  |
| Weather today | Calm in AM | Cloudy and windy in PM | Wind shifted the current & the water was a little choppy | Tide was coming in. |  |  |  |
| Weather last 3 days | Little sunny, little rainy and a little cloudy |  |  |  |  |  |  |
| Water | Calm in AM | Choppy in PM |  |  |  |  |  |
| Water Temperature | 10:55 AM | 15 C 65 F | 6 ft. depth |  |  |  |  |
| AM sunny warm and still | 11:35 am | 20 C 70 F | 6 ft. depth |  |  |  |  |
| PM breezy, cloudy, cooler w/ rippling waves | 12:00 PM | 20 C 70 F | 6 ft. depth |  |  |  |  |
| Turbidity |  |  |  |  |  |  |  |
| secchi | 10:50 AM | 51 cm 56 cm 53.5 | Murky, smelly, pretty turbid | Avg 53.5 cm |  |  |  |
|  | 11:23 AM | 63.5 cm 61 60 | Dark, smelly brown water | Avg. 61.5cm |  |  |  |
|  | 11:51 AM | 66 cm 59 66 | Murky, brown, can’t see bottom | Avg. 63.5cm |  |  |  |
|  | 12:55 PM | 69 cm 68 69 | Water turbid, very brown. Couldn’t see bottom even where shallow | Avg. 68.5cm |  |  |  |
|  | 1:30 PM | 61 cm 59 63 | Bottom is brownish. Can’t see the bottom. Pretty turbid | Avg. 61 cm |  |  |  |
| Long site tube | 10:50 AM | 33 cm |  |  |  |  |  |
|  | 11:23 AM | 45 cm |  |  |  |  |  |
|  | 11:51 AM | 35 cm |  |  |  |  |  |
|  | 12:55 PM | 35 cm |  |  |  |  |  |
|  | 1:30 PM | 30 cm |  |  |  |  |  |
| Chlorophyll | 10:17 am | 0.4 |  |  |  |  |  |
|  | 11:40 Am | 0.1 |  |  |  |  |  |
|  | 12:00 noon | 0.3 |  |  |  |  |  |
|  | 1:17 PM | 0.3 |  |  |  |  |  |
|  | 1:30 PM | 0.2 |  |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |
| DO Drop count | 8:30 am | 16.67C | 6 PPM | 60 % sat |  |  |  |
|  | 10:30 am | 16.67C | 6 PPM | 60 % sat |  |  |  |
|  | 11:30 am | 20C | 8 PPM | 90 % sat |  |  |  |
| pH Colormetric kit | 10:50 AM 11:24 AM | 9* 8.5 |  | * # seems very high |  |  |  |
|  | 12:00 PM | 8.0 |  |  |  |  |  |
|  | 1:00 PM | 8.0 |  |  |  |  |  |
|  | 1:30 PM | 8.0 |  |  |  |  |  |
| Salinity Titrator strips – low range |  | Quantab strip 1.0 | 248 mg/L chloride | 355 ppm total salinity |  |  |  |
|  | Note group also used a hydrometer and got readings of 6.0 ppt but there must have been some error since this reading is not in keeping with the other readings on the river for 10/14. |  |  |  |  |  |  |
| Fish Catch | Number Caught | Species | Length of largest | Comment |  |  |  |
| 11:00 AM | 1 | shrimp |  | Caught no fish which was quite a surprise! |  |  |  |
|  | 1 | crab |  |  |  |  |  |
|  | 3 | Baby crab |  |  |  |  |  |
| 11:30 AM | 2 | shrimp |  |  |  |  |  |
| 12:00 PM | 2 | shrimp |  |  |  |  |  |
|  | 2 | crabs |  |  |  |  |  |
|  | 1 | *Dead blue crab |  | *a possible molt? |  |  |  |
| 1:00 PM | 1 | Baby shrimp |  |  |  |  |  |
| 1:25 PM | 2 | crabs |  |  |  |  |  |
|  | 2 | shrimp |  |  |  |  |  |
| Tides - Measuring down from a start point | 10:45 am | 45 cm |  |  |  |  |  |
|  | 11:27 am | 39 cm | rising |  |  |  |  |
|  | 11:57 am | 33 cm | rising |  |  |  |  |
|  | 1:00 pm | 32 cm | rising |  |  |  |  |
|  | 1:27 pm | 29cm | rising |  |  |  |  |
| Currents | CM/60 secs | Cm/Sec | North/South Ebb/Flood/Still | Knots |  |  |  |
| 10:48 am |  |  | flood |  |  |  |  |
| 11:30 am |  |  | Flood |  |  |  |  |
| 12:00 PM |  |  | Flood |  |  |  |  |
| 12:50 AM |  |  | Flood |  |  |  |  |
| 1:42 AM |  |  | Flood |  |  |  |  |
| Core – | Length | Grain Size | Plant Material | Comments |  |  |  |
| Core #1 – 10:50 AM | 18 inches – no layers | Mix fine and large | Lots of plant material | smells like metal/smells bad |  |  |  |
| Core #2 – 10:50 AM | 6 inches | Fine grain | No plant material | smells like sulfur/metal |  |  |  |
| Core #3 – 11:51 AM | 12 inches – 6 inch top and 6 inch bottom | Mix of fine and large grain | Yes plant material | smells like sulfur. First layer was not compacted (cracked) –(dry). Second layer smooth – (wet) |  |  |  |
| Core #4 12:55 PM | 12 inches – 6 inch top and 6 inch bottom | Fine grain | Yes plant material | Sulfur smell, greenish color. You can see shells, different colors make up the sediment |  |  |  |
| Core #5 1:30 PM | 18 inch – 6 inch top and 12 inch bottom | Fine grain | Plant material | smells like sulfur it is brown and black, has plants, smells bad |  |  |  |
| Shipping | 11:25 AM | Hughes – grey and red | Southbound | Full |  |  |  |
|  | 1:30 PM | Ferryboat | Northbound | empty |  |  |  |
| Observations | Footprints and LOTS of trash which most of the student sheets commented on. Saw chipmunks, birds. No other people there. Boats on the river. Arrived at the site at low tide and the tide started to come in shortly after arrival. |  |  |  |  |  |  |