Snapshot Day 10/18/11 Data

(Salt Front RM 35.7)

RIVER MILE 108

Lasher Memorial Park, Germantown, Columbia County

Jenny Brinker, Columbia Land Conservancy

Jim McDarby, Hudson High School, 11th and 12th graders – 19 students

GPS Latitude 4209.50’ N Longitude 7353.21’W

Location: Lasher Memorial Park.

Area: Grassy area, used for picnics and boating

Surrounding Land Use: Beach – 25%; Forested  25%; Urban/residential 50%

Sampling Site: beach covered in vegetation

Shoreline: sandy, rocky

Plants in area:   No plants in water (water chestnuts gone from last year)

| Tidal -shallows | Phragmites 70% | Shrubs & Brush 20% | Willows 10% |
| --- | --- | --- | --- |

Water depth: 3 ft.

River Bottom : sandy, rocky

Notes: Lots of debris from storms in late August.  Water chestnut mats are gone.  Beach areas eroded.  Many of the willow and cottonwood trees uprooted and had to be removed from the site because of damage.

| ITEM | Time |  | Reading 1 | Reading 2 |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |
| Air Temperature | 10:01 AM |  | 55F 12C |  |  |  |
|  | 1:11 PM |  | 69F 20C |  |  |  |
| Wind Speed | Beaufort Calm |  | Wind Meter 0 knots |  |  |  |
| Cloud Cover | Partly cloudy |  |  |  |  |  |
| Weather today | Cloudy in the AM, sunshine in the PM |  |  |  |  |  |
| Weather recently | Highs in the 60’s, lows in the 40’s |  |  |  |  |  |
| Water Temperature | 10:15 AM | 14C |  |  | 3 repetitions the same | 2 ft. depth water |
|  | 11:30 AM | 14C |  |  | 3 repetitions the same | 2 ft. depth water |
|  | 1:00 PM | 15C |  |  | 3 repetitions the same | 2 ft. depth water |
| Water calm | yes |  |  |  |  |  |
| Turbidity – sight tube | 10:06 AM 11:12 AM 12:47 AM | 21 cm 10 cm 7 cm |  |  |  |  |
| Chlorophyll | 10:08 AM 11:16 AM | 1.0 2.0 |  |  |  |  |
| Chemical |  |  |  |  |  |  |
| DO (drop count kit) | 10:15 AM 11:30 AM 1:00 PM | 8 mg/L 10 mg/L 7 mg/L |  |  | 14C 14C 15C | 77% 96% 70% |
| pH –Test strips | 10:10 AM 11:30 AM 1:00 PM | 6.5 6.5 6 |  |  |  |  |
| Salinity – Quantabs | 10:15 AM 11:30 AM 1:00 PM | 22 ppm Cl 22 ppm Cl 20 ppm Cl |  |  | Total Salinity | 39.7 ppm salinity 39.7 ppm salinity 36 ppm Salinity |
| Fish Catch 40 ft. seine net X 3/8 inch mesh | Number Caught | Species |  |  | Length of largest | Macro Species |
| 10:30 AM | 15 | Spottail Shiner |  |  | 10 cm | Scud |
|  | 8 | Striped Bass |  |  | 15 cm |  |
|  | 20 | Herring – yoy |  |  | 6.4 cm |  |
| 11:00 AM | 22 | Spottail Shiner |  |  | 10 cm |  |
|  | 8 | Striped Bass |  |  | 7 cm |  |
|  | 2 2 3 | Tessellated Darter Pumpkinseed Herring - yoy |  |  | 6.5 cm 8 cm 6 cm |  |
| 1:00 PM | >1 | Spottail Shiner |  |  | 7 cm | Blue crab (2) |
|  | >1 | Pumpkinseed |  |  | 8.5 cm |  |
|  | >1 | Tessellated Darter |  |  | 6.3 cm |  |
|  | >1 | White Perch |  |  | 7 cm |  |
| Tides | 10:00 AM | 45 cm |  |  |  |  |
|  | 10:40 AM | 30 cm |  |  | Falling |  |
|  | 11:00 AM | 25 cm |  |  | Falling |  |
|  | 11:35 AM | 15 cm |  |  | Falling |  |
|  | 12:32 PM | 5 cm |  |  | Falling |  |
| Currents | 10:14 AM | ebb |  |  |  |  |
|  | 11:01 AM | ebb |  |  |  |  |
|  | 12:36 PM | ebb |  |  |  |  |
| Traffic |  |  |  |  |  |  |
| Other Items | Gulls | Monarch Butterfly |  |  |  |  |
| Core | Yes | Sent with runner |  |  |  |  |