Snapshot Day 10/18/11

(Salt Front RM 35.7)

RIVER MILE 17

Mt. St. Vincent College Campus, NYC

Pat Grove – Mt. St. Vincent College – ~ 5 undergraduate students

Jeannine Brown, Frederick Douglass Academy, 20 students 10th-12th graders

Latitude 4091282 N Longitude 7391150 W

Location: Waterfront of Mt. St. Vincent’s College

Area: Forested

Surrounding Land Use: Urban/residential 40%; Forested 50%, Beach 5%; industrial/commercial (RR tracks) 5%

Sampling Site: large riprap on shoreline, drainage pipes in area, beach area, covered in vegetation

Plants in water of sampling area: no plants in the water

Water depth: 38-52 cm

River Bottom – sandy, muddy, rocky

| ITEM | Time |  | Reading 1 |  | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |
| Air Temperature | 10:15 AM |  | 61F |  |  |  |
|  | 11:30 AM |  | 64F |  |  |  |
| Wind Speed | Beaufort #1 |  | *230 degrees SW |  |  | 1.4 kts |
| Cloud Cover | Partly cloudy |  |  |  |  |  |
| Weather today | Cool and clear |  |  |  |  |  |
| Weather recently | Colder than normal overnight |  |  |  |  |  |
| Water Temperature | 10:30 AM | 17C 62.6F |  | 18 C 64.4F | 18 C 64.4F | 17.6 C AVG 63.8F |
|  | 11:30 AM | 18 C 64.4F |  | 18 C 64.4F | 19 C 66.2C | 18.3 C 65F |
|  | 12:30 PM | 20 C 68F |  | 20 C 68F | 20 C 68F | 20 C 68F |
| Water surface: | calm |  |  |  |  |  |
| Turbidity (Secchi) | 10:30 AM | 20 cm 20 cm 20 cm |  |  |  | 20 cm |
| Long site tube | 10:30 AM | 27.6 cm |  |  | 276 NTU |  |
| Chlorophyll | 12:15PM | 2.0 |  |  |  |  |
| Chemical |  |  |  |  |  |  |
| DO LaMotte kit 7414 | 10:00 AM | 7.2 mg/L |  |  | Temp.17 C | 70% saturated |
| pH – LaMotte kit 5858 | 11:45 AM | 7.5 7.3 7.4 |  |  | 7.4 avg. |  |
| Phosphate | 1:07 PM | 1.0 ppm |  |  |  |  |
| Nitrate | 12:55 PM | 1.0 ppm |  |  |  |  |
| Alkalinity | 1:00 PM | 68 ppm |  |  | 68 ppm |  |
| Salinity - refractometer | 12:00 PM | 2 ppt 2 ppt 2 ppt |  |  | 2 ppt |  |
| Fish Catch | Number Caught | Species |  |  | CPUE | Minnow Traps |
| seine – 30 ft. net | 1 | Striped Bass |  |  |  |  |
| Macros |  | Mud crab |  |  |  |  |
|  |  | Shore shrimp |  |  |  |  |
| Tides | 10:42 AM | 33 cm |  |  |  |  |
| (high tide 1:35 PM per the charts) The recordings are confusing – some with same time and different measures and seem to go up and down? | 10:46 AM | 40 cm |  |  | Should be rising all sampling time |  |
|  | 11:00 AM | 38 cm |  |  |  |  |
|  | 11:00 AM | 45 cm |  |  |  |  |
|  | 11:15 AM | 43 cm |  |  |  |  |
|  | 11:30 AM | 35 cm |  |  |  |  |
|  | 11:30 AM | 72 cm |  |  |  |  |
|  | 11:45 AM | 55 cm |  |  |  |  |
|  | 11:45 AM | 51 cm |  |  |  |  |
| Currents |  |  |  |  |  |  |
| Core |  |  |  |  |  |  |
| Traffic | 11:15 AM | Cargo |  |  | Northbound | Light |
|  | 11:30 AM | Cargo |  |  | Anchored | Loaded/ Containers |
|  | 11:24 AM | Garbage Ship 4002 |  |  | Northbound | Loaded Garbage |
|  | 12:45 PM | Speedboat |  |  | Southbound |  |
| Other observations |  |  |  |  |  |  |

* To learn more about how oceanographers code wind direction go to the c.o.o.l. project  http://www.coolclassroom.org/pdfs/understandingwinddirection.pdf