Snapshot Day 10/18/11

(Salt Front RM 35.7)

RIVER MILE 2

Chris Anderson - The River Project

Bob Wallace - NYU

Karen Johnson, Dora Kong, Kim Esther, Jane O’Donnell

3rd grade Trevor Day School – 56 students + 12 parents + 4 teachers + RP staff

Latitude N 4043’71 N Longitude 7400’89 W

Location: Pier 40 Hudson River  (also called North River)

Area: Large pier with many facilities: athletic fields, kayaking and boathouse, seasonal moorings available, parking, fishing on the pier, benches & picnic tables for the public Surrounding Land Use: parking lot – area used for picnics, boating, fishing

Sampling Site: Banks altered; concrete & wood pilings, pier extends into river/cement bulkhead

Plants in area: None visible

Water depth:

River Bottom –

NOTE:

This highly urban site is just west of the busy Westside Highway – it is the largest pier in Hudson River Park (with a 14 acre footprint); Pier 40 is used for recreation; school groups use the athletic fields; there are 3 public boathouses; there are many people by on the river’s edge: jogging, bike riding, walking; River Project’s temporary home is on Pier 40.

(Measurements were taken from 10:45 AM to 11:45 AM, average given)

| ITEM | Time |  | Reading F | Reading C | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 10:45-11:45 AM |  | 63.3 F | 17.4 °C | Average – |
| Wind Speed | Time |  | Beaufort Chart | Kestral Meter |  |
|  | 10:45-11:45 |  | #2 | 1.03 kts | South |
| Cloud Cover | 10:30-noon - Partly cloudy to rapidly increasing clouds |  |  |  |  |
| Weather today | partly cloudy, beginning to clear, light rain for about 15 mins |  |  |  |  |
| Weather recently | Cool (high 50s - low 60s)/ breezy, windy recently/ no rain |  |  |  |  |
| Water | Fairly calm, minimally choppy |  |  |  |  |
| Water Temperature - | 10:56 AM 11:02 11:15 11:26 11:37 11:50 | 15C 15 18 17 17 20 |  | 60 F 60 F 65 F 63 F 63 F 68 F | Collected in the sun, off the pier ~ 500 feet into the water in ~ 6-10 feet of water |
| Turbidity (long site tube | 10:50-11:13 11:25 11:40 11:50 AM | 77.8 cm 71.25 79.8 69.8 81 |  |  | Water looked brownish/greenish and rather murky |
| Chlorophyll |  |  |  |  |  |
| Chemical | TIME | TEMP. |  | PPM | % SAT. |
| DO (drop count kit) | 10:56 AM 11:15 11:26 11:37 11:50 | 15C 18 17 17 20 |  | 5 PPM 6 PPM 4PPM 5 PPM 6 PPM | 49 62 40 50 65 |
| pH – color match LaMotte kit | 10:45 AM 11:58 11:10 11:22 11:33 11:45 | 7.5 8.5 7.5 8.5 7.5 8 |  |  | Average 7.35 |
| Salinity – hydrometer - | 10:45 AM 11:58 11:10 11:22 11:33 11:45 | 6.0 ppt 6.0 ppt 6.0 ppt 6.0 ppt 5.5 ppt 5 ppt |  |  |  |
| Vertebrate Catch | Number Caught | Species |  | Size | Minnow traps |
| Pots on the Lilac Ship – the pots were checked without the students since they are at a different location. | 1 | American Eel |  | 43 cm |  |
| Invertebrate Catch | 26 | Grass Shrimp |  |  |  |
|  | 1 | European green crab |  |  |  |
|  | 10-20 | Isopods & mud snails |  |  |  |
| Tides |  |  |  |  |  |
| Currents |  |  |  |  |  |
| Boat Traffic | 11:09 AM | Barge & Tugboat |  | The Evaleigh | Northbound |
|  | 11:15 AM | NY Waterway Ferry |  | The Henry Hudson | Northbound |
|  | 11:29 AM | Tug Boat |  | Harry Muncil | Northbound |
|  | 11:36 AM | Large Powerboat |  | Amusant | Southbound |
| This is a largely commercial area but there is also a paved walkway along the river used by pedestrians, joggers & bicyclists |  |  |  |  |  |