Snapshot Day 10/18/11

(Salt Front RM 35.7)

RIVER MILE 13

Bloomers Beach @ Engelwood Boat Basin

Kirk Weber – Meadowlands Environmental Center

Shabeena Sarker – West New York Middle School – 25 students, 7th & 8th graders

N40.881297	 -W73.943948

|  | Location: Bloomers Beach – north end Area: sandy beach, banks altered, pier with rip rap sections, wood bulkhead, collected wood/debris in area, piping approx. 30” located at sampling site, 2 stone breakwaters as well as on shoreline, + cement storm wall Surrounding Land Use: 20% urban/residential; 40% forested; 20% beach; 20% Industrial/commercial Bottom: sandy & muddy Plants in area: none in the water Water Depth:115 cm one hour before high tide, |
| --- | --- |

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 10:15 AM |  | 65F 18.3C |  |  |
| Wind Speed | Beaufort #2 |  |  |  | South |
| Cloud Cover | Cloud coverage decreased throughout the observation period |  |  |  |  |
| Weather today | No precipitation |  |  |  |  |
| Weather recently | 10/15 normal (.09 inches rain); 10/16 normal (no rain); 10/17 normal (.01 inches rain) |  |  |  |  |
| Water | Choppy |  |  |  |  |
| Water Temperature | 10:10 AM | 17.3C 16.6C 17.3C |  | 63.2 F 62.0F 63.1F | 17.07C 62.7F |
|  | 11:00 AM | 17.5C 17.9C 18C |  | 63.6F 64.3F 64.5F | 17.8C 64.2F |
|  | 11:40 AM | 17.8C 18.1C 18.4C |  | 64.1F 64.6F 65.1F | 18.1C 64.6F |
| Turbidity – long site tube | 12.30 AM | 22 cm 25 cm 29 cm |  |  | 25.3 cm |
| Chemical |  |  |  |  |  |
| Chlorophyll | 11:30 AM | 0.7 |  |  |  |
| DO – ampules | 11:45AM | 6.3 mg/L |  | 17.7 C | 65% saturated |
| pH – indicator sol. | 12:25 PM | 8.0 7.5 8.0 |  |  | 7.8 avg. |
| Salinity (hydrometer) | 11:30 PM | 6 ppt 6 ppt 6 ppt |  |  | 6ppt average |
| Fish Catch | Count | Fish |  | Size of largest | Macros |
| 14 ft. X 3 ft seine – pulled 20 ft. | 50 plus | Atlantic Silverside |  | 9 cm |  |
| 1/8 inch mesh | 9 | Striped Bass |  | 10 cm |  |
| 9 seines | 10 | Herring |  | 6 cm |  |
| Seine #1 | 10:15 AM | 15 fish |  | 20 ft pull | .18 |
| Seine #2 | 11:15 AM | 18 fish |  | 20 ft pull | .13 |
| Seine #3 | 12:15 PM | 25 fish |  | 20 ft pull | .29 |
| Tides | 8:58 | rising |  | 50 cm |  |
|  | 10:50 | rising |  | 95 cm | .40 cm/min |
|  | 11:35 | rising |  | 105 cm | .22 cm/min |
|  | 12:15 | rising |  | 115 cm | .25 cm/min |
| Currents | 10:58 | 7.01 cm/sec |  | North | Flood |
| Core – | 22 cm total – overall black in color | Top layer 4 cm –pudding like |  | Second layer 18 cm –clayey |  |
| Boats |  |  |  |  |  |