Snapshot Day 10/18/11 Data

(Salt Front RM 35.7)

RM -2 , Valentino Pier

Katie Mosher-Smith & Kerstin Kalchmayr, NY/NJ Baykeeper

Karine Kelley, PS 230 (NYC Dept. of Ed)    5th graders – 23 students

GPS Latitude N :  40°40’42. 10” - W-74°01’06.07

Location: Valentino Pier, Redhook, Brooklyn, NY

Area: beach area along the bay

Surrounding Land Use: Beach – 3%; Urban/residential 5%; Industrial/Commercial 60%; park 32%

Sampling Site: beach

Shoreline: Pier, riprap

Plants in area:  No plants in water

Water depth: 10 feet

River Bottom : rocky near the little beach area

| ITEM | Time |  | Reading 1 | Reading 2 |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |
| Air Temperature |  |  |  |  |  |  |
| Wind Speed | Beaufort Force |  | 1 |  |  |  |
| Cloud Cover | clear |  |  |  |  |  |
| Weather today | Some precipitation earlier that morning but the morning cleared up to bright sunny skies by the time we got out to the pier |  |  |  |  |  |
| Weather recently | Relatively warm weather the previous days, sunny skies. Overcast and some precipitation earlier in the morning on Oct 18 |  |  |  |  |  |
| Water Temperature | 11:27 AM | 15C 17 16 |  |  |  | Average 16C 60.8F |
| Water calm | yes |  |  |  |  |  |
| Turbidity – long sight tube | 11:08 AM | 100 cm 100 100 |  |  |  | 100 cm |
| Chlorophyll | 11:15 AM | 0.3 |  |  |  |  |
| Chemical |  |  |  |  |  |  |
| DO (YSI handmeter) | 11:49AM | 6.9mg/L |  |  | 16C | 70% |
| pH –pen | 11:26 AM | 8.02 8.04 8.7 |  |  |  | 8.25 |
| Salinity – hydrometer | 11:30 AM | 15 ppt 18 ppt 16 ppt |  |  |  | 16.3 ppt |
| Fish Catch 93 inch. seine net | Number Caught | Species |  |  | Length of largest | Macro Species |
| 12:30 AM | 7 | Atlantic Silversides |  |  |  |  |
|  | 1 |  |  |  |  | Comb jelly |
| Tides |  |  |  |  |  |  |
| Currents | 12:36 PM | ebb |  |  |  |  |
|  | 12:50 PM | ebb |  |  |  |  |
| Traffic |  |  |  |  |  |  |
| Other Items |  |  |  |  |  |  |
| Core |  |  |  |  |  |  |