Day in the Life of the Hudson River 10/4/12

(Salt Front RM 35.7)

RIVER MILE 17

Mt. St. Vincent College Campus, NYC

Pat Grove – Mt. St. Vincent College – ~ 20 undergraduate students

Alexandra Romero, Palisades Prep in Yonkers, 10 students 11th graders

Latitude 4091349 N Longitude 7391154 W

Location: Waterfront of Mt. St. Vincent’s College

Area: Forested, grassy

Surrounding Land Use: Urban/residential 30%; Forested 10%, Industrial/Commerial 10%, Ecology Lab 50%

Sampling Site: large riprap on shoreline, bulkhead, drainage pipes in area, beach area, covered in vegetation

Plants in water of sampling area: no plants in the water

Plants in the area:Moss 30% of rock, trees 75% of shore, grass 25% of shore, sea weed 10% drifting in water, 90% leaves in end off water, bushes in area, algea 10% of area

Water depth:

River Bottom – sandy, muddy, rocky

| ITEM | Time |  | Reading 1 |  | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |
| Air Temperature |  |  |  |  |  |  |
| Wind Speed |  |  | 2.63 km/hr |  | South |  |
| Cloud Cover | Mostly cloudy |  |  |  |  |  |
| Weather today | rainy |  |  |  |  |  |
| Weather recently | Some rain, |  |  |  |  |  |
| Water Temperature – 35 cm | 11:54 AM | 22C |  | 22 C | 22 C | 22 C AVG |
| Water surface: | calm |  |  |  |  |  |
| Turbidity (Long Site Tube) | 11:30 AM | 36 cm 35 cm 22 cm |  |  |  | 31 cm |
| Chlorophyll | 12:30 PM | 0.5 |  |  |  |  |
| Chemical |  |  |  |  |  |  |
| DO LaMotte kit 7414 | 11:54 AM | 8 mg/L 11 mg/L 6 mg/L |  |  | Temp.22 C | 95% 125% 70%saturated |
| pH – LaMotte kit 5858 | 11:52 AM | 7.5 7.5 |  |  | 7.5 avg. |  |
| Salinity - refractometer | 11:30 AM 11:35 AM | 10 ppt 10 ppt |  |  |  |  |
| Fish Catch | Number Caught | Species |  |  | CPUE | Minnow Traps |
| seine – 30 ft. net | 1 | eel |  |  |  |  |
|  | 1 | Oyster toadfish |  |  |  |  |
|  |  | Glass shrimp |  |  |  |  |
|  |  | Baby crabs |  |  |  |  |
| Tides |  |  |  |  |  |  |
| Currents | 11:43 AM | Flood - North |  |  | 0.164 meters per sec |  |
| Core |  |  |  |  |  |  |
| Traffic | 10:14 AM | No name – white |  |  | Southbound | Light |
|  | 10:07 AM | Caledonian Sky white |  |  | Southbound | Loaded/ |
|  | 10:07 AM | Brown |  |  | Southbound | Light |
|  | 10:40 AM | White |  |  | Northbound | Light |
|  | 10:30 AM | Black |  |  | Southbound | Light |
|  | 11:55 AM | Grey?White Tug |  |  | Southbound | Loaded |
| Other observations | White Birds – probably seagulls flying over the river | Dead fish floating |  |  | Geese |  |