Day in the Life of the Hudson River 10/04/12 Data

RIVER MILE 25E

Matthiessen Park, Irvington

Philip Levine and Joanna Morabito- Irvington Middle School Sixth Grade (144) and Irvington High School Marine Science Class 23 students, 11/12th graders

Latitude 41 ͦ 2’ 30” N Longitude 73 ͦ 52’ 32” W

Area: Matthiessen Park, Village of Irvington, Westchester County

Surrounding Land Use: 40% grass, 10% beach, 5 % forested, 45% river

Sampling Site: Matthiessen Park

Plants:

Water Depth: 4 feet

River Bottom: Mud, Sand, Rocks. The further you went out the muddier it got.

Weather Conditions: Humid, Warm, Foggy.

| ITEM | Time |  | Reading 1 | Reading 2 |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |
| Air Temperature | 9:15 am 10:23 am |  | 22C | 21C |  |  |
| Wind Speed | 9:30 am |  | No wind | No wind |  |  |
| Cloud Cover | 9:30 am 11:30 am |  | 100% | 100% |  |  |
| Weather today | Overcast, humid, foggy, muggy, warm. |  |  |  |  |  |
| Weather recently | Rainy, humid. |  |  |  |  |  |
| Water Surface |  |  |  |  |  |  |
| Water Temperature – | 10:30 am 8:30 am | 20CC |  |  | 18C |  |
| Turbidity –water chemistry test kit | 9:55 am 11:35 am | 20JTU |  |  | 40JTU |  |
| Chlorophyll | 11:30 AM | 0.5 |  |  |  |  |
| Chemical |  |  |  |  |  |  |
| DO (drop kits) | 8:30 am 8:51 am | 4 ppm 42% saturation |  |  | 4 ppm 42% saturation |  |
| pH | 8:30 am 9:35 am | 8 |  |  | 7 |  |
| Salinity - meter | 8:48 am 8:30 am | Readings ranged from 6 to 10 ppt collected from 9:30 to 11 a.m. |  |  | 7 |  |
| Fish Catch- 12:00 PM to 1:25 PM | Number Caught 30 2 4 1 1 | Species Atlantic Silverside Striped Bass Blue Crabs White Perch Oyster |  |  | Average Length 3 inches 3 inches 2 inches 2 inches 4 inches | Seine |
| *Lots of water celery in nets |  |  |  |  |  |  |
| Tides R= right tide stick L = Left tide stick | TIME (am) 8:29 R 9:25 R 9:33 L 9:33 R 9:51 L 9:51 R 10:04 R 10:04 L 10:15 L 10:15 R | Water height (cm) 12.5 30.5 (boat wave) 16 24 24 33 39 31 34.5 44 |  |  | Rise/Fall RISE |  |
| Currents | Time 10:30 am 11:45 am 9:08 am | Cm/60 sec 11.6 22.5 |  |  | Cm/sec 1.9 .375 | Direction First flowing north Second flowing south |
| Core – 8 cm long | TIME 9:45 am | Rare Coal |  |  | Common Pebbles and mud | Abundant Sand and gravel |
| Traffic | 9:30 am | Boat passed, Columbian Sky |  |  |  |  |
| Other Items | Students’ Hypotheses: For water chemistry majority of the students found results to be close to their predictions. |  |  |  |  |  |