Day in the Life of the Hudson River 10/4/12 Data

RIVER MILE 25W

Piermont Pier

Margie Turrin & 8 Lamonters, Lamont-Doherty Earth Observatory

Kathy Galione, Rockland County Americorps

Latitude N 41 degrees 02’592    Longitude W 73 degrees 53’789

| Tom Mullane, Eileen McCaffrey, and Catherine Bonanno– Pearl River HS APES – 41 students | Pat Kilkelly and Peter Dene Tappan Zee HS – AP Bio – 19 students |
| --- | --- |

Jen Mazza,– Clarkstown South High School –

9 IB Env. Sci.

Location: Piermont Pier, Piermont NY

Area: Man made fishing/shipping pier at the east end, long pier with paved roadway and vegetation along both sides

Surrounding Land Use:  Urban/residential 80% , Forested 20%

Sampling Site:  Pier, roadway in center of whole pier, parking area at end, Plants and trees along side.  Shoreline Rip Rap, collected wood in area and piping entering river on the south of sampling site

Plants in area: no submerged aquatic plants in the water but lots of plants along pier and in intertidal zone & lots of water celery washed up on shore and coming up in nets

Water depth: Varied by location on the pier and by tide

River Bottom – sandy/rocky bottom with marsh grass growing in or near the water

Plants in area: sumac 30%; marsh plants 70%; reeds & cattails

| ITEM | Time |  | Temperature |  | Observations |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature | 9:20AM |  | 23C 66F |  | Monarch butterfly, drifting boat dock (north) |  |  |  |  |
|  | 9:40 AM |  | 26C |  | none |  |  |  |  |
|  | 10:00 AM |  | 27C |  | none |  |  |  |  |
|  | 10:30 AM |  | 23 C |  | Monarch Butterfly |  |  |  |  |
|  | 10:45 AM |  | 25 C |  | Monarch Butterfly |  |  |  |  |
|  | 11:00 AM |  | 27 C |  | Monarch Butterfly, Schooling Fish - Alongside the pier a group of what we felt were menhaden were circling energetically up close to the surface – flashing wide silver from their sides as they seemed to tip towards the surface (lasted ~30 minutes) |  |  |  |  |
|  | 12:15 AM |  | 25 C |  | 3 Monarch Butterfly |  |  |  |  |
|  | 12:30 AM |  | 23 C |  | 3 Monarch Butterfly |  |  |  |  |
|  | 12:55 AM |  | 24 C |  | 3 Monarch Butterfly |  |  |  |  |
|  | 1:20 PM |  | 23.5 C |  | Monarch Butterfly |  |  |  |  |
| Wind Speed | Time |  | Beaufort # & Direction |  | Anenometer |  |  | Description |  |
|  | 9:20AM |  | Calm |  |  |  |  | Mostly Cloudy, |  |
|  | 9:40 AM |  | Calm |  |  |  |  | Very Cloudy |  |
|  | 10:00 AM |  | Calm |  |  |  |  | cloud cover |  |
|  | 10:30 AM |  | Feel wind but too light to measure |  |  |  |  | Fog, cloudy |  |
|  | 10:45 AM |  | Calm |  |  |  |  | Light rain |  |
|  | 11:00 AM |  | Calm |  |  |  |  | Light rain |  |
|  | 12:15 AM |  | Beaufort 2 – South |  | 6.0 gusts |  |  | Overcast, drizzle, misty |  |
|  | 12:30 AM |  | South |  | 4.9 gusts |  |  | Little rain |  |
|  | 12:55 AM |  | South |  | 4.1 gusts |  |  | Little rain |  |
|  | 1:20 PM |  | South |  | 2.2 gusts |  |  | Light rain, cloudy |  |
| Weather recently | Some rain last few days, some wind |  |  |  |  |  |  |  |  |
| Water Temperature | Time | Surface Sample |  |  |  | Depth Sample |  | Average |  |
|  | 9:30 AM | 21C |  |  |  | 19C |  | 20C |  |
|  | 10:10 AM | 20.5C |  |  |  | 22C |  | 21.25C |  |
|  | 10:40 AM | 20.5C |  |  |  | 20C |  | 20.25C |  |
|  | 12:20 PM | 21C |  |  |  | 21C |  | 21C |  |
|  | 1:10 PM | 22C |  |  |  | 21C |  | 21.5C |  |
| Turbidity – | Time | 3 readings |  |  |  | Average |  | Pier Northside |  |
| Long Site Tube | 9:05 AM | 37,37,33 |  |  |  | 35.5 cm |  | Long site tube |  |
|  | 10:15 AM | 26.5, 26, 24 |  |  |  | 25.5 cm |  |  |  |
|  | 10:40 AM | 22, 23, 23.4 |  |  |  | 22.8 cm |  |  |  |
|  | 10:58 AM | 26, 27, 24 |  |  |  | 25.6 cm |  |  |  |
|  | 11:19 AM | 27, 26 |  |  |  | 26.5 cm |  |  |  |
|  | 12:17 PM | 39, 38.2, 37 |  |  |  | 38 cm |  |  |  |
|  | 12:37 PM | 30, 31, 37.9 |  |  |  | 33 cm |  |  |  |
|  | 1:03 PM | 34, 39, 34 |  |  |  | 35.7 cm |  |  |  |
| Secchi | 9:10 AM | 53, 46.5, 43 |  |  |  | 47.5 cm |  | Secchi |  |
|  | 10:01 AM | 46, 3, 48 |  |  |  | 43.3 cm |  |  |  |
|  | 10:25 AM | 41, 42, 48 |  |  |  | 43.7 cm |  |  |  |
|  | 10:58 AM | 41, 41, 41 |  |  |  | 41 cm |  |  |  |
|  | 11:16 AM | 40 |  |  |  | 40 cm |  |  |  |
|  | 12:15 PM | 61, 61, 61 |  |  |  | 61 cm |  |  |  |
|  | 12:32 PM | 52, 52, 52 |  |  |  | 52 cm |  |  |  |
|  | 12:55 PM | 50, 47.5, 50 |  |  |  | 49.2 cm |  |  |  |
| Chlorophyll | 9:37AM | 2.0 |  |  |  |  |  | Pier Northside |  |
|  | 10:00 AM | 2.0 |  |  |  |  |  |  |  |
|  | 11:05 AM | 2.0 |  |  |  |  |  |  |  |
|  | 12:00 PM | 2.0 |  |  |  |  |  |  |  |
|  | 12:10 PM | 0.8 |  |  |  |  |  |  |  |
|  | 12:35 PM | 2.0 |  |  |  |  |  |  |  |
|  | 1:00 PM | 1.5 |  |  |  |  |  |  |  |
|  | 1:15 PM | 1.5 |  |  |  |  |  |  |  |
| Chemical | *Water sampled at surface and at depth with niskin bottle reported in two separate sets of readings below. Depth for bottom samples ranged from 3.4 meters to 4.3 meters |  |  |  |  |  |  |  |  |
|  |  | SURFACE SAMPLES |  |  |  |  |  |  |  |
| DO | TIME | DO Reading |  |  |  | Water Temp |  | % Saturation |  |
| Drop count | 9:30 AM | Not read |  |  |  | 21C |  |  |  |
|  | 10:10 AM | 7 mg/L |  |  |  | 20.5C |  | 68% |  |
|  | 10:40 AM | 5 mg/L |  |  |  | 20.5C |  | 55% |  |
|  | 12:20 PM | 11 mg/L |  |  |  | 21C |  | 125% |  |
|  | 1:10 PM | 7 mg/L |  |  |  | 22C |  | 80% |  |
|  |  | BOTTOM SAMPLES |  |  |  |  |  |  |  |
| DO** | TIME | DO Reading |  |  |  | Water Temp |  | % Saturation |  |
| **see HRECOS graph @ bottom | 9:30 AM | 7.0 mg/L |  |  |  | 19C |  | 75% |  |
| *Expect error | 10:10 AM | *13.0 mg/L |  |  |  | 22C |  | 130% |  |
|  | 10:40 AM | 7 mg/L |  |  |  | 20C |  | 76% |  |
|  | 12:20 PM | *10 mg/L |  |  |  | 21 C |  | 112% |  |
|  | 1:10 PM | 8 mg/L |  |  |  | 21C |  | 90% |  |
| pH | TIME | SURFACE SAMPLES |  |  |  | DEPTH SAMPLES |  |  |  |
| colorimeter | 9:30 AM | 7.0 |  |  |  | 7.0 |  |  |  |
|  | 10:10 AM | 7.3 |  |  |  | 7.3 |  |  |  |
|  | 10:40 AM | 7.0 |  |  |  | 7.3 |  |  |  |
|  | 12:20 PM | 6.8 |  |  |  | 6.8 |  |  |  |
|  | 1:10 PM | 7.0 |  |  |  | 6.5 |  |  |  |
| Phosphates | TIME | SURFACE SAMPLES |  |  |  | DEPTH SAMPLES |  |  |  |
| colorimeter | 9:30 AM | 0.58 mg/L |  |  |  | 0.52 mg/L |  |  |  |
|  | 10:10 AM | 0.62 mg/L |  |  |  | 0.54 mg/L |  |  |  |
|  | 10:40 AM | 0.64 mg/L |  |  |  | 0.56 mg/L |  |  |  |
|  | 12:20 PM | 0.60 mg/L |  |  |  | 0.4 mg/L |  |  |  |
|  | 1:10 PM | 0.38 mg/L |  |  |  | 0.56 mg/L |  |  |  |
| Nitrates | TIME | SURFACE SAMPLES |  |  |  | DEPTH SAMPLES |  |  |  |
| Drop count | 9:30 AM | 0.45 mg/L |  |  |  | 0.3 mg/L |  |  |  |
|  | 10:10 AM | 0.1 mg/L |  |  |  | 0.1 mg/L |  |  |  |
|  | 10:40 AM | 0.1 mg/L |  |  |  | 0.1 mg/L |  |  |  |
|  | 12:20 PM | 0.2 mg/L |  |  |  | 0.1 mg/L |  |  |  |
|  | 1:10 PM | 0.1 mg.L |  |  |  | 0.2 mg/L |  |  |  |
| Alkalinity | TIME | SURFACE SAMPLE |  |  |  | DEPTH SAMPLES |  |  |  |
| Drop count | 9:30 AM | Not read |  |  |  | 80 ppm |  |  |  |
|  | 10:10 AM | 113 ppm |  |  |  | 68 ppm |  |  |  |
|  | 10:40 AM | 73 ppm |  |  |  | 60 ppm |  |  |  |
|  | 12:20 PM | 88 ppm |  |  |  | 120 ppm |  |  |  |
|  | 1:10 PM | 62 ppm |  |  |  | 110 ppm |  |  |  |
| Salinity - | TIME | SURFACE SAMPLE |  |  |  | DEPTH SAMPLES |  | Refract. (R) GlassHydro(H) |  |
|  | 9:30 AM | 7 ppt –R 7.6 ppt - H |  |  |  | 10 ppt –R 7.1 ppt - H |  |  |  |
|  | 10:10 AM | 8 ppt –R 7.9 ppt - H |  |  |  | 10 ppt –R 7.6 ppt - H |  |  |  |
|  | 10:40 AM | 8 ppt –R 7.5 ppt - H |  |  |  | 9.5 ppt –R 8.8 ppt - H |  |  |  |
|  | *12:20 PM | 8.1 ppt –R 9.1 ppt - H |  |  |  | 8.9 ppt –R 9.5 ppt - H |  | *Recalibrated refractometer |  |
|  | 1:10 PM | 6.5 ppt –R 7.9 ppt - H |  |  |  | 7 ppt –R 7.6 ppt - H |  |  |  |
| Tides- Time | Total Depth | To Water Surface |  |  |  | Tide Depth |  | Ebb/Flood |  |
| 9:10 AM | 583 cm | 190 cm |  |  |  | 393 cm |  |  |  |
| 9:40 AM |  |  |  |  |  | 392 cm |  | Still |  |
| 10:00AM | 579 | 174 |  |  |  | 405 cm |  | Flood |  |
| 10:30 AM | 588 | 160 |  |  |  | 428 cm |  | Flood |  |
| 10:45 AM | 593 | 157 |  |  |  | 436 cm |  | Flood |  |
| 11:00AM | 586 | 147 |  |  |  | 439 cm |  | Flood |  |
| 12:11 PM |  |  |  |  |  | 453 cm |  | Flood |  |
| 12:33 PM | 594 | 121 |  |  |  | 473 cm |  | Flood |  |
| 1:00 PM |  |  |  |  |  | 509 cm |  | Flood |  |
| Currents | CM/30 SEC | Cm per sec |  |  |  | Knots/hr |  | North/South Ebb/Flood/Still |  |
| 9:18 AM | 16.5 | .55cm/sec |  |  |  | 0.0107 |  | Flood |  |
| 9:42 AM | 15.68 | .52 cm/sec |  |  |  | 0.0101 |  | Flood |  |
| 10:12 AM | 18.85 | .63 cm/sec |  |  |  | 0.0122 |  | Flood |  |
| 10:35 AM | 19.6 | .65 cm/sec |  |  |  | 0.0126 |  | Flood |  |
| 10:55 AM | 15.35 | .51 cm/sec |  |  |  | 0.0099 |  | Flood |  |
| 11:15 AM | 15.69 | .52 cm/sec |  |  |  | 0.0101 |  | Flood |  |
| 12:20 PM | 17.43 | .58 cm/sec |  |  |  | 0.0113 |  | Flood |  |
| 12:45 PM | 14.80 | .49 cm/sec |  |  |  | 0.0095 |  | Flood |  |
| 1:09 PM | 6.81 | .23 cm/sec |  |  |  | 0.0044 |  | Flood |  |
| 1:30 PM | 4.8 | .16 cm/sec |  |  |  | 0.0016 |  | Flood |  |
| Fish Catch Northside of pier | Number Caught | Species |  |  |  | Measure of largest |  | TIME & CATCH |  |
| Rotation #1 | 34 | Atlantic Silverside |  |  |  | 8 cm |  | 9:20 AM |  |
|  | 11 | Striped Bass |  |  |  | 12 cm |  | Diversity 6 |  |
|  | 3 | Herring |  |  |  | 13 cm |  | 49 total catch |  |
|  | 1 | Blue Crab |  |  |  | 5 cm |  | (Plus jellies) |  |
|  | Many | Comb Jelly |  |  |  |  |  |  |  |
|  | Many | Moon Jelly |  |  |  |  |  |  |  |
| Rotation #2 | 16 | Atlantic Silverside |  |  |  | 8 cm |  | 10:07 AM |  |
|  | 1 | Blue Crab |  |  |  |  |  | Diversity 4 |  |
|  | 8 | Jellies |  |  |  |  |  | 27 total catch |  |
|  | 2 | Crabs |  |  |  |  |  | Lots of water celery blades washed into net |  |
| Rotation #3 | 8 | Atlantic Silverside |  |  |  | 5 cm |  | 11:00 AM |  |
|  | 4 | Striped Bass |  |  |  | 10 cm |  |  |  |
|  | 1 | White Perch |  |  |  | 17 cm |  | Diversity34 |  |
|  | 1 | Blue Crab (F) |  |  |  | 6 cm |  | 14 total |  |
| Rotation #4 | 18 | Atlantic Silverside |  |  |  | 6 cm |  | 12:30 PM |  |
|  | 1 | Striped Bass |  |  |  | 7 cm |  |  |  |
|  | 7 | White Perch |  |  |  | 19 cm |  |  |  |
| *identified post trip | 1 | *Black Drum (Juvenile) |  |  |  | ~15 cm |  |  |  |
| **Fish trap | **1 | Naked Goby |  |  |  | 4 cm |  |  |  |
|  | 1 | Blue Crab (missing craw and swimmeret) |  |  |  | 15 cm |  | Diversity 7 |  |
|  | 2 | Jelly fish |  |  |  | 3 cm |  | 31 total catch |  |
| Rotation #5 | 103 | Atlantic Silverside |  |  |  |  |  | 1:10 PM |  |
|  | 3 | White Perch |  |  |  |  |  | Diversity 2 |  |
|  |  |  |  |  |  |  |  | 106 total |  |
|  |  | Grand Total |  |  |  |  |  | Diversity 7 (plus 2 types jellies) Count 227 |  |
| Plankton | 9:50 AM 10:50 AM 10:50 AM 12:10 PM 12:25 PM | Diatoms (all circular) Copepod – Calanus Copepod – Nauplius Pleurosigma (eliptical diatom) Coscinodiscus (Diatom chain) |  |  |  |  |  |  |  |
| Sediment Cores | Length | H2S Smell, Oil |  | Rare |  | Common |  | Abundant |  |
| 9:00 AM | 12 cm | H2S |  | Wood, brick |  |  |  | Clay, gravel, pebbles, |  |
| 9:30 AM | 20 cm | H2S & oil |  | Wood, brick |  |  |  | Clay, gravel |  |
| 10:00 AM | 15 cm – | Oil |  | Wood, brick |  |  |  | Clay, gravel, pebbles |  |
| 10:24 AM – core with 3 sections - 2 cm oxid. top 5 cm gravel 10 cm clay | 17 cm – 2 cm oxidized top, 5 cm gravel, 10 cm clay | H2S, Oxidized Top |  | Shells, Brick, Living Vegetation (roots) |  | Gravel (5 cm mid core) Pebbles, leaves, wood |  | Clay (10 cm bottom of core) |  |
| 10:40 AM –3 cm oxid. Top, 14 cm gravel | 17 cm – | H2S, oxidized top |  | Mud, sand, shells, brick |  | Pebbles |  | gravel |  |
| 11:00 AM |  | H2S, oxidized top |  | Sand, leaves, wood |  | Clay and brick |  | Gravel, pebbles |  |
| SHIPPING | TIME | Name |  | Color |  |  | North/South |  | Full/Empty |
|  | 9:16 Am | “S” |  | White/Navy |  |  | Southbound |  | passenger ship |
|  | 9:17 AM | Sailboat |  | White |  |  | Southbound |  |  |
|  | 11:00 AM | Pleasureboat |  | White |  |  | Northbound |  | People in it |
|  | 12:55 PM | TRAIN! |  |  |  |  |  |  |  |
|  | 1:28 PM | Tug |  | Black |  |  | Northbound |  | Full |

SCREEN CAPTURES FROM PIERMONT HRECOS FOR CHEMISTRY READINGS