Day in the Life of the Hudson River 10/15/14 Data

(Salt Front RM 67.5)

RIVER MILE 144

Rensselaer Boat Launch, Forbes Rd., Rensselaer, NY

Leah Penniman, Tech Valley High School - 9th grade

Environmental Science & Geography – 54 students, 3 Adults

Location: Rensselaer Boat Launch

Area: Boat launch – beach area

Surrounding Land Use: Boat launch with small beach, wooded, landscaped, adjacent to urban residential area

Sampling Site: boat launch with surrounding beach area

Plants in area:

Water depth: unknown

River Bottom :

Times: All sampling 11:45 AM to 1:15 PM

| ITEM | Time |  | Readings - |  | Averages |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature |  |  | 26C 27 27 26 27 27.5 17 26 24 25 |  | 25.5C |
| Wind Speed |  |  |  |  |  |
| Cloud Cover |  |  |  |  |  |
| Weather today | Clear and humid |  |  |  |  |
| Weather recently | Cool until last day or so has been humid |  |  |  |  |
| Water | calm |  |  |  |  |
| Water Temperature |  | 19C 18 18 20 17 18.5 18 17 16 17 |  |  | 17.85C |
| Turbidity (LaMotte) |  | 5cm 5 120 5 10 15 5 120 |  |  | 36.3 JTU |
| Chemical |  | PPM |  | % saturation | Average |
| DO (chemet color comparator) |  | 5 ppm 8 11 8 12 10 10 8 10 9 |  | 72.6% saturation (average) | 9.1 ppm |
| pH – pH meter |  | 7.25 6 7.25 7.79 7.4 7.92 6.4 7.95 7.6 |  |  | 7.3 |
| Phosphate - Chemet Color comparator |  | - mg/L 1.0 0.4 0.2 0.5 0.2 0.1 0.4 0.2 0.2 |  |  | 0.42 mg/L |
| Nitrate – LaMotte color comparator |  | 1.0 ppm 0 17.6* 0.4 0.45 0.03 |  | *didn’t include | 0.45 mg/L |
| Alkalinity |  | 60 ppm 50 130 |  |  | 80 ppm |
| Salinity |  | 50 ppm 75 |  |  | 63 ppm Total Salinity |
| Electrical Conductivity |  | 207 uS 107 |  | microseimens | 153.5 uS |
| Coliform Petrifilm plate 1 ml water • stated range is based on colonies per 100 ml sample |  | 0 1900 2000 300 9200 0 7 65 400 112 |  |  | 1398.4 average |
| Shipping Traffic |  | 2 boats |  |  |  |