Day in the Life of the Hudson River 10/16/14 Data

(Salt Front RM 65.9)

RIVER MILE 41

Steamboat Dock Park, Verplanck, Westchester

Bob Connick, Sott Rizzo, Mahopac High School;

Environmental Research and Field Biology 9th - 12th graders –14 students, 2 teachers

Latitude N 4114958 & 41.24936  Longitude W 73936474

Location: Steamboat Dock Park beach, City of Verplanck in Westchester County

Area: located in a small cove between a point and a bulkhead – Pier, Grassy, Parking Lot

Surrounding Land Use:  Boating, Picnics, Swimming, Fishing, Jogging

Sampling Site: beach, riprap, banks altered, dock, bottom rocky, bulkhead, pipe south of the sampling site 10-15 m.

Shoreline: Sandy, rocky

Plants in area:

Other: No plants in water

Water depth:  3 ft.

River Bottom – sandy, rocky

| ITEM | Time |  |  |  | Reading 1 | Reading 2 |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |  |
| Air Temperature |  |  |  |  |  |  |  |  |  |  |
| Wind Speed | 11:30 AM |  |  |  | Beaufort 5 |  |  |  |  |  |
| Cloud Cover | 11:30 AM |  |  |  | >75% cloud cover | Overcast |  |  |  |  |
| Weather today | 11:30 AM –cloudy, rainy |  |  |  |  |  |  |  |  |  |
| Weather recently | Humid and wet |  |  |  |  |  |  |  |  |  |
| Water Surface | Choppy |  |  |  |  |  |  |  |  |  |
| Water Temperature – | 11:30 AM |  |  |  |  |  | 21.1oC 21.1oC |  |  |  |
| Turbidity – | 11:35 AM |  |  | ~43 cm Transparency tube |  |  | 14 NTU |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |  |
| DO (drop kits) | 11:43 AM |  |  | 21.1 oC |  |  | 9 ppm |  |  | 98% Saturation |
| pH Indicator solution | 11:30- 11:40 AM |  |  | 8.0 7.6 |  |  |  |  |  | 7.8 average |
| Salinity YSI SCT meter | 11:25 AM |  |  | 5.0 ppt |  |  |  |  |  |  |
| Alkalinity (Hach dropcount) | 11:43 AM |  |  | 80 mg/L |  |  |  |  |  |  |
| Nitrates (Hach color kit) | 11:45 AM |  |  | 0 mg/L |  |  | Used Hach colorimeter w/10 mg/L standard – read 9.6 mg/L |  |  |  |
| Phosphate (colorimeter) | 10:50 AM |  |  | 0.57 ppm |  |  | Used Hach colorimeter w/3.0 ppm standard – read 2.89 ppm |  |  |  |
| Fish Catch- 12:05 AM | Number Caught |  |  | Species |  |  | Size of largest |  |  | Seine |
| 25 ft net X 4ft– 300 ft. pull | 3 |  |  | Striped Bass |  |  |  |  |  |  |
|  | 2 |  |  | Atlantic Silversides |  |  |  |  |  |  |
|  | 18 |  |  | Shiners |  |  |  |  |  |  |
| Total Catch | 23 |  |  | Total Diversity 3 species |  |  |  |  |  |  |
| CPUE | 25 ft x 100 ft |  |  | 23 fish/762 m |  |  | 0.030 CPU |  |  |  |
| Coring |  | Unable to collect |  |  |  |  |  |  |  |  |
| Tides* |  | TIME | Amt. Rise/ Fall |  |  |  |  |  |  |  |
|  |  | *Stick was swept away by wind, waves and current |  |  |  |  |  |  |  |  |
| Current * |  | Time |  | Direction |  | Ebb/Flood |  | Speed |  |  |
|  |  | *not recorded |  |  |  |  |  |  |  |  |
| Traffic - commercial |  | Type |  | Name |  | North/South |  | Loaded/Light |  |  |
| 11:140 AM 12:00 PM 12:00 PM |  | Tug |  | McRussel Kape Wendy Thomas DWhite |  | Southbound Southbound Southbound |  | Loaded |  |  |
| Birds |  | 6 Ring Bill Gull, 2 Canadian geese, 6 Turkey Vulture, 6 Mallards, 4 Cormorants, 1 Peregrine Falcon |  |  |  |  |  |  |  |  |