Day in the Life of the Hudson River 10/20/16 Data

(Saltfront ~ RM 83)

RIVER MILE 37 – Bowline Park, Haverstraw Waterfront

Chuck Barone, Nanuet High School – 30 students, 12th grade APES and NAPES

2 adults

41. 12’8” N  73. 57’ 33” W

Location: Bowline Park, Haverstraw waterfront, Rockland County NY

Area: Parking lot, recreation, fishing

Surrounding Land Use: 10% beach, 75% Industrial/commercial, 15% urban/residential

Sampling Site: shoreline is sandy, rocky, small beach, bank altered, large rip rap, bulkhead, industrial tress spans the water to the north and east, bricks in the sand

Other Notes: The beach has a distinctive marker showing the high tide line or more commonly referred to as the ‘rack line’ where natural and man made debris line up

Plants in area: 50% algae, 50% seaweed/moss

Water depth:

River Bottom – sandy, squishy, mucky

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 8:30 AM 9:30 AM |  | 20.1°C 19.3 | 69.2°F 66.7°F |  |
| Wind Speed | 8:30 AM __________ 9:30 AM |  | 4.7 mph 3.0 0 ________ 2.3 mph 0 0 | Maximum Average Minimum __________ Maximum Average Minumum | North ___________ North |
| Cloud Cover | Mostly cloudy |  |  |  |  |
| Weather today | Mostly Cloudy |  | 51-75% |  |  |
| Weather last 3 days | unseasonably warm and dry |  |  |  |  |
| Water | calm |  |  |  |  |
| Water Temp. avg depth 2.5 ft | 8:30 AM 9:30 AM |  | 18,19, 21°C 19,22,22°C | 62,66.70°F 56.8, 71, 71°F | 19.3C & 66F 21 C & 70 F |
| Turbidity – Long site tube | 8:30 AM 9:30 AM 10:30 AM 11:30 AM 12:30 PM |  | 21,22,23 cm 35, 40, 36 42, 40, 36 40.8,39.2, 42.2 47.6,42.2, 44.4 | AVERAGES | 23 cm 37 cm 41.6 cm 41.46 cm 45.1 cm |
| Chemical |  |  |  |  |  |
| DO Drop count | 9:30 AM 8:30 AM 9:30 AM 10:30 AM 11:30 AM |  | 10°C 7.5 ppm 7 5.5 6.7 | 8 ppm 19.3 C 21 C 20C 20.6 C | 70 % sat. 80% 75% 60% 75%. |
| pH – litmus paper | 8:30 AM 9:30 AM |  | 5,4,5 4,4,4 | AVERAGES | 4.6 4 |
| Salinity: Hydrometer | 8:30 AM 9:30 AM 10:30 AM 11:30 AM | 11, 10, 11 ppt 10, 10, 11 9,9,10 10, 10 11 |  | AVERAGE | 10.6 ppt 10.5 ppt 9.7 ppt 10.2 ppt |
| Fish Catch - Seine 4 meters. Mesh 0.25 inch | Number Caught |  | Species | Length of largest |  |
| Fish - Seine 1 8:30 AM | 2 |  | Striped Bass | 10.1 cm | DIV 3 TOTALS 4 |
|  | 1 |  | Atlantic Silverside | 8.8 cm |  |
|  | 1 |  | Bay Anchovy | 8.9 cm |  |
| Inverts | 1 |  | Shrimp | NR | DIV – 2 |
|  | 1 |  | Crab | NR | TOTALS 2 |
| Seine 2 9:30 AM | 3 |  | Atlantic Silverside | 9 cm | DIV – 3 TOTALS - 10 |
|  | 5 |  | Striped Bass | 20 cm |  |
|  | 2 |  | White Perch | 16.1 cm |  |
| Seine 4 | 3 |  | Blue Fish | NR | DIV 4 |
| Noon | 3 |  | Herring (sp) | NR | TOTALS 10 |
|  | 3 |  | Blue Herring | NR |  |
|  | 1 |  | Bay Anchovy | NR |  |
| Tides | 8:30 AM 9:30 AM 11:30 AM 12:45 PM |  | 73 cm 85 cm 75 cm tide stick under H2O | Rising Rising Rising | 0.2 cm/min 0.16 cm/min |
| Currents | CM/30 secs |  | Cm/Sec | North/South Ebb/Flood/Still | Knots |
|  | 106.67 |  | 3.56 cm/sec | South ebb | 0.069 |
|  | 109.36 |  | 3.65 cm/sec | North flood | 0.071 |
|  | 52 |  | 1.73 cm/sec | North flood | 0.034 |
|  | 110 |  | 3.67 cm/sec | North flood | 0.07 |