Day in the Life of the Hudson River 10/12/17 Data

(Salt Front ~ RM 65)

RIVER MILE 11.5 – Fort Washington Park

Katie White, Marymount School of New York

53 students 7th grade,  8 adults

Latitude 40°50’46.42444” N -  Longitude -73.°56’46.914”W

Location: Fort Washington Park just South of the George Washington Bridge

Area: Site is a sandy beach with rip rap adjacent to a paved jogging trail within a grassy park. Some trees on far side of jogging trail but none next to water. Very little aquatic vegetation.

Surrounding Land Use: 70% urban /residential, 25% forested, 5% beach,

Sampling Site: Park with a lot of trees, picnic tables, covered with vegetation, lots of riprap shore, collected wood debris. Large outflow pipe directly south of site <100 meters

Plants in water: None

Water depth: 2-4 ft.

Water: Choppy

River Bottom –mainly sandy and rocks, no mud this year (past years yes)

| ITEM | Time | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- |
| Physical |  |  |  |  |
| Air Temperature | 9:40 AM 10:10 AM 10:45 AM 12:20 PM 12:55 PM | 15°C 15 14.5 16 16.5 |  |  |
| Wind Speed – | Time | Clouds | Beaufort # | Wind m/s |
|  | 9:45 AM | Overcast >75% | 2 - N | 1.2 |
|  | 10:12 AM | Overcast >75% | 2 - N | 1.5 |
|  | 10:47 AM | Overcast >75% | 2 - N | 1.6 |
|  | 12:23 PM | Overcast >75% | 3 - N | 3 |
|  | 12:56 PM | Overcast >75% | 2 - N | 2 |
| Weather today | Rain | Light overnight rain until 9AM |  |  |
| Weather last 3 days | Weather for past 3 days: Rainy or overcast – high ~75 degrees F |  |  |  |
| Water Temperature | 9:35 AM 10:00 AM 10:38 AM 12:15 PM 12:44 PM | 20°C 20°C 20°C 20°C 20°C |  |  |
| Turbidity - Long site tube | 9:55 AM 10:13 AM 10:45 AM 12:20 PM 12:53 PM | 42.3 cm 43 cm 59 cm 86 cm 84cm |  |  |
| Chemical |  |  |  |  |
| DO Salifert Kit (each measurement is avg. of 4-5) | 10:02AM 10:28 AM 10:59 AM 12:36 PM 1:07 PM | 20°C 20 20 20 20 | 5 ppm 5 7.5 7.5 6 | 50% saturation 50% 85% 85% 65% |
| pH color match test kit - (each measurement is avg. of 4-5) | 9:45 AM 10:13 AM 10:45 AM 12:23 PM 12:53 PM | 7.5 8 7.7 7.8 8 |  |  |
| Salinity |  | refractometer | hydrometer |  |
|  | 9:40 AM 10:20 AM 10:45 AM 12:20 PM 12;55 PM | 22 ppt 20 20 20.5 20 | 24.25 ppt 23 21 20.25 20.25 | . |
| Fish Catch | Time | Fish Species | # caught | Length cm |
| Seine #1 | 9:39 AM | Atlantic Silversides | 110 | 10.5 |
| #2 | 9:49 AM | Striped Bass | 1 | 13 |
| #5 | 10:15 AM | Atlantic Silversides | 24 | 10 |
| #8 | 10:45 AM | Atlantic Silversides | 2 | 6 |
| #9 | 12:00 PM | Atlantic Silversides | 44 | 9.5 |
| #12 | 12:33 PM | Atlantic Silversides | 7 | unknown |
| TOTAL DIV – | 2 | TOTAL CATCH | 188 |  |
| Macroinvert | Number | Macros | Number | Additional Info |
| Seine #1 | 9:39AM | hermit crab | 1 |  |
| #2 | 9:49 AM | Blue Crab | 2 | F |
|  |  | Hermit crab | 2 |  |
|  |  | Swimming crab | 2 |  |
| #3 | 10:06 AM | Blue Crab | 1 | Male |
|  |  | Swimming crab | 1 |  |
|  |  | Hermit crab | 2 |  |
|  |  | Mud Snail | 1 |  |
|  |  | Ctenaphore | 1 |  |
| #4 | 10:15 AM | Swimming Crab | 3 |  |
|  |  | Hermit crab | 1 |  |
|  |  | Mud snail | 1 |  |
|  |  | Ctenaphore | 2 |  |
| #5 | 10:20 AM | Mud snail | 1 |  |
| #6 | 10:30 AM | Swimming crab | 4 |  |
|  |  | Mud snail | 1 |  |
|  |  | Ctenaphore | 2 |  |
| #7 | ? | Mud snail | 1 |  |
|  |  | Blue Crab | 1 | Male |
| #8 | 10:45 AM | Swimming crab | 1 |  |
|  |  | Blue crab | 1 | F |
| #10 | 12:15 PM | Swimming crab | 1 |  |
| #11 | 12:30 PM | Swimming crab | 1 |  |
| #14 | ? | Swimming crab | 1 |  |
| DIVERSITY | 5 | TOTALS 35 |  |  |
| Tides | 9:42 AM | Rising | 22.5 cm |  |
| Data collected | 10:16 AM | Rising | 37 |  |
| With stick in | 10:46 AM | Rising | 50.5 |  |
| the sand | 11:48 AM | Rising | 80 |  |
|  | 12:24 PM | Rising | 105 |  |
|  | 12:54 PM | Rising | 110 |  |
| Currents | Cm/Sec | Ebb/Flood/Still | Note |  |
| 9:42 AM 10:20 AM 10:46 AM 12:27 PM 1:00 PM | 20.05 cm/s 1.14* 27.46 72.99 41.65 | Ebb Ebb Ebb Flood Flood | *Had trouble throwing the orange far enough into the channel. Results are averages of 2 or 3 tosses. The current may reflect side current which was unusually strong from noon on. |  |
| Shipping – Commercial | 9:37 AM | Cargo Ship | Docked | Loaded |
|  | 9:53 AM | Small Cruise Boat to Statue of Liberty | North | Loaded |
|  | 10:43 AM | Coast Guard | North | Light |
|  | 12:12 PM | Coast Guard | North | Light |
|  | 12:12 PM | Cruise – Circle Line | South | Loaded |
| Recreational | 9:53 AM | Recreational Sailboat | South | n/a |
|  | 10:23 AM | Sail boat | South | n/a |
|  | 12:47 PM | Recreational – Elastic Failure | North | n/a |
|  | 12:50 PM | Recreational | North | n/a |