Day in the Life of the Hudson River 10/12/17 Data

(Saltfront ~ RM 65)

RIVER MILE 117.5 (a)

Hudson River Waterfront Park, City of Hudson

Shelley Forester, Cary Institute of Ecosystem Studies

Sandra Fischer, Chatham High School, Catham, NY

60 students, Environmental Science, Ecology Club, 8 adults

Latitude – N 4225’ Latitude – W -7380’

Location: Hudson River Waterfront Park, City of Hudson, Columbia County

Area: 1st dock in the waterfront property used for fishing, marina, picnics, large grassy area, just north of the old quarry site, has a parking lot –

Surrounding Land Use: 50% Industrial/Commercial, 50% Urban Industrial

Sampling Shoreline: waterfront with banks/shoreline is 70% is rocky and 30% muddy.

Sampling Site: 100% altered to include wooden  & concrete bulkheads and large riprap on shoreline, pier, grassy, parking lot

Plants in area: 0.5% % water chestnut, 0.5% water celery, 1% milfoil

Water - Calm

River Bottom – bottom rocky, muddy

|  | Time | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- |
| Physical |  |  |  |  |
| Air Temperature | 9:10 AM 9:50 AM 12:00 PM | 12C 13C 15C | F |  |
| Cloud Cover | 8:51 AM | Mostly Cloudy 51-75% |  |  |
| Wind – | Time AM | Beaufort N/R | m/s - meter 1-4 | Direction ENE |
| Weather today |  |  |  |  |
| Weather recently | Mild, no precipitation |  |  |  |
| Water | calm |  |  |  |
| Water Temperature – | 9:12 AM 12:07 PM 12:15 PM | Surface Surface 2.9 meters depth | 19.8, 19.7C 20.4, 20.4 21, 20.8 | 19.75 C avg 20.4 21 |
| Turbidity – 120 cm long site tube | 9:30 AM to 12:30 PM | 93 cm 97 cm 95 cm | < 10 NTU |  |
| We felt the water was not very turbid. You could see 1m down from the dock and kids were surprised by the results from the long tube. |  |  |  |  |
| Chemical | Time | Temp C | DO ppm | % sat. |
| DO - meter | 9:15 AM 10:20 AM 12:08 PM | 20 20 21 | 7.65 7.96 7.87 | 85% 89% 89% |
| pH – Aquacheck strips | 9:35 AM 10:16 AM 12:06 PM | 5, 5, 6 5, 6, 6 6, 5, 6 | averages | 5.3 5.6 5.6 |
| Salinity Quantabs | Time 9:15 AM 10:19 AM 12:41 PM | Quantab # 1.4 1.4 1.4 | Chloride ppm 32 ppm 32 ppm 32 ppm | Total Salinity 58 ppm 58 ppm 58 ppm |
| Nitrates - Aquacheck strip | 9:48 AM | 2 ppm 2 ppm |  |  |
| Phosphates – |  |  |  |  |
| Fish Catch Seine net 6 feet X 4 ft., 1.4 inch mesh | Time | Species | Total Catch for Day | Length |
| Fish |  | Banded Killifish | 38 | 7 cm |
|  |  | Bluegill Sunfish | 37 | 9 cm |
|  |  | White Perch | 2 | 4 cm |
|  |  | Bay Anchovy | 2 | 3 cm |
|  |  | Shiner | 2 | 7.5 cm |
| Fish Totals | DIV 5 | TOTAL | 81 |  |
| Macros |  | Cladocera, Waterfleas | 2 |  |
|  |  | Volvox algae | 2 |  |
|  |  | Ostracod - Crustacea | 1 |  |
|  |  | Amphipod | 2 |  |
| Macros Totals | DIV 4 | TOTALS | 7 |  |
| Tides | 8:52 AM 9:23 AM 9:50 AM 11:55 AM 12:22 PM 12:37 PM | 185 cm 200 cm 180 cm 178 cm 160 cm 149 cm | Set point Rising Falling Falling Rising Rising | On a dock measuring down to water |
| Currents | Time | Cm/30sec | Cm/sec | E/F/S |
|  | 8:55 AM. |  | 38.1 cm/sec | Ebb |
|  | 9:15 AM |  | 25.4 cm/sec | Flood |
|  | 9:26 AM |  | 224 cm/sec | Flood |
|  | 11:41 AM |  | 25 cm/sec | Ebb |
| Core Sample | Length/ | Rare | Common | Abundant |
| Collected in vegetated area | - 20.3 cm (8 inches) - no smell | Leaves Wood Brick Living Vegetation | Clay Mud Sand Gravel Pebbles |  |
| Traffic - commercial | Time | Name | Color | Direction/Loaded or Light |
|  | 10:45 AM | Coral Coast | Blue | Northbound, light |
|  | 10:59 AM | RTC107 | Black | Southbound, loaded |
|  | 11:04 AM | No Name | Black | Northbound, light |
|  | 11:45 AM | Coastguard | Orange | south |
|  | 12:15 PM | Doubleskin | Black/Yellow | Southbound, loaded |
| Recreational | 11:13 AM | Sailboat | Black | Northbound |
|  | 11:40 AM | Speed Boat | White | Southbound |
|  | 11:55 AM | Hudson Spirit | Red/White | Southbound, Loaded |
| Other Observations | Not overgrown, public park. Grassy peninsula with rocky shore, 1 pear, boat launch No bricks, coal, shells etc, manicured park The area is inlets with docks so perhaps the water is not moving as freely which might affect sampling. There is a creek from an industrial site flowing into the river There is rock processing next door as well. Lots of yellow jackets and a few seagulls in the area |  |  |  |