Day in the Life of the Hudson River 10/12/17 Data

(Salt Front ~ RM 65)

RIVER MILE 30.5 – Nyack Beach Hook Mountain State Park, Steps

Nicole Laible, Brianna Rosamilia, Rockland County Division of Environmental Resources

Peter Park, Nyack College

Jessica Flanagan, Clarkstown North HS 14 students grades 11/12, 7 adults

41.12049N  -73.911339W

FISH SAMPLING FROM SITE

Location: Nyack South Beach, Upper Nyack, Rockland County NY

Area: Park/Boat launch, near Hook Mountain Sandy Beach by the large sitting stairs, Surrounding Land Use: 40% forested, 10% industrial/commercial, 40% beach – used for recreation & picnics

Sampling Site: Beach – sandy and rocky at edges, muddy offshore, covered with vegetation beyond the beach, stone wall, riprap shore, collected wood debris.

Piping coming into river south of sampling site

Water depth:

River Bottom –muddy, sandy, rocky

Plants: None growing in area

| ITEM | Time | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- |
| Physical |  |  |  |  |
| Air Temperature | 10:30 AM 11:55 AM 12:18 PM | 13°C 15.5°C 15°C | 56°F 60°F 59°F |  |
| Wind Speed | NR |  |  |  |
| Cloud Cover | Mostly overcast | 51-75% |  |  |
| Weather today | Light rain in AM |  |  |  |
| Weather last 3 days | Warm and rain yesterday |  |  |  |
| Water Temperature | 9:56 AM 10:32 AM 12:12 PM | 20.65 C 20.3 C 20.12 C |  |  |
| Turbidity – Long site tube | 10:10 AM 10:40 AM 12:00 PM 12:24 PM | 42 cm 43 cm 30.4 cm 38 cm |  |  |
| Chemical |  |  |  |  |
| DO Drop count | 9:56 AM 10:32 AM 11:44 AM 12:12 PM | 8.72 mg/L 8.33 mg/L 8.51 mg/L 9:08 mg/L | 20.65 C 20.3 C NR 20.12 C | 102.5 % 97.9% NR 105% |
| pH - YSI | 9:59 AM 10:39 AM 11:47 AM 12:15 PM | 7.8 7.87 7.91 7.91 |  |  |
| Salinity – quantabs 8.4 quantab units High Range | 10:05 AM 10:30 AM | 4708 ppm 4708 ppm Cl- | 8505 ppm TS 8505 ppm TS | TS = Total Salinity |
| Fish Catch 8:55 AM |  | Number Caught | Species | Latin Name |
| Seine 30 ft. |  | 19 | Atlantic Silverside | Menidia menidia |
|  |  | 5 | Striped Bass | Morone saxatilis |
|  |  | 5 | Northern Pipefish | Synanathus fuscus |
|  |  | 22 | White Perch | Morone americana |
|  |  | 6 | Bay Anchovy | Anchog mitchilli |
|  |  | 12 | Banded Killifish | Fundulus diaphanus |
|  |  | 3 | Bluefish |  |
|  |  | 1 | Herring |  |
|  |  | 2 | Silver Perch |  |
|  |  | 1 | Mummichog |  |
| FISH TOTAL DIV | 10 | TOTAL CATCH | 76 |  |
| MACROINVERT |  | 1 | Moon Jelly |  |
| DIVERSITY |  | Too numerous to count but | Grass Shrimp | Palaemonetes spp. |
| TOTALS = Too numerous to count |  | 1 | Sand Shrimp | Crangon septemspinosa |
|  |  | 2 | Clam | Mercenaria mercenaria |
| MACRO DIVERSITY | 4 | TOTAL COUNT | 4 plus many Grass Shrimp |  |
| Tides | Time | Change in Time | Change in Height | Rate of change Flood/Ebb |
|  | 9:58 AM |  | 20 cm |  |
|  | 10:24 AM | 26 mins | 17 cm | 3cm/26 mins Ebb |
| RESET STICK | 11:48 AM |  | 25 cm |  |
|  | 12:10 PM | 22 mins | 32 cm | 7 cm/22 mins Flood |
| Currents |  |  |  |  |
| Distance Travelled | Time | Distance from Shore | Current Speed (cm/s) | Flood/Ebb |
|  | 9:58 AM |  |  | Ebb |
|  | 10:24 AM |  |  | Ebb |
|  | 11:48 AM |  |  | Flood |
|  | 12:10 PM |  |  | Flood |
| Sediment Core | Time | Description | Rare | Abundant |
| 3 feet water depth | 10:45 AM | 10 cm length | Clay Mud Brick Coal Slag | Sand Gravel Pebbles |
| Commercial Boat | 12:08 PM | Red Barge | Southbound | Loaded |
|  | 12:29 PM | Tugboat | Northbound | Loaded |
| Recreational Boat | 11:40 AM | White Sailboat | NR |  |
|  | 12:08 PM | White Sailboat | Southbound |  |
|  | 12:38 PM | Fishing boat | Northbound |  |
|  | 12:48 PM | White Sailboat | Southbound |  |
| Observations | Bricks, lumps of coal, slag, devil’s heads | Building a new bridge which could impact our sampling | Ducks and crane (sandhill?) |  |