Day in the Life of the Hudson River 10/12/17 Data

(Salt Front ~ RM 65)

RIVER MILE 61.2

Beacon Landing Park, City of Beacon, Dutchess County

Will Gunther, Nancy Fabrizio, Nancy Rypkema - Valley Central Middle School,

7th & 8th graders (95 students and 12 adults)

Latitude 41.507975, Longitude -73.986708

Location: Beacon Landing Waterfront Park, Beacon, Dutchess County NY

Area: The park at the end of Red Flynn Drive (past long dock park) near the Beacon train station – center at the park with stations all around it and fish in the cove at the end of the park - grassy lawns, forested around edges, parking lot to the side. Children’s play area, large riprap

Surrounding Land Use: Urban/residential 100% (Parkland)

Shoreline: A bridge, train station, covered in vegetation trees, concrete debris, dock

Sampling Site: beach, covered in vegetation, banks altered, rip rap

Plants in the water: no plants in the water

Water depth: <1 m

Bottom Surface: muddy

| ITEM | Time |  | Reading 1 | Reading |  | Comments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |
| Air Temperature | 9:50 AM 10:09 AM 10:41 AM 11:00 AM 11:20 AM 11:42 AM 12:09 PM 12:42 PM |  | 14.2C 15 15.4 16.4 17.8 17 18.2 17 | 58F 58.6 59.7 61.4 64 62.9 64 63.3 |  |  |  |  |
| Wind Speed | Beaufort |  | #2 | 5 kts |  |  |  |  |
| Cloud Cover | Partly Cloudy |  | 26-50% |  |  |  |  |  |
| Weather today | No rain |  |  |  |  |  |  |  |
| Weather recently | Rain on Monday, Warmer Tuesday and Wed. mid 70s, |  |  |  |  |  |  |  |
| Water Surface | Choppy |  |  |  |  |  |  |  |
| Water Temperature | Time | Degrees C |  |  |  |  | Average |  |
|  | 9:46 AM 10:15 AM 10:40 AM 12:16 PM | 10C 12 12 14 |  |  | X3 X3 12,13,12 X3 |  | 10C average 12 12.7 14 |  |
| Turbidity- long sight tube | 12:00 PM | 43 cm 45 35 |  |  |  |  | 41 cm average |  |
| Chemical | Time | DO |  |  | Temp |  | % saturation |  |
| DO (drop kit) | 9:46 AM | 9 ppm |  |  | 10C |  | 80% |  |
| pH – litmus paper | 11:51 AM | 6 7 7 |  |  |  |  | 6.7 average |  |
| Salinity –QTs- HR | Time | Chloride reading Cl- |  |  |  |  | Total Salinity conversion |  |
|  | 12:30 PM | 546 ppm 598 ppm 654 ppm |  |  |  |  | 986 ppm TS 1080 ppm 1181 ppm |  |
| Fish Catch seine 8X4feet | Number Caught | Species |  |  | Length |  |  |  |
| 4X6 ft. pull | 10 | Shiner |  |  | 7.5 cm |  |  |  |
|  | 3 | Striped Bass |  |  | 12.7 cm |  |  |  |
|  | 2 | Sunfish |  |  | 8.0 cm |  |  |  |
|  | 1 | Bluefish |  |  | 20.5 cm |  |  |  |
| DIV | 4 | TOTAL |  |  | 16 |  |  |  |
| Macroinvert | NONE |  |  |  |  |  |  |  |
| Sed. Core | NR |  |  |  |  |  |  |  |
| Tides | Time | Tide Stick |  |  | Flood/Ebb Still |  | Rate of change – cm/min |  |
|  | 10:01 AM 10:12 AM 10:38 AM 10:54 AM 11:23 AM 11:45 AM 12:09 PM | 30 cm 30 cm 28 cm 25 cm 25 cm 23 cm 20 cm |  |  | Falling Unchanged Falling Falling Unchanged Falling Falling |  | NA 0 0.125 0.125 0 0.08 0.125 |  |
| Currents | Time | Cm/30 secs |  |  | Cm/sec |  | knots | Ebb Flood |
|  | 10:55 AM 11:45 AM 12:35 PM | 152 216 162 |  |  | 2.5 3.6 2.7 |  | 0.05 0.07 0.05 |  |
| There is a 15’ rock outcropping. We attempted to measure the current in the main channel of the Hudson beyond the outcrop. |  |  |  |  |  |  |  |  |