Snapshot Day 10/12/17

(Salt front ~RM 65)

RIVER MILE 13

Bloomers Beach @ Engelwood Boat Basin

Natalie Macke – Pascack Hills High School AP Environmental Science Students

13 students, 11-12th graders

N40.881383	 -W73.943899

Location: Bloomers Beach – north end

Area: sandy beach, banks altered, pier with rip rap sections, wood bulkhead, forested area behind, with parking lot in front. Collected wood/debris in area, piping approx. 30” located at sampling site, 2 concrete bulkhead and stone breakwaters as well as on shoreline,

Surrounding Land Use: 25% forested; 25% beach; 50% Industrial/commercial

Shoreline: Loamy sand – sandy and muddy

Bottom: Muddy

Plants in area: none in the water

| ITEM | Time |  | Reading 1 | Reading 2 | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 9:32 AM 9:57 AM |  | 11.7C 11.1 | 55F 52 |  |
| Wind Speed | 9:36 AM |  | Beaufort #5 | 5.7 m/s | SW |
| Cloud Cover | 9:40 mostly overcast |  |  |  |  |
| Weather today | A little rain in the morning |  |  |  |  |
| Weather recently | Dry and warm until yesterday, then some rain |  |  |  |  |
| Water | Choppy |  |  |  |  |
| Water Temperature | 9:33 AM 10:07 AM 10:12 AM | 22C 22C 20C |  | F F F |  |
| Turbidity – long site tube | 9:35 AM | 5.5 cm 6.1 cm 5.1 cm |  | 5.567 cm |  |
| Description | Yes the water was very turbid. It was very murky. There was barely any water before we could see the bottom. |  |  |  |  |
| Chemical |  |  |  | Water | Saturation |
| DO –drop count | 9:33 AM 10:07 AM 10:12 AM | 8.4 mg/L 7.65 mg/L 7.12 mg/L |  | 22C 22C 20C | 95% 75% 85% |
| pH – litmus paper | NR |  |  |  |  |
| Salinity (refractometer) | 9:33 AM | 20 ppt |  |  |  |
| Fish Catch | Count | Fish |  | Size of largest | Macros |
| Tides | NR |  |  |  |  |
| Currents 9:46 AM | 1097/30secs | 37cm/sec |  | 0.72 kts | South/ebb |
| Core – 9:33 AM | Length: 3 inches | Other: No Scent No oil |  | Common: Sand Pebbles | abundant: Clay, Mud, Gravel |
| Location of core N40.881338 -W73.944172 |  |  |  |  |  |
| Commercial Boat Traffic | 9:36 AM | Ferry – Red |  | SW | Loaded |
| Recreational Traffic | 9:36 AM | Sailboat – white |  | SW | Light |
| Other Observations | Seagull Dog | Artificial shoreline & Pier/dock area could affect data |  |  |  |