Day in the Life of the Hudson River 10/16/18 Data

BROOLYN BRIDGE PARK (WEST), MANHATTAN

RIVER MILE: East River 2.3

Salt Front: ~RM 24

Leman Manhattan Preparatory School- Bill Livingston

60 students 10th -12th grade, 6 adults

BioBoat & BioBus- Mollie and James

Latitude 40.7076 N & Longitude – 73.99982 W

Location: Brooklyn Promenade- Brooklyn Bridge, Manhattan, NY

Area: Beach under the Brooklyn Bridge- 125 feet north of Dover street, west side of the East river. Pier and path for recreational walking, jogging, bike riding, and fishing. Surrounding land is a sandy beach with water. Water used for fishing and boating; beach is open to the public but no swimming; lots of rocks and sand; near parking lot.

Surrounding Land Use: 60% Urban Residential; 30% Industrial/Commercial; 10% Beach

Sampling Site: Sandy, beach, covered in vegetation, pier; banks altered; bulkhead with timbers; debris in area such as broken concrete, docking; riprap; pipe entering river (south CSO ~40 meters- not visible).

Water: Choppy, 4 feet deep

River Bottom: Sandy, rocky (pebbles and stones)

Plants: 3% plants covering the water area: Irish moss and Gracela

| ITEM | Time | Reading |  | Comments |
| --- | --- | --- | --- | --- |
| Physical |  |  |  |  |
| Air Temperature: | 10:00 AM 11:00 AM | 13.1 o C 18.61 o C | 55.5 o F 65.5 o F |  |
| Wind Speed: | 10:00 AM 11:00 AM 1:00 PM 2:00 PM | Beaufort: 4 Beaufort: 3 Beaufort: 2 Beaufort: 3 | South East East Southwest |  |
| Weather Today: | No Rain; Partly cloudy (26-50%) |  |  |  |
| Weather Previous: |  |  |  |  |
| Water Temperature | 10:00 AM 11:00 AM 2:00 PM | 17 o C 16 o C 14.9 o C | 62.6 o F 60.8 o F 58.82 o F |  |
| Turbidity - Long site tube | 9:40 AM 10:20 AM 1:10 PM 1:50 PM | Avg: 72.8 cm Avg: 68 cm Avg: 100 cm Avg: 145 cm |  |  |
| Chemical |  |  |  |  |
| Salinity- hydrometer | 9:40 AM 10:20 AM 1:30 PM 2:10 PM | 28 ppt 19 ppt 18.5 ppt 17.7 ppt | *Big adjustment between first and second reading; falling salinity with a rising tide is an interesting phenomena that may be due to a the excessive rain this fall. |  |
| DO – | Time | Temp | ppm DO | % saturation |
|  | 10:00 AM 1:50 PM 2:20 PM |  | 7 7.2 7 | 80 70 68 |
| pH - pH strips | 9:40 AM 10:20 AM 1:30 PM 2:20 PM | 8 7 7 7 |  |  |
| Nitrates – Flinn/Lamotte Color Comparator Kits | 9:40 AM 10:20 AM 1:40 PM 2:20 PM | 8.8 ppm 4.4 ppm 4.4 ppm 8.8 ppm |  | * Bill noted teacher had double-checked the results but kits are low resolution. These readings are really high |
| Phosphates – Flinn/LaMotte Color Comparator Kits | 9:40 AM 10:20 AM 1:40 PM 2:20 PM | 8.8 ppm 4.4 ppm 4.4 ppm 8.8 ppm |  |  |
| Fish – Seine Length of seine: 20’ Width of seine: 4’ Length of seine pull: 20’ Number of seines: 6 | Number | Name | Length of longest |  |
|  | 26 9 | Striped Bass Atlantic Silverside | 18 cm 8.5 cm |  |
|  | TOTAL: 35 | Diversity: 2 |  |  |
| Macroinvertebrates- hand net in tide pool | Number | Name | Length of longest | 2 F, 3 M |
|  | 2 5 20+ Encrusting 50% of rocks | Hermit Crab Asian Crab Sand Shrimp Barnacles |  |  |
|  | TOTAL: 27 | Diversity: 4 |  |  |
| Tides- | Time | Rising, Falling, Still | Height (cm) | Rate of Tidal Change (cm/min) |
|  | 9:40 AM 10:00 AM 10:40 AM 11:00 AM 1:40 PM | Falling Rising Rising Rising |  | - 0.05 0.0375 0.46 0.1238 |
| Current- *measured in a cove | cm/second | Rising, Falling, Unchanged | Height (cm) | Rate of Change (cm/min) |
| Currents Flow | Cm/second | Knots | North/South | Ebb/Flood/Still |
| 10:00 AM 10:20 AM 10:40 AM 1:30 PM 1:40 PM | 7.2 12.1 3.8 8.6 19.2 | 0.140 0.235 0.074 0.167 0.373 |  |  |