Day in the Life of the Hudson River 10/16/18 Data

RENSSELAER RIVERFRONT PARK

Rensselaer County, NY

RIVER MILE: 144.5

Salt Front: ~ RM 24

Poestenkill Elementary School & Delaware Community School-

120 students 4th grade and a total of 24 adults:

John Bishop and 3 other teachers, Poestenkill ES &

Willow Zielenski and 7 other teachers, Delaware Community School

Alene Onion, Sarah Rickard and 10 other NYS DEC Staff

2 Fish Biologist from Region 4

Latitude 42.642375 N & Longitude – 73.746393 W

(photo  shows dock and waterfront – by Michael Clark on Google maps)

Location: Rensselaer Riverfront Park, Rensselaer County, NY

Area: East side of the Dunn Memorial Bridge, 1.5 acre grassy park with walking esplanade, a few trees, fishing dock

Surrounding Land Use: Parkland (1.5 acre), Industrial/commercial, urban residential

Sampling Site: Off of the dock and waterfront

Shoreline: Sections with banks altered, sections left more natural

Sample Timeframe 10:00 AM – 1:00 PM

| ITEM | Time | Reading |  | Comments |
| --- | --- | --- | --- | --- |
| Physical |  |  |  |  |
| Air Temperature | 10:00 AM – 1:00 PM | 13 oC 15.2 oC 15.5 oC | 55.4 oF 59.36 oF 59.9 oF | Temperature at 4.6 meters |
| Wind Speed: Beaufort Force | 10:30 AM 11:00 AM 11:30 AM 12:00 PM 12:30 PM | 4-7 mph - - 4-7 mph 4-7 mph | Beaufort: #2 | Southeast Southeast - Southeast Southeast |
| Water Temperature | 10:00 AM – 1:00 PM | 14.43 oC 14.4 oC 14.43 oC 14.50 oC 14.54 oC | 57.97 oF 57.92 oF 57.97 oF 58.1 oF 14.54 oF |  |
| Turbidity - secchi disk | 10:00 AM- 1:00 PM | 180 cm, 190 cm, 230 cm, 180 cm |  | Avg: 195 cm |
| Chemical |  |  |  |  |
| Salinity- Low Range Quantabs | 10:00 AM – 1:00 PM | <1.4 quantab units | <32 ppm Cl- | Below Detection for this instrument |
| DO – | Time | Temp | ppm DO | Specific conductivity |
|  | 10:00 AM – 1:00 PM | 14.43 oC 14.4 oC 14.43 oC 14.50 oC 14.54 oC | 8.29 - 8.36 8.45 8.37 | 210.9 - 213.9 216.0 217.3 |
| pH – | 10:00 AM – 1:00 PM | 7.47, 7.63, 7.81, 7.68 |  |  |
| Fish | Number | Name | Length of longest |  |
| Fish Totals | TOTAL: | Diversity: |  |  |
| Macroinvertebrates | Number | Name | Length of longest |  |
| Overall assessment of water quality using macro community is GOOD! | 2 2 1 1 1 | Mayfly Riffle Beetle Damselfly Dragonfly Worm | High Quality High Quality Moderate Moderate Low Quality | Pollution Intolerant Pollution Intolerant Moderately Tolerant Moderately Tolerant Pollution Tolerant |
| Macroinvertebrate Totals | TOTAL: 7 | Diversity: 5 |  |  |
| Tides- | Time | Rising, Falling, Unchanged | Height (cm) | Rate of Tidal Change (cm/min) |
|  | 10:30 AM 11:00 AM 11:30 AM 12:00 PM 12:30 PM | - - Rising Rising Falling | 360 - 390 450 390 | - - .5 2 2 |
| Current- | cm/second | Knots | North/South | Ebb/Flood/Still |
|  | 25 28 19.5 18.7 | 0.49 0.54 0.38 0.36 | South South South South | Ebb Ebb Ebb Ebb |
| Boat Traffic Totals | Name | Color | North or South | Loaded or Light |
| 5 Boats Total |  |  |  |  |