Day in the Life of the Hudson River 10/16/18 Data

CORNING PRESERVE, ALBANY

Albany Boat Basin, NY

RIVER MILE: 145.5

Salt Front: ~ RM 24

Montessori Magnet School-

45 students 4th grade, 10 adults

Natalie Aiello, Montessori

Latitude 42.64906 N & Longitude – 73.74696 W

Location: Floating dock at riverfront park

Area: A path along the beach with rising forested hills, trees along both sides, plants along shore, bridges and boats. Dock on the shore, 2-3 pipes going into the water. Large bridge for trains over water. Highway elevated.

Surrounding Land Use: 70% Industrial/Commercial; 25% Urban/Residential, 5% Park- used for fishing, jogging, boating

Sampling Site: vegetation, banks altered, shoreline with riprap- floating dock, concrete bulkhead, pier and boat launch

Shoreline: Cottonwood and maple trees

Plants in area: No plants in water area.

River Bottom description: muddy

Sample Timeframe 9:00 AM -1:45 PM

| ITEM | Time | Reading |  | Comments |
| --- | --- | --- | --- | --- |
| Physical |  |  |  |  |
| Air Temperature | 9:53 AM- 12:31 PM | 15oC- 20oC | 59oF -68oF |  |
| Wind Speed: anemometer | 9:53 AM- 12:31 PM | 3 mph -7 mph | 2.6 knots- 6.1 knots | North |
| Weather today | Partly cloudy to mostly cloudy (56-75%) |  |  |  |
| Water Temperature | 9:53 AM- 12:31 PM | 13.5 oC | 56.3 oF |  |
| Turbidity - long sight tube | 9:53 AM- 12:31 PM | 71.2 cm -116.2 cm |  |  |
| Chemical |  |  |  |  |
| Salinity- Hydrometer | 9:53 AM- 12:31 PM | 500 ppm (hydrometer is read ppt not ppm - expect air bubble) | *This is very high for this area and hydrometer would not be the right instrument here. |  |
| DO – | Time | Temp | ppm DO | % saturation |
|  |  | 13.5 oC | 11 | 100 |
| pH – color match test kit | 9:53 AM- 12:31 PM | 7.1 |  |  |
| Fish | Number | Name | Length of |  |
|  | 11 39 1 1* 1 1 1 1 | Spottail Shiner Herring White Perch White Sucker* Bluegill American Eel Banded Killifish Striped Bass |  | *record here as one although dumped before counted |
| Fish Totals | TOTAL: 55 | Diversity: 8 |  |  |
| Current- | cm/second | Knots | North/South | Ebb/Flood/Still |
| 9:53 AM- 12:31 PM | 26.28 cm/sec- 15.24 cm/sec | 0.51 knots – 0.296 knots | - | - |