Day in the Life of the Hudson River 10/16/18 Data

GREEN ISLAND PARK

Green Island, NY

RIVER MILE: 153

Salt Front:~ RM 24

Rensselaer Polytechnic Institute- 16 students, undergraduates

Classical Conversations- 11 students 6th – 10th grade, 5 adults

Sarah Cadieux, RPI

Eleni Solar, Classical Conversations

Latitude 42.7481671 N & Longitude – 73.6889391 W

Location: Green Island Park boat ramp, below the Troy Dam, Albany County, NY

Area: Beach area with a lot of vegetation, very muddy and rocky. The Troy dam is directly to the north.

Surrounding Land Use: 88% Rocky; 41% Industrial/Commercial; 5% Urban/Residential, 5% Forested

Sampling Site: Beach; banks altered; riprap (large rocks); concrete bulkhead; pipe entering the water

Shoreline: 80% Rocky; 20% Muddy

Water Depth: 2-4 feet

Plants in area: No plants in water area; leaf litter on ground

River Bottom description: muddy, weedy, rocky

Troy Dam

| ITEM | Time | Reading |  | Comments |
| --- | --- | --- | --- | --- |
| Physical |  |  |  |  |
| Air Temperature | 2:39 PM | 12 oC | 53oF |  |
| Wind Speed: anemometer | 2:40 PM 2:41 PM 2:41 PM 2:41 PM 2:45 PM 2:45 PM 2:45 PM | 3.1 knots 3.0 knots 2.7 mps = 5.2 kts 3.1 kts 2.7 mps= 5.2 kts 4.8 mps= 9.3 kts | Beaufort #2 Beaufort #1 Beaufort#0.5 Beaufort #1 Beaufort #1 | North North North Northwest |
| Weather today | No rain, partly cloudy (26-50%) |  |  |  |
| Weather recently |  |  |  |  |
| Turbidity - long sight tube | 3:00 PM 3:07 PM 3:10 PM 3:15 PM 3:15 PM | 66.4 cm 66.4 cm 66.4 cm 66.4 cm, 40.8 cm 66.4 cm, 70.8 cm |  |  |
| Chemical |  |  |  |  |
| Salinity- Low Range Quantabs | 3:30 PM | <32 ppm Cl- |  | *Below Detection |
| pH – Test strips | 3:15 PM 3:20 PM 3:24 PM | 7, 6, 6, Avg: 6.33 6, 6, 7, Avg: 6.33 6, 7, 6, Avg: 6.33 |  |  |
| Fish | Number | Name | Length of longest |  |
| DEC electro-shocker | 3 1 1 3 2 1 1 | Catfish Shorthead Red Horse (Sucker) Sunfish American Eel Walleye Striped Bass Small Mouth | YOY | * Each group looked at the bucket of fish that the DEC caught with an electro-shocker. They didn’t have enough time to do a comprehensive fish count. |
| Fish Totals | TOTAL: 12 | Diversity: 7 |  |  |
| Macroinvertebrates | Number | Name | Length of longest |  |
|  | 1 | Crayfish |  |  |
| Macroinvertebrate Totals | TOTAL: 1 | Diversity: 1 |  |  |
| Tides- tide stick | Time | Rising, Falling, Unchanged | Height | Rate of Tidal Change (cm/min) |
|  | 2:10 PM 3:30 PM | - Falling | - 8” lower | - 0.1 |
| Current- | cm/second | Knots | North/South | Ebb/Flood/Still |
| 2:55 PM 3:10 PM | 38.1 33.2 | 0.74 0.65 | South South | Ebb Ebb |
| Boat Traffic | Name | Color | North or South | Loaded or Light |
| 2:21 PM | Dutch Apple Cruise | White | North | People on boat |