Day in the Life of the Hudson River 10/16/18 Data

SARAH LAWRENCE COLLEGE CURB: BECZAK, RIVER MILE: 18.3

Salt Front: ~RM 24

High School for Environmental Studies – Sean Abbot-Klafter

30 students 11th -12th grade, 5 adults

Elisa Caref, SLL CURB

Latitude 40.938483 N & Longitude -73.90291 W

Location: Sarah Lawrence College CURB at Beczak, Habirshaw County Park, Yonkers, Westchester County, NY

Area: 2 acre riverfront county park covered mostly by a grass lawn leading down to the river where it meets a tidal wetland and sandy beach. Property used for fishing, boats, seining, field trips and some education programs.

Surrounding Land Use: 75% Urban/Residential; 25% Beach/Waterfront

Sampling Site: Sandy beach, tidal marsh area covered with vegetation, banks altered, large riprap on shoreline, collected wood/debris in area

Plants in area: No plants in the water area; however in the marsh, there is mud wort, reed grass, salter cordgrass, big cordgrass

River Bottom: Muddy

| ITEM | Time | Reading |  | Comments |  |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature: |  | 17 o C | 60 o F |  |  |
| Wind Speed: | 12:11 AM 12:40 PM 1:02 PM | Beaufort 2 Beaufort 2 Beaufort 2 | Northwest Northwest Northwest |  |  |
| Weather Today: | No rain, partly to mostly cloudy |  |  |  |  |
| Water Temperature- YSI Probe Thermometer | 11:59 AM 12:40 PM 1:10 PM 11:59 AM 12:40 PM 1:10 PM | 15.6 o C 16.1 o C 16.1 o C 15 o C 15 o C 15 o C | 60.08 o F 60.98 o F 60.98 o F 59 o F 59 o F 59 o F |  |  |
| Turbidity - YSI Probe Long site tube | 11:59 AM 12:40 PM 1:10 PM 11:59 AM 12:40 PM 1:10 PM | 10.4 NTU 10.8 NTU 8.3 NTU 35 cm 35 cm 40 cm |  |  |  |
| Chemical |  |  |  |  |  |
| Salinity- YSI Probe ____________________ *Hand tool - hydrometer or Refractometer? | Time | Total Salinity |  |  | *hydrometer is low resolution in comparison to YSI. 0 ppm isn’t realistic; maybe it was out of hydrometer range. |
|  | 11:59 AM 12:40 PM 1:10 PM | 1.15 ppt 1.08 ppt 0.950 ppt |  |  |  |
|  | 11:59 AM 12:40 PM 1:10 PM | 3 ppt 0 ppt 1 ppt |  |  |  |
| DO – YSI Probe | Time | Temp | ppm DO |  | % saturation |
|  | 11:59 AM 12:40 PM 1:10 PM | 15.6 o C 16.1 o C 16.1 o C | 9.77 ppm 9.66 ppm 9.84 ppm |  | 90 91 93 |
| pH – YSI Probe | 11:59 AM 12:40 PM 1:10 PM | 8.14 8.14 8.21 |  |  |  |
| Fish- Seine 15 Total Hauls | Number | Name | Length of longest |  |  |
|  | 80 8 2 1 12 2 | Atlantic Silverside Blueback Herring Atlantic Menhaden Alewife Striped Bass White Perch |  |  |  |
|  | TOTAL: 105 | Diversity: 6 |  |  |  |
| Tides- | Time | Rising, Falling, Still | Height (cm) |  | Rate of Tidal Change (cm/min) |
| Group 1 Group 2 Group 3 | 11:52 AM 12:20 PM 12:31 PM 12:52 PM 12:58 PM 1:20 PM | - Rising Rising Rising Still Rising | 27 38 40 48 48 53 |  | - 0.39 0.18 0.38 - 0.23 |
| Currents Flow | Time | North/South | Ebb/Flood/Still |  |  |
|  | 12:45 PM | North | Flood |  |  |
| Sediment Core | Description | Abundant | Common |  | Rare |
| 7 cm long core | H2S (Sulfur smell) Oxidized top | Mud | Clay, Sand, Pebbles |  | Gravel Wood Macroinvertebrates |