Day in the Life of the Hudson River 10/16/18 Data

SOUTH DOCK, WEST POINT

RIVER MILE: 52

Salt Front: ~RM 24

United States Military Academy- Patrick Baker

16 students K-12 grade, 4 adults

Latitude 41.393241 N & Longitude – 73.951802 W

Location: South Side of Gee’s Point, West Point in the vicinity of Duck Island, Orange County, NY

Area: Beach area on the waterfront along the West Point property

Surrounding Land Use: 60% urban/residential; 40% forested

Sampling Site: Grassy, forested, parking lot

Shoreline: 60% muddy; 40% rocky; riprap, covered in vegetation, pipe entering the water (not used)

Plants in area: No plants in the water area; water chestnut seeds

Water: 6 inches water depth, calm

River Bottom: muddy, rocky

| ITEM |  | Time | Reading |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |
| Air Temperature: |  | 9:48 AM 9:55 AM 10:13 AM | 10.4 o C 10.1 o C 9.7 o C |  | 50.72 o F 50.18 o F 49.46 o F |  |
| Wind Speed: |  | 9:52 AM | 1.5 m/s |  | 2.916 knots |  |
| Weather Today: |  | No rain, partly cloudy (60%) |  |  |  |  |
| Weather Previous: |  | Light rain, cooler temperatures |  |  |  |  |
| Water Temperature |  | 9:36 AM 9:53 AM 10:01 AM | Avg: 14.9 o C Avg: 16.15 o C Avg: 15.9 o C |  | 58.82 o F 61.07 o F 60.62 o F |  |
| Turbidity - Turbidimeter |  | 9:36 AM | 15.7 NTUs, 9.86 NTUs, 21.7 NTUs |  | Avg: 15.75 NTUs | A higher third reading because a boat passed by which stirred up some silt/sand. |
| Chemical |  |  |  |  |  |  |
| Salinity- meter – read in Total Salinity |  | 9:30 AM 9:53 AM 10:01 AM | 285 ppm, 284 ppm, 284 ppm 283 ppm, 280 ppm, 278 ppm 279 ppm, 277 ppm, 277 ppm |  | HRECOS @ South Dock: 125 ppm | *readings seem high, meter wasn’t calibrated |
| DO – meter |  | Time | Temp |  | ppm DO | % saturation |
|  |  | 11:45 AM 11:50 AM | 12.3 o C 15.5 o C |  | 10.03 9.63 | 95.8% |
| pH - meter |  | 9:45 AM 9:55 AM 10:03 AM | 5.92, 5.90, 5.88 5.85, 5.86, 5.87 5.84, 5.87, 5.88 |  | HRECOS @ South Dock: pH ~7.5 | *readings seem low, meter was not calibrated |
| Fish – 30 foot seine 12:00 PM 3:00 PM |  | Number | Name |  | Length of longest | Seine #1: Catch Total 84, Length of Pull: 150’ CPUE: 1.837 Seine #2: Catch Total 30, Length of Pull: 50’ CPUE: 1.969 |
|  |  | 2 80 2 1 6 30 1 | Banded Killifish Alewife Striped Bass American Eel White Perch Alewife Spottail Shiner |  | Hand capture Fishing pole |  |
|  |  | TOTAL: 122 | Diversity: 7 |  |  |  |
| Macroinvertebrates- Seine |  | Number | Name |  | Length of longest |  |
|  |  | 3 | Blue Crabs |  | Male |  |
|  |  | TOTAL: 3 | Diversity: 1 |  |  |  |
| Tides- |  | Time | Rising, Falling, Still | Height (cm) | Rate of Tidal Change (cm/min) |  |
|  |  | 9:42 AM 9:54 AM 10:00 AM 10:10 AM 10:20 AM | Rising Rising Rising Rising Rising | 50 170 240 265 300 | - 10 11.67 2.5 3.5 |  |
| Currents Flow |  | Time | Cm/second | Knots | North/South | Ebb/Flood/Still |
|  |  |  | 8.33 | 0.16 |  |  |
| Sediment Core |  | Water depth | Length | Oxid Top | Grain Size | Other |
| NA |  |  |  |  |  |  |
| Boats | Time | Name | Color |  | North/South | Loaded/Light |
| Recreational | 9:50 AM 9:57 AM | Small boat Med. boat | White White |  | North South | Light Light |
| Other Observations |  | Water chestnut seeds, broken glass, broken brick, acorns, drift wood, large number of Canadian Geese, 1 caterpillar. |  |  |  |  |