Day in the Life of the Hudson River 10/22/19 Data

BROOKLYN BRIDGE PARK (EAST), BROOKLYN

RIVER MILE: 1.7 (East River)

Salt Front: ~RM 53

MS915 – Nicole Marcellin

3 groups (28, 27 25 students) 80 total 8th grade students, 10 adults

Brooklyn Bridge Park- Christina Tobitsch

Latitude 40.69653 N & Longitude – 73.99971 W

Degrees, minutes, seconds 40 o 41’47.6 N and - 73 o 59’56.7”W

Location: Pier 5 of Brooklyn Bridge Park, Brooklyn, NY

Area: Mixed use park, marina south and park pier north of beach

Surrounding Land Use: 100% Urban/residential and park land

Sampling Site: South of boat marina and in front of the soccer field.

Shoreline: beach, vegetation, riprap, pipe entering the water

Water: Calm

River Bottom: Sandy, rocky

No plants – just algae ~ 10%

| ITEM |  | Time | Reading |  | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature: |  | 10:47AM 11:34 AM 12:06 PM 12:35 PM | 15.5o C 15.5o C 16.7 C 17.8o C | 60 o F 60 o F 62 o F 64 o F |  |
| Wind |  | 10:00 AM | Beaufort #2 |  |  |
| Weather Today: |  | 10:30-11:00 AM light rain, full sampling time overcast (75%) |  |  |  |
| Weather Previous: |  | N/R |  |  |  |
| Water Temperature- in a bucket |  | 10:30 AM 11:30 AM 12:30 PM | 16o C 16o C 17o C | 61o F 61o F 62o F |  |
| Turbidity - HORIBA multiparameter probe |  | Time 10:50 AM 11:45 AM 12:50 PM | Long Tube 70.5 cm 83.5 cm 94 cm | Probe 21.7 ntu 15.6 ntu 8.4 ntu | Turbidity decreased during the day and students speculated that incoming tide or addition of freshwater (rain) may have influenced it. |
| Chemical |  |  |  |  |  |
| Salinity- H= hydrometer M=meter |  | 10:15 AM | 22 ppt (H) 20 ppt (H) 19.9 ppt (M) | 20.75 average |  |
|  |  | 11:15 AM | 22 ppt (H) 21 ppt (H) 20.45 ppt (M) | 21.15 average |  |
|  |  | 12:30 PM | 21 ppt (H) 19 ppt (H) 18.9 ppt (M) | 19.6 average |  |
| DO – C = Chemetrics M = Meter |  | Time | Temp | ppm DO | % saturation |
|  |  | 10:30 AM |  | 8 ppm (C) 6.5 ppm(C) 6.96 (M) | 80.3% |
|  |  | 11:30 AM |  | 7 ppm(C) 6 ppm(C) 7.51 ppm (M) | 87.4% |
|  |  | 12:30 PM |  | 7 ppm(C) 7 ppm(C) 7.02 (M) | 80.9% |
| pH - TK = Test Kit M = Meter |  | 10:20 AM | *6 (TK) *7.8 (M) | *They noted both of these may be inaccurate |  |
|  |  | 11:30 AM | 8 (TK) 8 (TK) 8.56 (M) |  |  |
|  |  | 12:30 PM | 8 (TK) 8/9 (TK) 8.59 (M) |  |  |
| Fish – seine – 20 ft. length 16 hauls total |  | Number | Name | Length of longest |  |
|  |  | 584 11 1 1 7 | Atlantic Silverside Striped Anchovy Winter flounder Pipefish Menhaden | 11.5 cm 5 cm 7 cm 8 cm 8 cm |  |
|  |  | TOTAL: 604 | Diversity: 5 |  |  |
| Macroinvertebrates |  | Number | Name | Length of longest |  |
|  |  | 1 1 5 6 21 | Hermit crab Mussel Moon Jelly Comb jelly Beroe Comb jelly Leidyi |  |  |
|  |  | TOTAL: 34 | Diversity: 5 |  |  |
| Tides- | Time | Rising, Falling, Still | Height (cm) | Tidal Notes |  |
|  | 10:00 AM - 1:00 PM | Flood |  | Low Tide 9:48 AM 0.9 ft | High Tide 3:58 AM 4.1 ft |
| Currents- | cm/second | Rising, Falling, Unchanged | North/South | Ebb/Flood/Still |  |
| N/R |  |  |  |  |  |
| Core | Length | Description | Rare | Common | Abundant |
|  | 18 cm | Bottom – wet and more compact Top – drier sand & looser | Wood ?macroinvertebrates (need a microscope) | - Gravel - Shells (small fragments) - Brick - Living vegetation (algae) | Sand |
| Boats | Time | Name | Color | North or South | Loaded or Light |
| N/R |  |  |  |  |  |
| Other Observations |  | Shell fragments (clam, oyster, mussel), small pebbles, algae (red, sea lettuce, rockweed), large pieces of driftwood Birds and bees abundant in vegetated areas of the beachfront |  |  |  |