Day in the Life of the Hudson River 10/22/19

ENGLEWOOD BOAT BASIN, ENGLEWOOD NJ

Bergen County, NJ

RIVER MILE: 14

Salt Front: RM 52 (West Point)

Paterson Academy for the Gifted and Talented-

50 students 6th grade, 5+ adults

Maureen Bruins, Paterson Academy

Kirk Weber, Meadowlands Environmental Center

Latitude 40.880581 N & Longitude – 73.944525 W

1: sediment core, salinity, turbidity

2: DO, pH, water temp.

3: Biotic sampling

Location: Bloomers Beach (North end), Englewood Boat Basin, NJ

Area: Sandy beach, banks altered, pier with riprap sections, wood bulkhead, debris in area. Piping ~30 ft located at sampling site. 2 concrete bulkheads and stone breakwaters on shoreline.

Surrounding Land Use: 40% Forested; 20% Beach; 20% Industrial/Commercial; 20% Urban/Residential

Shoreline: Loamy sand- sand and mud, a little weedy and rocky

Water: Choppy, 50 cm

River Bottom: Muddy

| ITEM | Time | Reading |  | Comments |
| --- | --- | --- | --- | --- |
| Physical |  |  |  |  |
| Air Temperature | 10:05 AM | 11 o C | 51 o F |  |
| Wind Speed: anemometer | 10:05 AM | 2-5.5 mph | 4.8 knots | Beaufort: 1-2 Direction: East |
| Weather today | Overcast, estimated <1 inch of rain/drizzle at 10:00 AM for 45 minutes. |  |  |  |
| Tides- | Time | Rising, Falling, Unchanged | Height (cm) | Rate of Tidal Change (cm/min) |
|  | 10:00 AM 11:30 AM | Falling Rising | 50 cm 80 cm | .33 cm/min |
| Current- | cm/second | Knots | North/South | Ebb/Flood/Still |
| 10:55 AM | 12.29 cm/sec | .24 knots | South | Ebb |
| Water Temperature | 10:15 AM 11:40 AM | 11 o C 16 o C 13 o C Avg: 13.3 o C 13.4 o C 12.8 o C 13.9 o C Avg: 13.4 o C | 51 o F 60 o F 55 o F Avg: 55.3 o F 56 o F 55 o F 57 o F Avg: 56 o F |  |
| Turbidity - long sight tube | 10:45 AM | 37 cm, 40 cm, 38 cm, Avg: 38.3 cm |  |  |
| Chemical |  |  |  |  |
| Salinity- Hydrometer | Time | Chloride | Total Salinity |  |
|  | 10:30 AM | - - - - | 5 ppt 6 ppt 6 ppt Avg: 5.7 ppt |  |
| DO – ampules | Time | Temp | ppm DO | % saturation |
|  | 10:30 AM | 15.6 o C | 6 ppm | 60% |
| pH – Indicator solution | 10:30 AM | 8, 8, 7, Avg: 7.7 |  |  |
| Fish | Number | Name | Length of longest | Comments |
| Seine #2 10:15 AM Seine #5 Seine #7 11:30 AM Seine #10 | 2 2 3 2 | Atlantic Silverside Atlantic Silverside Atlantic Silverside Atlantic Silverside | 7.5 cm 7 cm 8 cm 6 cm | Seine Net: 252” x 43 “ mesh size: ¼” |
| Fish Totals | TOTAL: 9 | Diversity: 1 |  |  |
| Macroinvertebrates | Number | Name | Length of longest |  |
|  | 18 2 1 | Blue Crab Sand Shrimp Moon Jelly |  | Seine and Dip net |
| Macroinvertebrate Totals | TOAL: 21 | Diversity: 3 |  |  |
| Boats: Time | Name | Color | North/South | Loaded/Light |
| 10:00 AM Recreational 10:00 AM | Circle Line Tugboat |  | South South | Light |
| Observations | Also observed some small private recreational vessels, both sail and motor driven. |  |  |  |