Day in the Life of the Hudson River 10/22/19

UPPER NYACK STATE PARK, LOWER BEACH

RIVER MILE: 30.4

Salt Front: ~ RM 53

Blue Rock School- Charlotte Jack

45 students 4-8th grade, 8 adults

Strawtown Studios- Laurie Seeman and Joanna Dickey

Latitude 41.119759 N & Longitude – 73.91243 W

Location:  Nyack lower Beach, Upper Nyack, Rockland County, NY

Area: Park/Boat launch near Hook Mountain; sandy beach by the large sitting stairs

Surrounding Land Use: 40% Forested; 10% Industrial/Commercial; 40% Beach- used for recreation and picnics

Sampling Site: Beach, covered with vegetation beyond the beach, stonewall, riprap shore, pipe coming into the river south of the sampling site

Shoreline: Sandy and rocky, muddy offshore

River Bottom: sandy inshore (<3 ft depth), muddy off shore (>3 ft depth), scattered large rock (<2ft length inshore)

Plants in water: Water chestnuts and water celery

Water: calm, 1 ft depth

| ITEM | Time | Reading |  | Comments |
| --- | --- | --- | --- | --- |
| Physical |  |  |  |  |
| Air Temperature | 10:50 AM 1:30 PM | 55 o F 59 o F |  |  |
| Wind Speed: |  | Beaufort 3 | 8-12 mph | East |
| Weather today | Overcast (100%), no rain |  |  |  |
| Weather the past 3 days |  |  |  |  |
| Tides- | Time | Rising, Falling, Unchanged | Height (cm) | Rate of Tidal Change (cm/min) |
|  | 10:30 AM 11:00 AM 11:30 AM 12:00 PM 1:20 PM | NA Rising Rising Rising Falling | 0 23 40 100 80 | NA 0.767 0.567 2 0.25 |
| Current- | Time | Speed | North/South | Ebb/Flood/Still |
|  | 10:45 AM | 4.13 cm/sec, 0.08 knots | North | Flood |
| Water Temperature | 10:30 AM 11:00 AM 11:30 AM 1:00 PM | 60 oF 60 oF 60 oF | 16 oC |  |
| Turbidity- short sight tube | 10:30 AM | 40, 40, 30, 40 JTUs |  |  |
| Chemical |  |  |  |  |
| Salinity- Quantabs | Time | Chloride ppm | Total Sal. ppm |  |
|  | 10:45 AM | 1505 ppm | 2719 ppm |  |
| DO – | Time | Temp | ppm DO | % saturation |
|  | 11:30 AM | 16 o C | 7.5 | 74% |
| pH – | 10:30 AM | 8.5, 8.5, 8.5 |  |  |
| Biological |  |  |  |  |
| Fish | Number | Name | Length of longest |  |
| Seine | 15 1 1 0 | Atlantic Silverside White Perch American Eel 0 | 2.5 cm 3 cm 12.5 cm 0 | 1 Seine 10:30 AM 2 Seine 11:00 AM 3 Seine 11:50 AM 4 Seine 1:15 PM |
| Fish Totals | TOTAL: 17 | Diversity: 3 |  |  |
| Macroinvertebrates | Number | Name | Length of longest |  |
|  |  | NR |  |  |
| Macroinvertebrate Totals | TOTAL: 8 | Diversity: 3 |  |  |
| Sediment Core | Time | Oxid Top | Grain Size | Other Features |
|  | NR |  |  |  |
| Boats | Time | Name/Color | North/South | Loaded/Light |
| Recreational | 11:30 AM | 2 Rowing | South | NA |
| Observations |  |  |  |  |