Day in the Life of the Hudson River 10/22/19 Data

(Salt Front ~RM 53)

RIVER MILE 35E

Croton Point Park, Croton on Hudson

Donna Light-Donovan – Croton Point High School – 50, 9th & 12th graders, 5 adults

Latitude 41.2084 N Longitude 73.8912 W

Location: Mother’s Lap, Croton Park NY

Area: Rocky beach surrounded by rising forested land, pier, beach, grassy, landfill

Surrounding Land Use: Beach 80%, 20% landfill across the road, Forested behind

Sampling Site: Beach area with with raprap @ north end, collected wood debris in area

River Bottom – sandy bottom

| ITEM | Time | Reading 1 |  | Comments |
| --- | --- | --- | --- | --- |
| Physical |  |  |  |  |
| Air Temperature | 9:30 AM 2:30 PM | 21°C 20 °C |  |  |
| Wind Speed |  | NR |  |  |
| Cloud Cover | overcast |  |  |  |
| Weather today | Overcast and damp |  |  |  |
| Weather recently | N/R |  |  |  |
| Water Temperature | 12:30 PM 1:00 PM 1:30 PM 1:47 PM | 17 ° C 16.7° C | 61, 61, 59 °F 60, 61 °F 52 (?), 58°F | 60.75 °F /17 °C 60.5 °F 55 °F |
| Water | N/R |  |  |  |
| Chemical | TIME | Reading Hach Kit | E Probe |  |
| DO - surface | 12:45 PM. 1:00 PM. 1:23 PM. 1:41 PM. 1:47 PM. 1:54 PM. | 12 ppm. 6 ppm. 10 ppm. 34 ppm (error). 11 ppm. | 9.66 ppm | 100% |
| pH | 1:47 PM | 7.89 |  |  |
| Salinity – Quantabs HR | 12:30 PM 1:30 PM | 5.8 Q.Tab unit 5.6 Q Tab unit | 0.259%NaCl 0.248%NaCl | 1634 ppm Cl- *2824 ppm TS 1505 Cl- TS *2719 ppm TS |
| Refractometer* | 12:30 PM 1:00 PM | 20,20,20 23, 25, 23 | Avg. 20 ppt Avg. 23.66 ppt | *these readings were too high (20 to 23.66 ppt? Should be 2.0 and 2.4 ppt) |
| Salinity e-probe | 1:47 PM | 2.85 ppt |  | 2850 ppm TS |
| Turbidity - Secchi | 12:30 PM 1:00 PM 1:30 PM | 40, 52, 78 cm 45, 49, 60 cm 70, 65, 79 cm | Average | 56.67 cm 51.33 cm 71.33 cm |
| Turbidity – Long tube | 12:30 PM 1:00 PM 1:30 PM | 47, 44, 46 cm 46, 45, 44 cm 39, 41. 43 cm | 45.67 cm 45 cm 41 cm | E Probe 9.5 fnu = 45 cm |
| Sediment Core | Comment | Abundant | Common | Rare |
| 1:45 PM in 3 ft. water |  | Sand |  | Mud |

| Fish – | PM | Species | length |  |
| --- | --- | --- | --- | --- |
| Seine 15 ft. X 4 ft. tall – 12:30 PM | 8 | Atlantic Silversides |  |  |
|  | 2 | Striped Bass |  |  |
| 1:00 PM | 2 | White Perch |  |  |
| Macros 12:30 | 2 | Blue Crab |  |  |
| TOTAL FISH | 12 | DIVERSITY 3 |  |  |
| TOTAL MACRO | 2 | DIVERSITY 1 |  |  |
| Tides | N/R |  |  |  |
| Currents | N/R |  |  |  |