Day in the Life of the Hudson River 10/22/19

KINGSTON POINT PARK

RIVER MILE: 92

Salt Front: ~ RM 53

Kingston YMCA Farm Project – Susan Hereth

Kingston Parks & Recreation- Julie Noble

12 students 9-12th grade

Latitude 41.930213 N & Longitude – 73.96467 W

Location: Kingston Point Park, City of Kingston, Ulster County, NY

Area: Swimming beach, forested, parking lot; used for picnics, boating, fishing, swimming, jogging, kayaking

Surrounding Land Use: 5% Forested; 10% Industrial/Commercial; 85% Beach

Sampling Site: Beach Area

Shoreline: Sandy shoreline (75 ft from shoreline to parking lot and 200 ft wide); boat launch; banks are altered; riprap to north along brickyard shoreline; large amount of brick debris from prior industry

Water: 1 ft, calm

River Bottom: Sandy

Plants in water: none but evidence on shore

| ITEM | Time | Reading |  | Comments |
| --- | --- | --- | --- | --- |
| Physical |  |  |  |  |
| Air Temperature | NA |  |  |  |
| Wind Speed: |  | Beaufort 1 | South/Southwest |  |
| Weather today | Overcast (100%), drizzling rain |  |  |  |
| Weather the past 3 days: | Rain on Sunday |  |  |  |
| Tides- | Time | Rising, Falling, Unchanged | Height (in) | Rate of Tidal Change (cm/min) |
| 3:18 Low tide | 3:00 PM 3:35 PM 3:45 PM 4:20 PM | Falling Unchanged Rising Rising | - 0 2 10 | NA NA 0.2 0.23 |
| Current- | Time | cm/second | North/South | Ebb/Flood/Still |
|  | 3:40 PM | NR | NR | Still |
| Water Temperature | 3:56 PM | 12.8 oC, 15.5 oC, 13.6 oC | Avg: 13.97 oC |  |
| Turbidity- | NA |  |  |  |
| Chemical |  |  |  |  |
| Salinity- Hydrometer | Time | Chloride ppm Cl- | Total Sal. Ppm |  |
|  | 4:00 PM |  | Below Detection |  |
| DO – | Time | Temp | ppm DO | % saturation |
|  | NA |  |  |  |
| pH – pH strips | 4:00 PM | 6.5 |  |  |
| Alkalinity- | 4:00 PM | 60 ppm |  |  |
| Fish | Number | Name | Length of longest |  |
|  | 2 24 1 | Striped Bass Spottail Shiner Banded Killifish | 7 inches 5 inches 1 inch |  |
| Fish Totals | TOTAL: 27 | Diversity: 3 |  |  |
| Macroinvertebrates | Number | Name | Length of longest |  |
| Macroinvertebrate Totals | TOAL: | Diversity: |  |  |
| Sediment Core | Time | Oxid Top | Grain Size | Other Features |
| Boats | Time | Name/Color | North/South | Loaded/Light |
| Recreational Recreational | 2:45 PM 4:20 PM | Clearwater? NA | South South | Sails down |
| Observations | Water celery, spatterdock, water chestnuts, duckweed on shore. |  |  |  |