Day in the Life of the Hudson River 10/22/20 Data

HUNT’S POINT RIVERSIDE PARK, BRONX

RIVER MILE: 4 (Bronx River)

Hudson Salt Front: ~RM 75

Rocking the Boat- Kris Mielenhausen & 2 Associates

Latitude 40. 81769 N & Longitude – 73.88149

Location: Hunt’s Point, Riverside Park, Bronx River, NY

Area: Mostly used for boating and fishing. Pier with grassy area and parking lot.

Surrounding Land Use: 95% Industrial/Commercial; 5% Beach

Sample Site: Beach, pier, altered shoreline, riprap, covered with vegetation, debris in area

Shoreline: Rocky, riprap with pipe entering the water

River Bottom: Muddy, rocky

Water: 6 feet deep, calm

| ITEM | Time | Reading |  | Comments |
| --- | --- | --- | --- | --- |
| Physical |  |  |  |  |
| Air Temperature: | 5:00 PM | 20 o C | 68 o F |  |
| Wind Speed: | NR |  |  |  |
| Weather Today: | Foggy early – no precipitation |  |  |  |
| Weather Previous: | Cloudy, highs around 70 F |  |  |  |
| Tides | High Tide 11:16 AM | Low Tide 5:03 PM | Flooding at sample time 5:00 PM |  |
| Water Temperature | 5:00 PM | 23 o C | 73.4 o F |  |
| Turbidity- long sight tube | 5:00 PM | 82 cm |  |  |
| Chemical |  |  |  |  |
| Salinity- hydrometer | 5:00 PM | 14 ppt |  |  |
| pH- color match kit | 5:00 PM | 7 |  |  |
| Nitrates | 5:00 PM | 1 ppm |  |  |
| DO – Lamotte Drop Count | Time | Temp | ppm DO | % saturation |
|  | 5:00 PM | 23 o C | 4 ppm | 47% |