Day in the Life of the Hudson River 10/22/20

GARRISON LANDING

RIVER MILE: 51.5

Salt Front: ~ RM 75

Katie Krueger Hirt, 5 participants

Latitude 41.38207 N & Longitude – 73.94799 W

Seining at Garrison Landing      Looking Northwest at West Point  	      Checking pH

Location: Garrison Landing Park, Garrison, Putnam County, NY

Area: Waterfront beach area with a pier in front of grassy park primarily used for boating and picnics Surrounding Land Use: 60% Forested; 20% Urban/Residential; 10% Industrial/Commercial; 10% Other

Sampling Site: Rocky shoreline, pier, shore with bulkhead, riprap, grassy, parking lot

Shoreline: Sandy, muddy

Plants in area: no plants in water area

River Bottom: muddy; sandy

| ITEM | Time | Reading |  |  | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 8:12 AM 12:45 PM 2:15 PM | 17o C 22.4 o C 22.78 o C |  | 65.5 o F 72.32 o F 73 o F |  |
| Wind Speed: | 8:12 AM 12:45 PM 2:00 PMM | 2.6 m/s 0 m/s 1.2 m/s |  | Northeast Northeast |  |
| Weather today | No Rain |  |  |  |  |
| Weather past 3 days: | Temperature Highs: Mon 63F, Tues. 72F, Wed 70F. No rain the past 3 days |  |  |  |  |
| Tides- | Low Tide | High Tide |  | Time of Sampling | Tide |
|  | 10:52 AM | 4:45 PM |  | Morning 12:45 PM 3:18 PM | Ebbing Flooding Flooding |
| Current- | Time | cm/second, Knots |  | North/South | Ebb/Flood/Still |
|  | Morning 12:45 PM 3:18 PM | 18.3 cm/s 26.1 cm/s 13.3 cm/s |  | South North North | Ebbing Flooding Flooding |
| Water Temperature- | 8:12 AM 12:30 PM 2:10 PM | 18 o C 19 o C 19 o C | 64.4 o F 66.2 o F 66.2 o F |  |  |
| Turbidity- Secchi Disk | 8:03 AM 12:36 PM 2:06 PM | 60 cm 38 cm 56 cm |  |  |  |
| Chemical |  |  |  |  |  |
| Salinity- High Range Quantab | 8:32 AM 12:39 PM 2:20 PM | 2100 ppm Cl- 1634 ppm Cl- 1776 ppm Cl- |  | 3794 ppm total salinity 2952 ppm total salinity 3208 ppm total salinity |  |
| DO – Hach Kit | Time | Temp |  | ppm DO | % saturation |
|  | 8:23 AM 12:30 PM 2:20 PM | 18 o C 19 o C 19 o C |  | 12 ppm 13 ppm 12 ppm | 125% 140% 130% |
| pH- Color Match Kit | 8:32 AM 12:22 PM 2:23 PM | 7.5 7 8 |  |  |  |
| Biological |  |  |  |  |  |
| Fish | Number | Name |  | Length of longest | Length of smallest |
| Seine ~12:30 PM Seine ~2:00 PM | 3 2 1 2 3 1 1 | American shad Atlantic silverside Striped Killifish White perch Striped bass American shad striped killifish |  | 6.4 cm 5 cm 8.5 cm 13.7 cm 9.2 cm 4.2 cm 6.4 cm | 6.2 cm 4.9 cm 7.5 cm 4.0 cm |
| Fish Totals | TOTAL: 13 | Diversity: 5 |  |  |  |
| Macroinvertebrates | Number | Name |  | Length of longest | Length of smallest |
| Macroinvertebrate Totals | TOTAL: | Diversity: |  |  |  |
| Observations |  |  |  |  |  |